3GPP TSG-RAN WG3 Meeting #125-bis	R3-245520

Hefei , China, 14th – 18th Oct 2024

Title: 	Discussion on Supporting MBS broadcast service for NR NTN

Source: 	CMCC

Agenda item: 	14.2

Document Type:	Discussion

- Introduction

In the 3GPP TSG RAN Meeting #103, the work plan for Non-Terrestrial Networks (NTN) for NR Phase 3 has been approved in RP-240775.

For RAN3, one of the objectives is to specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify the necessary signalling between CN and NG-RAN. [RAN3]

During RAN3 #125, agreements were achieved as following:

Turn WA into agreement:

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

However, Whether and how to define the geographical area information over NG interface? needs to be further discussed.

2	Discussion

RAN2 has gotten consensus that the geographical information can be introduced in the intended broadcast service area from last meeting. Anyway, RAN2 doesn’t use the GNSS description:

The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

RAN3 can begin the discussion to introduce more information in Intended Service Area, but need clear use cases to decide what and how to include the extra information than Mapped Cell ID.

In some scenarios, the MBS broadcast intended geographical location can’t be exactly mapped to Cell coverage area. Especially when a NTN cell’s coverage is much more larger than the intended MBS broadcast service’s geographical area. For example, a NTN cell covers multiple countries. And the MBS broadcast service is only intended to a specific country’s geographical location in the area. UE may roam to another PLMN which is also covered by the same cell, then AMF may need to carry extra information to RAN, to indicate the intended service area.

It is not only to improve the accuracy of the MBS service area, but also useful in some NG enhancement solutions, such as Store&Forward for IoT NTN, or NG suspend/resume or Xn Handover enhancements for specific UEs. The small granularity location information in MBS Service Area can help the satellite (gNB on board) make decisions to choose different solution/enhancement for a given UE. These UE may be under the same cell coverage area but needs different solutions according to their geographical location.

Proposal 1: It is beneficial to introduce geographical information under MBS intended service area, to provide smaller granularity service area or different service area than Mapped cell.

However how to describe the geographical information needs further discussion. Since here we discuss the NG interface MBS intended service area, it doesn’t need to be perfectly match with UE side.

Observation 1: Whether to introduce smaller granularity of MBS service area should be defined according to the service requirement, not the granularity in UE.

Proposal 2: It is suggested to discuss how to describe the geographical information in MBS intended service area for NG interface.

In previous Tdocs, some companies suggested to use GNSS description to represent the geographical information. The GNSS description may be more accurate than using a (set of) referenceLocation and radius or by a (set of) polygon(s) which are adopted by RAN2. But it is also more complicated. The GNSS location information may have too many parameters which are not useful in MBS service areas. And GNSS is a general concept, the format can vary a lot according to the technology and scenarios.In MBS intend service area, the accuracy provided by a (set of) referenceLocation and radius or by a (set of) polygon(s) should be enough and concise.

Proposal 3: It is proposed to introduce referenceLocation and radius information in MBS intended service area, as RAN2, to present the geographical information, and provide smaller granularity service area than cell level.

3	Summary

In this contribution, we provide considerations on the NTN broadcast service and PWS service in RAN3, following observations and proposals are made:

Proposal 1: It is beneficial to introduce geographical information under MBS intended service area, to provide smaller granularity service area or different service area than Mapped cell.

Observation 1: Whether to introduce smaller granularity of MBS service area should be defined according to the service requirement, not the granularity in UE.

Proposal 2: It is suggested to discuss how to describe the geographical information in MBS intended service area for NG interface.

Proposal 3: It is proposed to introduce referenceLocation and radius information in MBS intended service area, as RAN2, to present the geographical information, and provide smaller granularity service area than cell level.

4	Reference

[1] 	TS 38.413, NGAP

[2]       TS 23.247, Architectural enhancements for 5G multicast-broadcast services

[3]       TS 38.300, NG-RAN stage2