3GPP TSG-RAN WG3 Meeting #125bis	                                                                R3-245527

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	31.2

Source:	CMCC, ZTE, CATT, China Telecom, NTT DOCOMO

Title:	Discussion on mechanism for UAV regulation

Document for:	Discussion

Introduction

Uncrewed aerial vehicles (UAVs), which is commonly used for logistics, delivery, and surveillance, have attracted extensive attention in recent years. In order to strengthen the regulation of UAVs, it is necessary for the network side to support aerial UE identification. During current standard discussion, no network-based mechanism of identifying, detecting and reporting the flying status of UAV or UAV-like UE is specified, leading to the UAV regulation in 5G system ineffective or even unreliable today. In this contribution, we would discuss the potential mechanism for UAV regulation used in NG-RAN node and analyze the potential solutions.

Discussion

- 2.1 Requirement

As UAV UEs have been already widespread used across society, the government for public security raised a clear requirement of security surveillance relying on close cooperation with mobile network operator, which demands a high level of standard and well-defined functions and specific procedures designed to support safe, efficient and secure access to airspace for large numbers of drones in a continuous manner, which applicability covers both the pre-R18 UAV device and R18 device.

Specifically, the requirement from the government aims to track the real-time motion status of a legitimate UAV UE, in support of the operations have the required competency and information for effective control and management of the UAV UEs (e.g. altitude-awareness, geo-awareness, detection, flight direction and speed of the UAVs). Moreover, for spurious drones, the network is required to report the above information of the drone device or identification result of the spurious drone device to the application entity of the government for public security as well.

Rel-18 UAV device

For Rel-18 aerial UE, the functionality for subscription-based identification of aerial UE has been supported for Rel-18 UAV UEs with following description in the specification.

### 16.18.2 Subscription-based Identification of Aerial UE Support for Aerial UE functions is stored in the user’s subscription information in UDM. UDM transfers this information to the AMF during Registration, Service Request and Mobility Registration Update procedures. The Aerial UE subscription information can be provided by the AMF to the NG-RAN node via the NGAP INITIAL CONTEXT SETUP REQUEST message during the Registration, Mobility Registration Update and Service Request procedures. The subscription information can also be updated via the NGAP UE Context Modification procedure and NGAP Path Switch Request procedure. In addition, for Xn-based handover, the source NG-RAN node can include the Aerial UE subscription information in the XnAP HANDOVER REQUEST message and RETRIEVE UE CONTEXT RESPONSE message to the target NG-RAN node. For intra- and inter-AMF NG-based handover, the AMF provides the Aerial UE subscription information to the target NG-RAN node after the handover procedure.

Rel-18 aerial UE can report its capability to the CN in the UE Registration procedure. Once receiving the UE capability information, the UDM will send the Aerial UE subscription information to the AMF. Then the subscription information could be provided from AMF to the UAV monitor platform. However, once an aerial UE exceeds a certain altitude within a controlled area, there may be a security risk of data exposure within the area. Currently, the NG-RAN node has an accurate knowledge about whether a UE is an aerial UE based on the authentication procedure and UE capability/measurement/flight path reporting. However, existing NGAP procedure does not support NG-RAN node to inform CN the aerial UE’s flight altitude, flight direction and horizontal/vertical speed. As shown in [2], the UAV is not allowed to fly below 120ms and exceed a certain speed to avoid the influence of assemblies of people within the range in read circle where the unmanned aircraft is flown. Thus, to support of the operations have the required competency and information for effective control and management of the UAV UEs, the flying status information, such as, altitude, detection, flight direction and speed of the UAVs, associated with UE NGAP ID should be transferred from NG-RAN node to CN, then be exposed of the information to the AF of the government for public security by NEF.

Figure 1 EU Regulation for UAS operations in the open and specific categories [2]

Observation 1: For Rel-18 UE, the NG-RAN node has an accurate knowledge about whether a UE is an aerial UE based on the authentication procedure and UE capability/measurement/flight path reporting.

Observation 2: Existing NGAP procedure does not support NG-RAN node to inform CN the aerial UE’s flight status information, such as flight altitude, flight direction and horizontal/vertical speed.

Pre-Rel 18 device and non-UAV Rel-18 device

For legacy devices (i.e. a Rel-15/16/17 device and non-UAV Rel-18 device) equipped in an aerial vehicle, strict regulation demand for those spurious aerial UEs is required since the legacy UEs are not subscribed as aerial UEs and no subscription information is stored in UAV management/surveillance platform. To identify those illegal devices among grounded UEs, NG-RAN node should have the ability to identify the flight status of the UE. The altitude and velocity of flying UE can be considered as a kind of flight status information. As specified in RRC spec, the CommonLocationInfo IE included in MeasResultNR is introduced to transfer detailed location information available at the UE to correlate measurements and UE position information.

Within the CommonLocationInfo IE, the locationCoordinate-r16 carries the altitude information and the velocityEstimate-r16 carries the velocity information. The content in locationCoordinate-r16 and velocityEstimate-r16 can be found in TS 37.355 reflected as LocationCoordinates IE and Velocity IE:

Hence, the NG-RAN node can differentiate the flying UEs and grounded UEs based on the reported altitude information and velocity information.

Observation 3: The NG-RAN node can differentiate the flying UEs from grounded UEs based on the CommonLocationInfo IE included in MeasResultNR in RRC message.

In addition, as flying UEs and grounded UEs have some obvious distinctions, NG-RAN node also has the capability to identify a UE as a flying UE based on some specific information in implementation, such as measurement reporting, handover frequency, flight path reporting, and interference level. For example, if the NG-RAN node identifies a UE trajectory follows a specific trajectory or performs frequent handover, it can identify the UE as an aerial-like UE. From the network trail, the identification of flying UE is detected through altitude-dependent serving cell distribution or RSRP/RSRQ as shown in the below figure.

Figure 2 Serving Cell distribution and Handover rate simulation data for drone UEs [R2-1713897, TR36.777]

Observation 4: NG-RAN node has the capability to identify a UE as a flying UE based on some specific information, such as measurement reporting, handover frequency, flight path reporting, and interference level, and report associated UE ID to the CN.

- 2.2	Potential solution

From above analysis, the flight status information should be transferred from NG-RAN node to 5GC over the NGAP procedure. In order to minimize RAN3 specification impact, the existing class-2 Location Report Procedure can be enhanced to provide the flying status information, including the altitude and velocity of aerial UE. In general, the purpose of the Location Report procedure is to provide the UE's current location, the UE's last known location with time stamp, or the UE's presence in the area of interest to the AMF. The altitude and velocity of aerial UE can be seen as a location information for aerial UE, so it is reasonable to include the altitude and velocity information for UAV in the NR user location information in the LOCATION REPORT message.

Proposal 1: It is proposed to include the altitude and velocity information for UAV in the NR user location information in the LOCATION REPORT message.

For Rel-18 aerial UE, the above change is enough for NG-RAN node as the flying status information to the CN. The Aerial UE Subscription Information is already associated with NGAP UE ID in the INITIAL CONTEXT SETUP REQUEST message from AMF to the NG-RAN node during Initial Context Setup procedure. Obviously, when receiving the LOCATION REPORT message with the unique NGAP UE ID, the CN knows the information is for which UE.

Except Rel-18 aerial UE, other legacy devices riding in aerial vehicle does not support to report UAV capability during Registration procedure. Regard to those legacy flying UEs, another UAV-like ID, i.e., named Non-UAV Aerial UE Indicator needs to be introduced in the LOCATION REPORT message to indicate flying UEs can be equivalent to aerial UE. Once receiving this ID, the network also should treat these UEs the same as aerial UEs.

Proposal 2: It is proposed to introduce Non-UAV Aerial UE Indicator in the LOCATION REPORT message to indicate legacy devices riding in aerial vehicle can be equivalent to aerial UE.

- 2.3	Impact on SA2

Rel-18 UAV device

If the AMF retrieves the flight status information along with UE ID, the AMF finally needs to report that information to monitor platform. Thus, the transfer mechanism from AMF to monitor platform is related to SA2’s work. The following specification is included in the SA2 TS 23.256.

From the specification, the tracking mode “List of Aerial UEs in a geographic area” is chosen by the USS/TPAE. In this mode, the USS/TPAE requests UAS NF for a list of the UAVs in the geographic area and served by the PLMN. The UAS NF may include Aerial UE indication as an event filter in the request, that is used by the AMF/MME to separate out the UEs that are actual UAVs based UEs with aerial subscriptions. The UAS NF performs the filtering on the received list from AMF before responding back to the USS/TPAE. The UAS-NF includes the 3GPP UAV ID for each of the UAVs in the tracking and location response to the USS/TPAE.

Pre-Rel 18 device and non-UAV Rel-18 device

Likewise, if the AMF retrieves the flight status information along with UE ID, the AMF needs to report that information to monitor platform via NEF. Since pre-Rel 18 device and non-UAV Rel-18 device does not have UAV ID, those UE needs to report its SUPI to the monitor platform. As existing procedure in TS 23.502, the NEF can invoke Nnef_UEId service. This service will output the AF specific UE Identifier represented as an External Identifier or SUPI.

In sum, whatever the Rel-18 UAV device or Pre-Rel 18 device and non-UAV Rel-18 device, it seems the enhancement on UAV regulation will bring minor impact on SA2 for both Rel-18 device and non-UAV Rel-18 device.

Observation 5: It seems the enhancement on UAV regulation will bring minor impact on SA2 for both Rel-18 device and non-UAV Rel-18 device.

Conclusions

In this contribution, we discuss the potential mechanism for UAV regulation used in NG-RAN node. The following observations and proposals are listed below:

Observation 1: For Rel-18 UE, the NG-RAN node has an accurate knowledge about whether a UE is an aerial UE based on the authentication procedure and UE capability/measurement/flight path reporting.

Observation 2: Existing NGAP procedure does not support NG-RAN node to inform CN the aerial UE’s flight status information, such as flight altitude, flight direction and horizontal/vertical speed.

Observation 3: The NG-RAN node can differentiate the flying UEs from grounded UEs based on the CommonLocationInfo IE included in MeasResultNR in RRC message.

Observation 4: NG-RAN node has the capability to identify a UE as a flying UE based on some specific information, such as measurement reporting, handover frequency, flight path reporting, and interference level, and report associated UE ID to the CN.

Observation 5: It seems the enhancement on UAV regulation will bring minor impact on SA2 for both Rel-18 device and non-UAV Rel-18 device.

Proposal 1: It is proposed to include the altitude and velocity information for UAV in the NR user location information in the LOCATION REPORT message.

Proposal 2: It is proposed to introduce Non-UAV Aerial UE Indicator in the LOCATION REPORT message to indicate legacy devices riding in aerial vehicle can be equivalent to aerial UE.

References

- 3GPP TS 38.300, NR; NR and NG-RAN Overall Description; Stage 2

- EASA UAS Regulation in the EASA Regulatory Structure