3GPP TSG-RAN WG3 Meeting #125bis	             R3-245530

Hefei, CN, 14th – 18th Oct 2024

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on RAN Architecture for Ambient IoT

Document for:	Discussion

1	Introduction

The TR capturing agreed TP associated with RAN architecture aspects for Ambient IoT is listed as follows:

### 6.4.1 Support of Topology 1 Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN. Figure 6.4.1-1 Logical system architecture for topology 1 In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface. Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport: Figure 6.4.1-2. Protocol Stack for Topology 1 Editor’s Note 1: Figure 6.3.1-2 serves as a starting point for further discussions. For topology 1, the XXAP is terminated at an A-IoT RAN node. Editor’s Note 2: the signalling transport for XXAP is FFS. Editor’s Note 3: The protocol stack does not detail how A-IoT upper layer information is transported over XXAP, details are pending on SA2 agreements. And the A-IoT CN may include AMF and A-IoT related functions which is also up to SA2 decision. Editor’s Note 4: aspects of interaction between upper layer information exchange and XXAP in order to trigger the A-IoT RAN node functions are FFS. ### 6.4.2 Support of Topology 2 Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB. The following definitions apply: A-IoT-enabled gNB: A gNB supporting A-IoT RAN node function, which is able to communicate with the A-IoT enabled UE via NR Uu interface. A-IoT-enabled UE: A UE supporting Common reader function, which is able to communicate with the A-IoT device via the A-IoT radio interface. Figure 6.4.2-1 Logical system architecture for topology 2 Editor’s Note 1: Figure 6.3.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2. Editor’s Note 2: In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS. Editor’s Note 3: The A-IoT CN could include AMF and A-IoT related functions. This is up to SA2 decision. Editor’s Note 4: The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms. #### 6.4.2.1 Solutions for Topology 2 To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information: - RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC. - NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS. - UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data. Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS. Editor’s note 2: Depiction and further details of the options above are FFS In this contribution, we will continue to provide our views on RAN architecture aspects for Ambient IoT.

In this contribution, we will continue to provide our views on RAN architecture aspects for Ambient IoT.

2	Discussion

2.1	Basic Principle for A-IoT deployment

From the SID, there is a detailed description about the frequency band for deployment scenarios.

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

It is noted that the 3GPP A-IoT deployment will use FR1 licensed spectrum in FDD and cover spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). The OAM will configure the AIoT RAN node and AIoT-enabled gNB with the available frequency resources for topology 1 and topology 2. Utilizing the license band from the operator will provide a reliable, manageable and controllable way for A-IoT system communication. From the statement in the SID, it gives a big picture for 3GPP A-IoT deployment with the prospect of building a large-scale network with seamless coverage. In the future deployment, the base station plays the role of precisely coordinating the radio resources accessed by each A-IoT device to support a large number of device connection and minimize the interference as much as possible. Obviously, these requirements cannot be met if using unlicensed spectrum. Therefore, within the discussion in 3GPP scope, RAN3 should follow the basic principle that the A-IoT deployment should utilize the licensed spectrum only.

Observation 1: The base station plays the role of precisely coordinating the radio resources accessed by each A-IoT device to support a large number of device connection and minimize the interference as much as possible.

Proposal 1: Within the discussion in 3GPP scope, RAN3 should follow the basic principle that the A-IoT deployment should utilize the licensed spectrum only.

2.2	Protocol Stack for topology 2

During last meeting, three solutions are provided to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

For topology 2, based on the above summary in pCR, three corresponding protocol stacks are shown as follows:

RRC based solution

Figure 1: RRC based solution of Topology 2

In this case, for RRC-based solution, the A-IoT related signalling or data exchanged between AIoT enabled gNB and AIoT enabled UE is in RRC message. The A-IoT enabled gNB generates RRC messages based on the message type indicated by the CN when transmitting downlink signalling/data to A-IoT enabled UE. To this end, it is more efficient for A-IoT enabled gNB allocation A-IoT air interface resources since the inventory or command from CN can be visible to A-IoT enabled gNB directly.

Observation 2: For RRC-based solution, the A-IoT enabled gNB generates RRC messages based on the message type indicated by the CN when transmitting downlink signalling/data to A-IoT enabled UE.

NAS based solution

Figure 2: NAS based solution of Topology 2

In this case, CN communicates with A-IoT enabled UE using NAS message directly. The A-IoT related signalling/data between the A-IoT CN and the A-IoT enabled UE are carried via A-IoT enabled UE’s DL/UL NAS packets. The A-IoT enabled gNB is transparent to the NAS packet which is possibly transferred via DL NAS Transport and UL NAS Transport procedures. The A-IoT enabled UE needs to request the resources used in A-IoT radio interface from A-IoT enabled gNB, which brings the signalling overhead compared to RRC-based solution.

Observation 3: For NAS-based solution, the A-IoT enabled UE may need to request the A-IoT radio resources from gNB when no available A-IoT radio resources allocated, thus increases the signalling overhead compared to RRC-based solution.

UP based solution

Figure 3: UP Session based solution of Topology 2

In this case, the A-IoT related signalling/data between the A-IoT CN and the A-IoT enabled UE are included in A-IoT enabled UE’s PDU Session. The A-IoT enabled gNB handles the A-IoT enabled UE’s user plane data as legacy, i.e., using NG-U GTP-U tunnels. Note that for PDU session-based solution, signalling transmission via control plane is also needed because A-IoT radio resources should be configured and controlled by A-IoT enabled gNB.

Observation 4: For PDU session-based solution, signalling transmission via control plane is also needed because A-IoT enabled gNB controls A-IoT radio resource allocation.

Proposal 2: RAN3 should discuss and compare three solutions with the consideration of AIoT radio resource allocation.

A-IoT enabled UE authorization

In topology 2, if a UE wants to act as UE reader, the precondition is this UE should be authorized by the network. Only if the UE is authorized to perform A-IoT service, the UE can communicate with the A-IoT device and obtain the AIoT radio resources controlled by the A-IoT enabled gNB. To support UE authorization, UE can report its capability to the CN in the UE Registration procedure. Once receiving the UE capability information, the UDM will send the UE subscription information to the AMF. If the UE is authorized, then the AMF informs to the NG-RAN node UE is authorized to perform A-IoT service. Hence, the signalling procedure over NGAP should support the A-IoT enabled UE authorization.

Observation 5: Only if the UE is authorized to perform A-IoT service, the UE can communicate with the A-IoT device and obtain the AIoT radio resources controlled by the A-IoT enabled gNB.

Proposal 3: RAN3 should discuss the signalling procedure over NGAP should support the A-IoT enabled UE authorization.

Paing UE reader in RRC_IDLE and RRC_INACTIVE

Regard to topology 2, A-IoT enabled UE is not always in RRC_CONNECTED state when it does not conduct A-IoT procedure. For A-IoT enabled UE in RRC_INACTIVE state, it can be resumed to RRC_CONNECTED by a RAN-initiated paging at first. And for A-IoT enabled UE in RRC_IDLE state, it should be transfer to RRC_CONNECTED state by CN-initiated paging at first which is triggered by AMF. While, for A-IoT enabled UE in RRC_IDLE or RRC_INACTIVE, they should be transferred to RRC_CONNECTED under network control before initiating A-IoT procedure. After UE is selected, it conducts the inventory or command procedure according to the received parameters from the A-IoT CN. With the consideration that the feedback of A-IoT procedure should be transmitted to A-IoT CN, there may be some signalling sent from reader to CN when conducing A-IoT procedure, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard. To this end, it is proposed that RAN3 assumes the intermediate UE is in RRC_CONNECTED state when conducting A-IoT procedure in R19.

Observation 6: With the consideration that the results of A-IoT service should be transmitted to CN and there may be some signalling sent from reader to A-IoT CN, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard.

Proposal 4: RAN3 assumes the intermediate UE is in RRC_CONNECTED state when conducting A-IoT procedure in R19.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide following observations and proposals on RAN architecture aspects for Ambient IoT:

Observation 1: The base station plays the role of precisely coordinating the radio resources accessed by each A-IoT device to support a large number of device connection and minimize the interference as much as possible.

Observation 2: For RRC-based solution, the A-IoT enabled gNB generates RRC messages based on the message type indicated by the CN when transmitting downlink signalling/data to A-IoT enabled UE.

Observation 3: For NAS-based solution, the A-IoT enabled UE may need to request the A-IoT radio resources from gNB when no available A-IoT radio resources allocated, thus increases the signalling overhead compared to RRC-based solution.

Observation 4: For PDU session-based solution, signalling transmission via control plane is also needed because A-IoT enabled gNB controls A-IoT radio resource allocation.

Observation 5: Only if the UE is authorized to perform A-IoT service, the UE can communicate with the A-IoT device and obtain the AIoT radio resources controlled by the A-IoT enabled gNB.

Observation 6: With the consideration that the results of A-IoT service should be transmitted to CN and there may be some signalling sent from reader to A-IoT CN, keeping the intermediate UE in RRC_CONNECTED state is a good solution and minimizes impacts on standard.

Proposal 1: Within the discussion in 3GPP scope, RAN3 should follow the basic principle that the A-IoT deployment should utilize the licensed spectrum only.

Proposal 2: RAN3 should discuss and compare three solutions with the consideration of AIoT radio resource allocation.

Proposal 3: RAN3 should discuss the signalling procedure over NGAP should support the A-IoT enabled UE authorization.

Proposal 4: RAN3 assumes the intermediate UE is in RRC_CONNECTED state when conducting A-IoT procedure in R19.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[3] 3GPP TR 38.848, Study on Ambient IoT (Internet of Things) in RAN;