3GPP TSG-RAN WG3 Meeting #125bis	             R3-245531

Hefei, CN, 14th – 18th Oct 2024

Agenda Item:	16.3

Source:	CMCC

Title:	Discussion on signalling and procedure for topology1

Document for:	Discussion

1	Introduction

In current TR, the candidate procedure for A-IoT Inventory for Topology 1 is shown as follows:

### 6.5.2 Signalling and Procedures for Topology 1 #### 6.5.2.1 Candidate procedures for A-IoT Inventory for Topology 1 Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc. Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

In this contribution, we will continue to discuss the signalling and procedure for Topology 1.

2	Discussion

2.1	Inventory and Command Procedure for Topology 1

As existing message flow for topology 1 only includes the inventory procedure, the message flow for command procedure should be discussed in this meeting. In order to present the entire procedure more clearly, the inventory and command procedure are placed in one message flow as follows:

Figure 1 Message flow for A-IoT Inventory and Command in Topology 1

For topology 1, the steps are described as follows:

- The AIoT CN sends an Inventory request message to the AIoT RAN node.

- The AIoT RAN node sends an Inventory response message to the AIoT CN.

NOTE 1: In step 2, the AIoT RAN node may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

- The AIoT RAN node initiates inventory procedure towards the AIoT device(s) over the AIoT radio interface.

4/4b. After receiving inventory result from the AIoT device(s), the AIoT RAN node may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 2: The Step 4/4b may happen in parallel with Step 3.

- The AIoT CN performs security check to verify device ID.

- The AIoT CN sends a Command request message to the AIoT RAN node.

- The AIoT RAN node initiates command procedure towards the AIoT device(s) over the AIoT radio interface.

- After receiving command result from the AIoT device(s), the AIoT RAN node may send Command response towards the AIoT CN including the received command result.

From the above message flow, the AIoT CN can initiate a class-1 command procedure to the AIoT RAN node.

Proposal 1: The AIoT CN can initiate a class-1 command procedure to the AIoT RAN node.

In addition, when the A-IoT CN can initiates a class-1 Inventory request procedure to the AIoT RAN node, the AIoT RAN node can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Proposal 2: The AIoT RAN node can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

The overall message flow illustrates the “command after inventory” case, where the CN requests the AIoT RAN node to perform the inventory first and subsequently requests the AIoT RAN node to perform the command. During this case, for identical device, AIoT RAN node may utilize the same resource for inventory and command. In order to identify the device over XXAP, the device XXAP ID, consisting of CN device XXAP ID and RAN device XXAP ID, should be separately stored in AIoT RAN node and CN as the device context.

Observation 1: During the “command after inventory” case, for identical device, AIoT RAN node may utilize the same resource for inventory and command.

Proposal 3: The device XXAP ID, consisting of CN device XXAP ID and RAN device XXAP ID, should be separately stored in AIoT RAN node and CN as the device context.

2.2	Synchronization between RAN2 and RAN3

As RAN2 and RAN3 are discussing paging over AIoT radio and over XX interface respectively, RAN3 can take following RAN2 agreements about paging over AIoT radio interface as reference.

RAN2#126: - RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

It has been agreed that AIoT paging message can contain a single device ID, a group ID and no device ID. The common part for AIoT paging is used in inventory and command, which is consistent over AIoT radio and XX interface. Hence, the type of device ID in AIoT paging message over XXAP can follow the same design of AIoT paging over AIoT radio.

Observation 2: The type of device ID in AIoT paging message over XXAP can follow the same design of AIoT paging over AIoT radio.

Considered that the inventory can be sent by the A-IoT CN for a group of devices or all devices, we believe the command could be targeted at a group of devices or all devices. The scenario of reading or writing is commonly used in AIoT deployment, e.g., sellers want to retrieve the product information of a group of devices, or sellers may update the device IDs for a group of sold items.

Observation 3: The scenario of reading or writing a group of devices or all devices is commonly used in AIoT deployment, e.g., sellers want to retrieve the product information of a group of devices, or sellers may update the device IDs for a group of sold items.

Proposal 4: It is proposed that command over CN-RAN interface can be sent for a group of devices or all devices.

RAN2#127:

1	At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

Based on RAN2 latest agreements, they think some information is useful to be visible to the reader from CN. From RAN3 perspective, it is necessary to check whether above information is worth to be visible from CN-RAN interface.

Regard to the service type of AIoT, no matter for topology 1 or topology 2, in order for the AIoT RAN node or gNB to allocate resources efficiently for different types of services, it is beneficial for AIoT RAN node and gNB be aware of the service type. Regard to different kinds of procedure, the resource allocation can be different. For example, if the AIoT paging is inventory, AIoT RAN node or gNB will allocate the D2R resource for AIoT devices up to 96bits according to the inventory type. If the AIoT paging is read, AIoT RAN node or gNB will reserve 1000bits at most to retrieve the content from AIoT device. If the AIoT paging is write, AIoT RAN node or gNB will only allocate limited resource to receive the feedback on whether the result of write is successful or not. Therefore, it is necessary to include service type (e.g., inventory, read, write, etc.) in the AIoT paging message.

Observation 4: In order for the AIoT RAN node or gNB to allocate resources efficiently for different types of services, it is beneficial for AIoT RAN node and gNB be aware of the service type of AIoT paging.

Proposal 5: Service type (e.g., inventory, read, write, etc.) should be included in the AIoT paging message.

As RAN2 mentions the information on targeted for one or more than one device is useful to be visible to the reader from CN, this agreement is made with the consideration that the information may help AIoT RAN node allocate radio resources and RA type selection. If it is targeted for single device, the AIoT RAN node can enable the dedicated device to perform CFRA. If it is targeted for more than one device, the AIoT RAN node enable all devices to perform CBRA.

Observation 5: The information on targeted for one or more than one device helps AIoT RAN node allocate radio resources and RA type selection.

Proposal 6: Targeted for one or more than one device should be included in the AIoT paging message.

For the estimated number of AIoT devices, if the number is available, it can be informed from CN to AIoT RAN node. It is not mandatory to include the approximate number in AIoT paging message. The AIoT RAN node can always estimates the device number by itself. Therefore, the estimated number of target AIoT devices is not mandated in AIoT paging message.

Proposal 7: The estimated number of target AIoT devices is not mandated in AIoT paging message.

RAN2#127:

1	Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

During RAN2 discussion, triggering multiple A-IoT paging messages which are associated with the same request from CN should be supported considering the device unavailability caused by energy status. However, each paged device is expected to respond only once for one request from CN, otherwise, duplicated response may case more collisions and greatly decrease the efficiency of the A-IoT system. To avoid the duplicated response from devices, an identifier representing the CN request, such as session ID, can be included in A-IoT paging message over both AIoT radio and XXAP to identify separate request from different CNs or one CN.

Observation 6: To avoid the duplicated response from devices, an identifier representing the CN request, such as session ID, can identify separate request from different CNs or one CN.

Proposal 8: The session ID should be included in AIoT paging message.

2.3	Other assistance information

Furthermore, when reviewing the legacy paging procedure, the paging triggered by CN is broadcasted among all cells within the corresponding TAI list, while the paging triggered by RAN is broadcasted among all cells within the RAN Notification Area (RNA). For AIoT, it has been assumed that the device will not support tracking/RAN area update procedure. To improve the efficiency of AIoT paging, it is beneficial for CN to maintain an association relationship between A-IoT devices and associated area. To support the association, something like AIoT paging area composed of AIoT readers can be included in AIoT paging message. The CN can inventory the AIoT device(s) within this specific area instead of always inventorying all A-IoT devices.

Observation 7: To improve the efficiency of AIoT paging, it is beneficial for CN to maintain an association relationship between A-IoT devices and associated area.

Proposal 9: AIoT paging area should be included in AIoT paging message.

Moreover, sometimes the inventory requirements from the server are periodic. Naturally, the CN will also provide the RAN with inventory periodicity, indicating how often inventory or command should be performed.

Proposal 10: The inventory periodicity may be included in AIoT paging message.

3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the signalling and procedure for Topology 1:

Observation 1: During the “command after inventory” case, for identical device, AIoT RAN node may utilize the same resource for inventory and command.

Observation 2: The type of device ID in AIoT paging message over XXAP can follow the same design of AIoT paging over AIoT radio.

Observation 3: The scenario of reading or writing a group of devices or all devices is commonly used in AIoT deployment, e.g., sellers want to retrieve the product information of a group of devices, or sellers may update the device IDs for a group of sold items.

Observation 4: In order for the AIoT RAN node or gNB to allocate resources efficiently for different types of services, it is beneficial for AIoT RAN node and gNB be aware of the service type of AIoT paging.

Observation 5: The information on targeted for one or more than one device helps AIoT RAN node allocate radio resources and RA type selection.

Observation 6: To avoid the duplicated response from devices, an identifier representing the CN request, such as session ID, can identify separate request from different CNs or one CN.

Observation 7: To improve the efficiency of AIoT paging, it is beneficial for CN to maintain an association relationship between A-IoT devices and associated area.

Proposal 1: The AIoT CN can initiate a class-1 command procedure to the AIoT RAN node.

Proposal 2: The AIoT RAN node can send an Inventory failure message to reject the inventory request from AIoT CN due to an ongoing process.

Proposal 3: The device XXAP ID, consisting of CN device XXAP ID and RAN device XXAP ID, should be separately stored in AIoT RAN node and CN as the device context.

Proposal 4: It is proposed that command over CN-RAN interface can be sent for a group of devices or all devices.

Proposal 5: Service type (e.g., inventory, read, write, etc.) should be included in the AIoT paging message.

Proposal 6: Targeted for one or more than one device should be included in the AIoT paging message.

Proposal 7: The estimated number of target AIoT devices is not mandated in AIoT paging message.

Proposal 8: The session ID should be included in AIoT paging message.

Proposal 9: AIoT paging area should be included in AIoT paging message.

Proposal 10: The inventory periodicity may be included in AIoT paging message.

4	Reference

[1] 	RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2]	RP-234058, Time_budget_request_R19_Ambient_IoT_SID