3GPP TSG-RAN WG3 Meeting #125bis	             R3-245533

Hefei, CN, 14th – 18th Oct 2024

Agenda Item:	16.4

Source:	CMCC

Title:	Discussion on locating Ambient IoT device

Document for:	Discussion

1	Introduction

The current TR related to locating Ambient IoT device is shown as follows:

### 6.9.x General Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network. A-IoT device location information may be used for the following purposes: (a) improving the A-IoT operation itself, e.g. by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the A-IoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device. Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS. Editor’s Note 2: Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study. Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study. Editor’s Note 4: Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

In this contribution, we will try to solve the questions mentioned in Editor’s Note in TR.

2	Discussion

On behalf of the device location information, the reader ID needs to be perceived by the network. The first question needs to be solved is how to provide the reader ID of the AIoT device to the AIoT CN as described in Editor’s Note 2.

In topology 1, the AIoT RAN node plays the role of the reader and the AIoT RAN node ID can be regarded as the reader ID. During the XX Interface Setup procedure, the AIoT RAN node will inform its AIoT RAN node ID to AIoT CN. Then, the AIoT RAN node does not need to report its ID during the future inventory or command procedure as the AIoT CN already knows where the report message is originated from.

Observation 1: The AIoT RAN node does not need to report its ID during the inventory or command procedure as the AIoT CN already knows where the report message is originated from during the XX interface Setup procedure.

Proposal 1: In topology 1, the AIoT RAN node will inform its AIoT RAN node ID to AIoT CN during the XX Interface Setup procedure.

In topology 2, UE plays the role of the reader and the UE ID is already agreed to consider as the reader ID. Currently, three solutions are under discussion for topology 2, the difference between these three solutions lies in whether the gNB perceives the device location information.

Regard to the RRC-based solution, the UE is known by gNB since the UE needs to collect the device ID and to report them by RRC message. It is assumed that the UE ID along with the collected device ID are reported from gNB to AIoT CN via Inventory Report procedure. Then, the AIoT CN is aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and gNB i.e., the UE is under the control of which gNB.

Regard to the NAS-based solution and UP-based solution, as the signalling or data is terminated in AIoT CN and transparent to the gNB, the gNB does not know the device location information. The AIoT CN collects the NAS packets or the PDU sessions reported from UE via the inventory report or command response message from UE. After the the AIoT CN receives the message from the UE, it automatically knows the UE reader ID.

Observation 2: For RRC-based solution, the AIoT CN is aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and NG-RAN node.

Proposal 2: Regard to the RRC-based solution for topology 2, the UE ID along with the collected device ID are reported from gNB to AIoT CN via Inventory Report procedure.

Proposal 3: Regard to the NAS-based solution and UP-based solution for topology 2, the AIoT CN automatically knows the UE reader ID after receiving the inventory report or command response message from UE.

Editor’s Note 1 is asking how to know the “reader” location for AIoT CN. For the topology 1, the reader is AIoT RAN node, which is stable and not move. The way for AIoT CN getting the reader location is configured by OAM or by reporting GNSS information from AIoT RAN node during XX Interface Setup procedure. For topology 2, UE may be stationary or moving, so the movement of UE should be considered. The AIoT CN can trigger LCS to obtain the UE’s location using existing positioning method.

Proposal 4: For topology 1, the way for AIoT CN getting the reader location is configured by OAM or by reporting GNSS information from AIoT RAN node during XX interface Setup procedure.

Proposal 5: For topology 2, the AIoT CN can trigger LCS to obtain the UE’s location using existing positioning method.

Editor’s Note 1 also states that whether to use more than one “readers” within one A-IoT RAN for location purposes. From the note, it implies that more than one “readers” are within one AIoT RAN, which reflects the proposals provided by some companies that the TRP within one AIoT RAN node can be considered as a kind of location information for AIoT device. Followed by this way, a TRP is assumed as a reader within A-IoT RAN node. From our view, RAN3 first needs to confirm whether one AIoT RAN node may support multiple readers is a real case for future deployment. If available, RAN3 will decide whether the TRP can be considered as the reader of the AIoT RAN node. If the TRP can be seen as a reader, it indeed provides a finer reader granularity for locating an Ambient device besides the AIoT RAN node ID, which gives a clear answer to the Editor’s Note 3.

Proposal 6：RAN3 needs to confirm whether one AIoT RAN node may support multiple readers is a real case for future deployment, if available, RAN3 will decide whether the TRP can be considered as the reader of the AIoT RAN node.

Even if TRP is agreed to be considered as a finer reader granularity of the AIoT RAN node, we think RAN3 should avoid to utilize more readers for locating an AIoT device in this release. If multiple readers are used, it seems that some company support to use positioning method with multiple TRPs or multiple readers to measure the power of the backscattered signal or the output power of the carrier wave. The utilization of more than one “reader” for location purposes will bring new positioning method, which needs additional work for other working groups to evaluate the performance of AIoT device and check the feasibility of new positioning method.

Observation 3: The utilization of more than one “reader” for location purposes will bring new positioning method, which needs additional work for other working groups to evaluate the performance of AIoT device and check the feasibility of new positioning method.

As a reference, the SID is shown as follows:

E. Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). …. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). …. - RAN3-led: - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

In additional, the traffic types for device will be restricted on indoor inventory and indoor command. There is no description in SID about traffic type for sensing or positioning, so measuring the strength of the backscattered signal or the strength of the signal transmitted by AIoT devices is not in the scope of Rel-19 AIoT discussion.

Observation 4: The traffic types for device will be restricted on indoor inventory and indoor command and no description in SID about traffic type for sensing or positioning, so measuring the strength of the backscattered signal or the strength of the signal transmitted by AIoT devices is not in the scope of this release discussion.

Proposal 7: RAN3 should avoid to utilize more readers for locating an AIoT device in this release.

Regard to the Editor’s Note 4, the SID points out that RAN3 should identify potential solutions for locating an Ambient IoT device with no specification impact by reusing existing user location report, or minimal specification impact to convey location information to CN. From current NGAP, the User Location Information IE is included in multiple procedures from gNB to AMF for UE location reporting. The location information in User Location Information is represented as cell ID and TAI. Likewise, considered that locating AIoT device, the AIoT RAN node ID or UE ID is enough to convey location information to CN. Hence, there is no gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

Observation 5: From current NGAP, the User Location Information IE is included in multiple procedures from gNB to AMF for UE location reporting. The location information in User Location Information is represented as cell ID and TAI. Likewise, considered that locating AIoT device, the AIoT RAN node ID or UE ID is enough to convey location information to CN.

Proposal 8: There is no gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

3	Conclusion

Based on the discussions mentioned above, we provide some observations and proposals on locating Ambient IoT device:

Observation 1: The AIoT RAN node does not need to report its ID during the inventory or command procedure as the AIoT CN already knows where the report message is originated from during the XX interface Setup procedure.

Observation 2: For RRC-based solution, the AIoT CN is aware the mapping of device and UE reader, i.e., the device is under the control of which UE reader, and the mapping of UE reader and NG-RAN node.

Observation 3: The utilization of more than one “reader” for location purposes will bring new positioning method, which needs additional work for other working groups to evaluate the performance of AIoT device and check the feasibility of new positioning method.

Observation 4: The traffic types for device will be restricted on indoor inventory and indoor command and no description in SID about traffic type for sensing or positioning, so measuring the strength of the backscattered signal or the strength of the signal transmitted by AIoT devices is not in the scope of this release discussion.

Observation 5: From current NGAP, the User Location Information IE is included in multiple procedures from gNB to AMF for UE location reporting. The location information in User Location Information is represented as cell ID and TAI. Likewise, considered that locating AIoT device, the AIoT RAN node ID or UE ID is enough to convey location information to CN.

Proposal 1: In topology 1, the AIoT RAN node will inform its AIoT RAN node ID to AIoT CN during the XX Interface Setup procedure.

Proposal 2: Regard to the RRC-based solution for topology 2, the UE ID along with the collected device ID are reported from gNB to AIoT CN via Inventory Report procedure.

Proposal 3: Regard to the NAS-based solution and UP-based solution for topology 2, the AIoT CN automatically knows the UE reader ID after receiving the inventory report or command response message from UE.

Proposal 4: For topology 1, the way for AIoT CN getting the reader location is configured by OAM or by reporting GNSS information from AIoT RAN node during XX interface Setup procedure.

Proposal 5: For topology 2, the AIoT CN can trigger LCS to obtain the UE’s location using existing positioning method.

Proposal 6：RAN3 needs to confirm whether one AIoT RAN node may support multiple readers is a real case for future deployment, if available, RAN3 will decide whether the TRP can be considered as the reader of the AIoT RAN node.

Proposal 7: RAN3 should avoid to utilize more readers for locating an AIoT device in this release.

Proposal 8: There is no gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

4	Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA;

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN (Release 18)