3GPP TSG-RAN WG3 Meeting #125bis	R3-245534

Hefei, CN, 14 – 18 Oct, 2024

Agenda Item:	10.4

Source:	CMCC, Huawei, CATT, Ericsson, Lenovo, ZTE

Title:	MHI/UHI Enhancement for SCG Deactivation/Activation

Document for:	Discussion

# 1 Introduction

In last RAN2#127 meeting, potential MHI/UHI Enhancement for SCG Deactivation/Activation was discussed with following agreements:

It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

From the RAN2 LS, they provide two options, which are called network-based solution and UE-based solution shown as below, to collect the time spent information.

Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state. Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information).

RAN2 hopes that RAN3 checks if the network-based solution is sufficient enough for network optimization, or a UE-based solution is needed. In this contribution, we will further compare two solutions and provide our analysis.

# 2 Discussion

2.1 MHI and UHI

The Mobility History Information (MHI) and UE History Information (UHI) for Standalone have been supported in Rel-16, where UE or NG-RAN node records the mobility history information of maximum of 16 most recently visited primary cells or time spent in any cell selection state and/or camped on any cell state in NR or E-UTRA. Then in Rel-17, MHI and UHI enhancement for Dual Connectivity are introduced. The MHI includes the mobility history information of maxPSCellHistory most recently visited primary secondary cell group cells and time spent in the PSCell across all the primary cells. The most recently visited cell is stored first in the list. The list includes cells visited in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED states for NR and RRC_IDLE and RRC_CONNECTED for E-UTRA. Meanwhile, the UHI includes the mobility history information of visited PSCells list, which is consist of the PSCell ID and time spent in the PSCell.

UE report MHI in MobilityHistoryReport, while UHI is transferred from source NG-RAN node to the target NG-RAN node during mobility. Once receiving the MHI and UHI, the network could understand coverage of the wireless network and adjust the mobility related parameters, e.g., A3-offset, hysteresis, timeToTrigger, and the ping-pong issue or handover failure could be mitigated. Similarly, for SCG of Dual Connectivity, the cell id of the PSCell and time spent in the current PSCell while being connected to the current PCell are included in MHI or UHI. Based on the specific information for SCG, the MN/SN could understand coverage of the SN, enable fast DC setup, and adjust SCG mobility parameters, so that the frequent ping pong PSCell change or PSCell change failure could be mitigated.

Observation 1: The MHI and UHI for SCG help the network understand coverage of the SN, enable fast DC setup, and adjust SCG mobility parameters, so that the frequent ping pong PSCell change or PSCell change failure could be mitigated.

2.2 SCG activation and deactivation

The mechanism for SCG activation and deactivation is introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated. Currently, the UHI and MHI for SCG include no information about SCG activation and deactivation.

In our understanding, the information of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity. The detailed information could be the relative time in SCG activated state. For instance, if the relative time in SCG activated state is quite low, it means that the Dual Connectivity is possibly not so necessary for the UE, and the network might need not configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. On the other hand, if the relative time in SCG activated state is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently.

Observation 2: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

2.3 Network-based solution and UE-based solution

As can be seen in the incoming LS, RAN2 provides network-based solution and UE-based solution. The network-based solution based on UHI will only maintain while the UE is performing mobility in connected state. Otherwise, the information will not be not available in the target NG-RAN node. The UE-based solution based on MHI is only provided from the UE to the network when coming to connected from idle/inactive, and the information is not reported from the UE to the network during connected mode mobility. The NW cannot maintain the PSCell info because it removes all the UE context while UE leaves connected state.

In RAN2, companies have common understanding that it is beneficial for the network having information about time spent in the PSCell in activated state or deactivated state. However, companies do not reach consensus on whether the network-based solution is sufficient enough for network optimization, and if a UE-based solution is required. The discussion on the benefit of the UE-based solution boils down to whether we should support scenarios where the UE is connected with SCG moved to idle and then transition to connected again.

Observation 3: The discussion on the benefit of the UE-based solution boils down to whether we should support scenarios where the UE is connected with SCG moved to idle and then transition to connected again.

The following example is given to illustrate the possible change of UE states and the corresponding information collected by NW in UHI and recorded by UE in MHI respectively:

In Phase 1, the UE works in RRC_CONNECTED state and in the coverage of PCell 1. Initially the UE is configured with PSCell A, and after 30s, it is configured with PSCell B for 20s. Then the UE returns to standalone for 10s. It is observed that, the information recorded in MHI and UHI are the same.

Figure 1: MHI and UHI for UE in RRC_CONNECTED

In Phase 2, the UE goes to RRC_IDLE state for 120s and still in the coverage of PCell 1. It is observed that, the information of 120s in idle is only recorded in the MHI of UE, but such information cannot be recorded by NW, because all the UE context has been removed by the NW.

Figure 2: MHI and UHI for UE in RRC_IDLE

In Phase 3, the UE goes back to RRC_CONNECTED state and configured with PCell 1 and PSCell A for another 60s. It is observed that, the information of the another 60s in RRC_CONNECTED is recorded by UE in MHI with the same entry of PCell 1. But for NW, it has no idea that the same UE comes back, and it will start recording the mobility information with a new UE, i.e. UE2, in its UHI.

Figure 3: MHI and UHI for UE when state transition happens

Therefore, only the UE maintains all the mobility history information for both RRC_CONNECTED state and RRC_IDLE state, the NW only keeps the information of UE in RRC_CONNECTED state. For the UHI of NW, two entries are created for the same UE when UE comes back to RRC_CONNECTED, and the NW could not link the two entries of the same UE for DC optimization, since NW has no idea about the PSCell information neither in previous RRC_CONNECTED state nor in RRC_IDLE state.

Observation 4: Only UE could record the whole mobility history information, including cells visited in RRC_IDLE, and RRC_CONNECTED states. The network could only record the mobility information when UE is in RRC_CONNECTED state, and the information of RRC_IDLE is lost.

Observation 5: NW could not link the Mobility Information of UE when UE transits to RRC_IDLE and comes back to RRC_CONNECTED.

Therefore, from RAN3 point of view, the network-based solution is insufficient for the optimization of dual connectivity configuration, and the UE-based solution is needed. Once the UE-based solution is introduced, the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state, should be include in MHI.

Proposal 1: The network-based solution is insufficient for the optimization of dual connectivity configuration, and the UE-based solution is needed.

Proposal 2: It is proposed to include in MHI the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state.

# 3 Conclusion

This contribution provides our considerations on the MHI/UHI enhancement for SCG activation and deactivation, enabling network to understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, following observations and proposal are made:

Observation 1: The MHI and UHI for SCG help the network understand coverage of the SN, enable fast DC setup, and adjust SCG mobility parameters, so that the frequent ping pong PSCell change or PSCell change failure could be mitigated.

Observation 2: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Observation 3: The discussion on the benefit of the UE-based solution boils down to whether we should support scenarios where the UE is connected with SCG moved to idle and then transition to connected again.

Observation 4: Only UE could record the whole mobility history information, including cells visited in RRC_IDLE, and RRC_CONNECTED states. The network could only record the mobility information when UE is in RRC_CONNECTED state, and the information of RRC_IDLE is lost.

Observation 5: NW could not link the Mobility Information of UE when UE transits to RRC_IDLE and comes back to RRC_CONNECTED.

Proposal 1: The network-based solution is insufficient for the optimization of dual connectivity configuration, and the UE-based solution is needed.

Proposal 2: It is proposed to include in MHI the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state.

# Reference

[1]	Draft_R2_127_Meeting_Report

[2]	R2-2407664, LS on MHI enhancement solution for SCG deactivation/activation, CMCC