3GPP TSG-RAN3 Meeting #125-bis	R3-245547

# Hefei, China, 14th October - 18th October

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1799 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | X2 correction for Resource Status Reporting Initiation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Jio, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON,TEI15 |  |  |  |  |  | Date: |  |  | 2024-10-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For Resource Status Reporting Initiation procedure, the receiver behavior is not clear in terms of which cells shall be considered for the reporting of requested cell specific measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Resource Status Reporting Initiation procedure, specify that, if eNB2 receives a request from eNB1 to report cell specific measurements, the eNB2 shall provide to the eNB1 the admitted cell-specific measurements only for the cells listed for which reporting has been requested. Impact Analysis Impact assessment towards the previous version of the specification (same release): this CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change clarifies the receiver behavior for an existing function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The receiver behavior is not clear for resource status reporting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

### 8.3.6	Resource Status Reporting Initiation

#### 8.3.6.1	General

This procedure is used by an eNB to request the reporting of load measurements to another eNB.

The procedure uses non UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from eNB1 to eNB2. Upon receipt, eNB2:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	if supported, stop cell measurements and terminate the reporting for cells indicated in the Cell To Report IE list, in case the Registration Request IE is set to "partial stop"; or

-	if supported, add cells indicated in the Cell To Report IE list to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add".

If the eNB2 received a RESOURCE STATUS REQUEST message, which includes the Registration Request IE set to "stop", the Cell To Report IE list shall be ignored.

If the Cell To Report IE is included in the RESOURCE STATUS REQUEST message, the eNB2 shall consider the request for measurements valid only for the cells listed in the Cell To Report IE. The eNB2 shall report the admitted measurements only for the cells listed in the Cell To Report IE.

If the Registration Request IE is set to "start" then the Report Characteristics IE shall be included in RESOURCE STATUS REQUEST message. The eNB2 shall ignore the Report Characteristics IE, if the Registration Request IE is not set to "start".

The Report Characteristics IE indicates the type of objects eNB2 shall perform measurements on. For each cell, the eNB2 shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the S1 TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

-	the ABS Status IE, if the fifth bit, "ABS Status Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1 and eNB1 had indicated the ABS pattern to eNB2;

-	the RSRP Measurement Report List IE, if the sixth bit, "RSRP Measurement Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the CSI Report IE, if the seventh bit, "CSI Report Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

-	the Radio Resource Status IE within the NR Neithbour Cell Measurement Result IE, if the eighth bit, "Neighbour Cell CAC Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1.

NOTE:	In order to avoid duplication, the eNB2 may include only one copy of Measurement Result for NR Cells Possibly Aggregated Item per RESOURCE STATUS UPDATE messages per NR cell, even if this NR cell possibly aggregated to multiple E-UTRA cells served by eNB2. The eNB2 should only include Measurement Result for NR Cells Possibly Aggregated Item for NR cells that may be aggregated with at least one cell served by eNB1.

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the time interval between two subsequent RESOURCE STATUS UPDATE messages that include the Radio Resource Status IE, S1 TNL Load Indicator IE, Hardware Load Indicator IE, Composite Available Capacity Group IE, or ABS Status IE.

If the Reporting Periodicity of RSRP Measurement Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the RSRP Measurement Report List IE.

If the Reporting Periodicity of CSI Report IE is included in the RESOURCE STATUS REQUEST message, eNB2 shall use its value as the minimum time interval between two subsequent RESOURCE STATUS UPDATE messages that include the CSI Report IE.

If eNB2 is capable to provide all requested resource status information, it shall initiate the measurement as requested by eNB1, and respond with the RESOURCE STATUS RESPONSE message.

If eNB2 is capable to provide some but not all of the requested resource status information and the Partial Success Indicator IE is present in the RESOURCE STATUS REQUEST message, it shall initiate the measurement for the admitted measurement objects and include the Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE message.

#### 8.3.6.3	Unsuccessful Operation

Figure 8.3.6.3-1: Resource Status Reporting Initiation, unsuccessful operation

If none of the requested measurements can be initiated, eNB2 shall send a RESOURCE STATUS FAILURE message. The Cause IE shall be set to an appropriate value e.g. "Measurement Temporarily not Available" or "Measurement not Supported For The Object" for each requested measurement object. The eNB may use the Complete Failure Cause Information IE to enhance the failure cause information per measurement in the RESOURCE STATUS FAILURE message.

#### 8.3.6.4	Abnormal Conditions

If the initiating eNB1 does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the eNB1 may reinitiate the Resource Status Reporting Initiation procedure towards the same eNB, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message.

If the initiating eNB1 receives the RESOURCE STATUS RESPONSE message including the Measurement Initiation Result IE containing no admitted measurements, the eNB1 shall consider the procedure as failed.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then eNB2 shall initiate a RESOURCE STATUS FAILURE message, the cause shall be set to appropriate value e.g. "ReportCharacteristicsEmpty".

If the Reporting Periodicity IE value is not specified when at least one of the bits of the Report Characteristics IE, for which semantics is specified, other than the sixth or seventh bit, is set to 1 then eNB2 shall initiate a RESOURCE STATUS FAILURE message, the cause shall be set to appropriate value e.g. "NoReportPeriodicity".

If the Reporting Periodicity of RSRP Measurement Report IE value is not specified when the sixth bit of the Report Characteristics IE is set to 1, then eNB2 shall initiate the RESOURCE STATUS FAILURE message and the cause shall be set to appropriate value e.g. "NoReportPeriodicity".

If the Reporting Periodicity of CSI Report IE value is not specified when the seventh bit of the Report Characteristics IE is set to 1, then eNB2 shall initiate the RESOURCE STATUS FAILURE message and the cause shall be set to appropriate value e.g. "NoReportPeriodicity".

If the eNB2 received a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the eNB1Measurement ID IE corresponding to an existing on-going load measurement reporting, then eNB2 shall initiate a RESOURCE STATUS FAILURE message, the cause shall be set to appropriate value e.g. "ExistingMeasurementID".

If the Registration Request IE is set to "stop", "partial stop" or "add" and the RESOURCE STATUS REQUEST message does not contain eNB2 Measurement ID IE, eNB2 shall consider the procedure as failed and respond with the RESOURCE STATUS FAILURE message, the cause shall be set to appropriate value e.g. "Unknown eNB Measurement ID".

If the Registration Request IE is set to "partial stop" and the Cell To Report IE contains cells that have not been initiated for the reporting before, eNB2 shall consider the procedure as failed and respond with the RESOURCE STATUS FAILURE message, the cause shall be set to appropriate value e.g. "Cell not Available". If the Registration Request IE is set to "add" and the Cell To Report IE contains cells that have been initiated for the reporting before, eNB2 shall consider the procedure as failed and respond with the RESOURCE STATUS FAILURE message, the cause shall be set to appropriate value e.g. "Cell not Available".

End of Changes