3GPP TSG-RAN WG3 Meeting #125-bis	R3-245557

Hefei, China, 14 – 17 October, 2024

Agenda Item:	11.3

Source:	Nokia, Jio, Telecom Italia

Title:	Scenarios and open points for AI/ML-based Coverage and Capacity Optimization (CCO)

Document for:	Discussion

# 1	Introduction

The present work item is based on WID in [1] and TR 38.743 which captures the outcome of the preceding Rel-19 study item on enhancements for AI/ML in NG-RAN. We here provide initial discussion on the Rel-19 normative work for support of AI/ML-based CCO, starting with description of the work item objective on CCO. We include a description of the CCO scenarios to be covered by this normative work, taking into account that the study item discussions focused mainly on aspects related to RAN architecture and AI/ML framework to be used. We then discuss timing aspects and future CCO state recommendation, which were identified as open points in the TR.

# 2	Discussion

## 2.1	Overview of work item objective for CCO

RAN#105 (September 2024) has agreed the following Rel-19 objective for CCO within work item description for AI/ML for NG-RAN [1]:

- Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Slicing and AI/ML based CCO. [RAN3]

As per outcome of the study item, captured in TR 38.743 clause 4.2, the Rel-19 work shall be based on the CCO framework defined in Rel-17 and described in TS 38.300 and TS 38.401.

The use case description for AI/ML based CCO TR 38.743 refers to the term “CCO issue” without further definition:

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

TS 38.300 describes the CCO issues using two examples:

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues, e.g. coverage and cell edge interference issues.

In stage 3, the F1AP CCO issue detection IE is defined as follows:

| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the type of CCO issue detected, or network energy saving cause. |
| --- | --- | --- | --- | --- |

It can be observed that this IE contains three code-points: coverage, cell edge capacity and network energy saving. It should be noted that the network energy saving code-point was added in Rel-18 in order to simplify co-existence of CCO and network energy saving in F1AP and XnAP signalling. The same three code-points are replicated in the Coverage Modification Cause IE (F1AP and XnAP). So from this it seems clear that Rel-19 AIML-based CCO shall support two CCO issues, i.e. coverage and cell edge capacity.

Proposal 1: RAN3 to confirm that Rel-19 AIML-based CCO supports two CCO issues, i.e. coverage and cell edge capacity.

An example of a coverage issue may be a coverage hole appearing between two cells (Figure 1a). This situation, and a corresponding coverage change to compensate for this coverage issue (Figure 1b), may be illustrated as follows:

Figure 1: (a) illustrates a coverage hole (coverage issue) between cell A and B. (b) illustrates an updated CCO state of cell B, compensating for the coverage hole.

Similarly, with reference to Figure 2, a cell edge capacity issue may occur if a high number of UEs are located within the area where two cells overlap, hence in the edges of cells A and B as illustrated in  Figure 2a. Some of these UEs may be served by cell A and some may be served by cell B. This will result in both DL and UL inter-cell interference leading to a cell edge capacity issue. A possible way to solve this issue can be to move the cell edge area away from the area with high concentration of UEs. In this case one of the cells (cell B in this example) will be in charge of serving the concentration of the UEs as show in Figure 2b, and can autonomously coordinate the radio resources and hence avoid or reduce UL and DL inter-cell interference issues.

Figure 2: (a) illustrates a scenario of high concentration of UEs in cell edge leading to cell edge capacity issue. (b) illustrates updated CCO states of cell A and, moving the cell edge and hence compensating for the cell edge capacity issue.

## 2.2	Timing information for predicted CCO issue and future CCO state

The study item concluded that predicted CCO issue and future CCO state will be provided with associated timing information, but there was no agreement whether signalling should include common or separate timing information for the predicted CCO issue and the future CCO state.

Before answering this question let us first understand the significance of timing information in each case. Timing information for a CCO issue may represent

•	prediction of when the CCO issue will occur (predicted start time)

•	prediction of for how long time the CCO issue will last (predicted issue duration)

The predicted start time was subject to most attention and discussion at last meeting, but the predicted issue duration can also be relevant information in case of a transitory event like the cell edge capacity issue due to a high concentration of UEs as illustrated in Figure 2. A CCO state reconfiguration process may represent some time delay or energy cost (or gain) depending on the targeted new CCO state. In case of a transitory event, the gNB-DU may therefore use the predicted issue duration information to determine the new coverage configuration to use to solve the issue, by taking into account the delay and potential energy cost or gain related to the reconfiguration. A very short duration of the CCO issue could also then lead the gNB-DU to not perform any reconfiguration at all. If the duration of the CCO issue can’t be predicted by the gNB-CU, or the CCO issue is assumed to be permanent, it would be sufficient to not provide any CCO issue duration information together with the CCO issue.

Proposal 2: Timing information for predicted CCO issue includes predicted start time and the predicted issue duration.

Figure 3 illustrates a transitory CCO issue with associated predicted start time and duration.

Figure 3: CCO state reconfiguration in case of transitory CCO issue (e.g. due to transitory event like UE concentration in cell edge as in Figure 2).

As agreed in study item phase, the future CCO state, derived at the gNB (gNB-DU in split architecture) is shared with neighbouring gNB(s) over Xn. This is done in order to enable the neighbouring nodes to adjust their coverage by applying a new matching coverage state, and also for the purpose of MRO. Now the crucial information for neighbouring gNB is not only to know the future CCO state of a neighbour, but also when it plans to apply it. For this, the gNB sharing the future coverage state over Xn is also including the timing information when the said future CCO state is planned to be applied, denoted as activation time in Figure 3. A further question is whether there is benefit to additionally provide the activation duration, i.e. the time duration when the new CCO state will be applied by the gNB-DU as illustrated in the figure. However, this would come with an assumption of fallback to the initial CCO state after the end of the issue, and therefore correspond to predictions not available in the framework described in TR 38.743.

We also believe that this should not be a real shortcoming with the described framework, considering that the gNB-CU will evaluate whether, after the end of the predicted CCO issue, the CCO state 2 could remain or whether there is a need to switch back to e.g. CCO state 1 or some other CCO state x. Based on this, any decision in the gNB-DU about activation duration could come in conflict with the outcome of the next cycle of prediction taking place in the gNB-CU. The activation time is therefore sufficient in terms of timing information for activation of the new CCO state.

Proposal 3:  The activation time is sufficient in terms of timing information for activation of the new CCO state. There is no need to signal activation duration over network interfaces.

It is worth mentioning that in case of split architecture, the future CCO state and the activation time should also be sent by gNB-DU to gNB-CU.

The question of whether the activation time and predicted start time information is same or not remains. However, the TR provides in clause 4.2.1 some motivation to resolve the predicted CCO issue before its occurrence i.e. to prevent the CCO issue from happening:

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

In case it is assumed that the activation time and prediction time are same it forces the future CCO state to be applied at the point in time when the CCO issue is predicted to occur. And in split architecture, using same time information would in practice mean that the gNB-DU would have to apply the CCO state change at the point in time where the gNB-CU predicts the CCO issue to occur. In this case there could be some hiccups e.g., activation of future CCO state may take some time, depending on the antenna equipment used, and due to this the UEs being served by the affected cells/beams get affected, whereas, in case if activation time is allowed to be earlier than the prediction time then the NG RAN can find out the best activation time that minimizes adverse effects on the served UEs. Therefore, activation time and prediction time should be different and the activation should in principle occur before the predicted start time of the CCO issue.

Proposal 4: Predicted CCO issue and future CCO state are provided with separate timing information.

## 2.3	Definition and dimensioning of timing information

The normative work on IE definitions will require detailed definition of the timing information, as well as dimensioning (e.g. overall range, and in a further step encoding and range of the IE). The work on elaborating definitions will need to take into account that some issues, e.g. cell edge capacity issues, may arise gradually. This aspect is accounted for in the use case description captured in the TR: “a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance”.

For this initial discussion we propose the following definitions of the timing information proposed in section 2.2 based on Figure 3.

Predicted Start Time:	The point in time where the CCO issue is expected to occur. For CCO issues occurring gradually, it represents a point in time of an early stage of the CCO issue.

Predicted Duration: The expected duration of the CCO issue, starting at the Predicted Start Time.

Activation Time: The expected point in time where the new CCO state will take effect.

Proposal 5: RAN3 to discuss and agree upon definitions of timing information, e.g.: Predicted Start Time: The point in time where the CCO issue is expected to occur. For CCO issues occurring gradually, it represents a point in time of an early stage of the CCO issue.  Predicted Duration: The expected duration of the CCO issue, starting at the Predicted Start Time. Activation Time: The expected point in time where the new CCO state will take effect.

The exact starting point for Predicted Start Time and Activation Time will need to be further defined, e.g. with reference to sending or receiving F1AP/XnAP messages carrying this information. This aspect could therefore be further refined in later meetings when detailed signalling messages have been defined.

The dimensioning of the timing information discussed in section 2.2 will need to take into account the availability of input information, and in particular availability of information conveyed on network interfaces. TR 38.743 indicates the following input information conveyed over Xn for CCO issue prediction:

From neighbouring gNBs:

-	Measured/Predicted radio resource status

The following range and definition for requested prediction time was agreed in Rel-18 (XnAP):

| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) |
| --- | --- | --- | --- | --- |

This means that the maximum prediction time currently available for predicted radio resources is 60 seconds.

There was no time in the study item to discuss the time scale of CCO issues, e.g. whether a prediction horizon within the Rel-18 agreement of 1 minute is sufficient or whether consideration of a longer time scale could have benefits and would be feasible. In this context we also notice that the list of input information for the AI/ML model, as captured in the TR, is very short, and in particular doesn’t include predicted UE trajectory. The latter allows for predictions well beyond 1 minute. We believe that the predicted UE trajectory information is valuable information both for prediction of coverage issue (Figure 1) and cell edge capacity issue (Figure 2). Information about incoming UE traffic from neighbour cells, e.g. predicted incoming data volume, would also be valuable information in our view to predict cell edge capacity issue.

Proposal 6: RAN3 to take into account additional input information for AI/ML-based CCO, including predicted UE trajectory information and predicted incoming UE traffic (e.g. data volume) from neighbour cells.

Furthermore, because CCO issues are often determined based on variations in UE concentrations and the Rel-19 work targets to enable proactive mitigation of such issues, it seems to us that a horizon of 5 or 10 minutes should be considered for the Predicted Start Time illustrated in Figure 3. However this would be conditioned on the possibility to include additional input information for AI/ML-based CCO as indicated in the proposal above, and will also potentially require enhancement of the Rel-18 prediction horizon at least for Predicted radio resource status.

Proposal 7: RAN3 to discuss benefits and feasibility of prediction horizon of 5 or 10 minutes for CCO issue prediction (Predicted Start Time).

Dimensioning of other timing parameters, e.g. Activation Time and Predicted Issue Duration illustrated in Figure 3, will depend on the dimensioning on the Predicted Start Time. These additional parameters are also currently pending  RAN3’s confirmation.

## 2.4	Recommended CCO state

In legacy CCO depoyed in split architecture, the gNB-DU may derive the new CCO state based on configured information, the CCO issue signalled by the gNB-CU, the current CCO state as well as other information available in the gNB-DU. This information may be based on L1 and L2 information available for UEs that the gNB-DU is serving.

For AIML-based CCO, the corresponding mechanism based on predicted CCO issue is described as follows in the TR:

NOTE 5:	For split architecture, future CCO State is derived based on the predicted CCO issue signaled from gNB-CU and other information in the gNB-DU.

In order to allow for further investigation on providing additional information to the gNB-DU, the following was captured in the TR:

NOTE 4:	Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion.

For what concerns “other information in the gNB-DU” as indicated in NOTE 5 copied above, it can be observed that as per current standard no prediction information is available in the gNB-DU, and that the use of available measured L1/L2 information may not be relevant for analysis of a future CCO issue. In our paper submitted to previous meeting [2], we provided support for making additional information available in the gNB-DU by letting the gNB-CU generate the suggested future CCO: “the gNB-CU could send the predicted (future) CCO state to the gNB-DU as a suggested future CCO state together with time information indicating when the CCO state should be applied. The final decision to apply the suggested CCO state could then still be taken by the gNB-DU. Furthermore, in this case the CCO state prediction would benefit from feedback information already available in the gNB-CU.”

An additional advantage with this approach is that the CCO state prediction would benefit from feedback information already available in the gNB-CU. We also believe this approach will prepare the grounds for advanced implementations. For example, an AI/ML model based on reinforcement learning could use received feedback to build a reward table and propose the future CCO state e.g. as a simple look-up operation in this reward table.

Proposal 8: The gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information.

# 3	Conclusion

We have made the following proposals:

Proposal 1: RAN3 to confirm that Rel-19 AIML-based CCO supports two CCO issues, i.e. coverage and cell edge capacity.

Proposal 2: Timing information for predicted CCO issue includes predicted start time and the predicted issue duration.

Proposal 3:  The activation time is sufficient in terms of timing information for activation of the new CCO state. There is no need to signal activation duration over network interfaces.

Proposal 4: Predicted CCO issue and future CCO state are provided with separate timing information.

Proposal 5: RAN3 to discuss and agree upon definitions of timing information, e.g.: Predicted Start Time: The point in time where the CCO issue is expected to occur. For CCO issues occurring gradually, it represents a point in time of an early stage of the CCO issue.  Predicted Duration: The expected duration of the CCO issue, starting at the Predicted Start Time. Activation Time: The expected point in time where the new CCO state will take effect.

Proposal 6: RAN3 to take into account additional input information for AI/ML-based CCO, including predicted UE trajectory information and predicted incoming UE traffic (e.g. data volume) from neighbour cells.

Proposal 7: RAN3 to discuss benefits and feasibility of prediction horizon of 5 or 10 minutes for CCO issue prediction (Predicted Start Time).

Proposal 8: The gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information.

# References

[1]	RP-242385, New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, approved at RAN#105

[2]		R3-244377, Discussion on how to determine the future CCO state, Nokia, Telecom Italia, Jio, Vodafone