3GPP TSG-RAN WG3 Meeting #125bis	R3-245561

Hefei, China, 14 – 17 October 2024

Agenda item:	17.3

Source:	Nokia

Title:	Further thoughts on open Issues in support of On-Demand SIB1

Document for:	Discussion and Decision

# 1	Introduction

According to [1] the support of on-demand SIB1 for UEs in idle/inactive mode shall be studied by RAN1, RAN2 and RAN3 with a check-point for normative work at RAN#105 (September 2024). In this context RAN1 also evaluates the NES gain achievable by this technique.

The revised WID [2] RP-242354 has now clarified on the requirements based on the discussions in R19 so far.

In this paper we provide our view on these revised requirements, their potential RAN3 impacts and propose solutions in RAN3 for these requirements.

# 2	Discussion

The following were agreed in RAN3#125

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration

- Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Observation-1 : Since the basic UL WUS configuration signalling across F1 and Xn have been agreed, we may move forward and see how this could be realized using actual RAN3 messages.

Accordingly, we look at the existing F1 and Xn messages to accommodate the WUS configuration. Since it is tied to a cell and has similar dynamic nature (ie frequency of updates) similar to other cell configurations, we propose that the WUS configuration be part of the Served Cell Information IE in both these interfaces. Accordingly, we capture the signalling in Fig.1

Fig.1 : Proposed RAN3 Signaling for WUS Configuration

Steps 0-4 show the exchange of initial WUS Configuration using the F1 and Xn Setup procedures

Steps 5-9 show the exchange of updated WUS Configuration information using the gNB-Configuration Update and NG-Configuration Update procedures respectively

Proposal-1: The UL WUS Configuration could be part of the Served Cell Information IE and could reuse the F1 Setup, F1 gNB-DU Configuration Update, Xn Setup and Xn RAN Configuration Update procedures.

The key revised requirements from [2] are as below

Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

- Specify procedures and signaling method(s) for Case 2 [RAN1/2]

- Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

- Triggering method by UL WUS using PRACH.

- Specify inter NG-RAN node signalling at least for the configuration of UL WUS , [RAN3]

The highlighted part shows that along with the WUS Configuration, RAN3 may also need to support other, related information too. Given that OD-SIB1 is a dynamic feature that needs to be activated or deactivated based on various conditions, the most basic additional information to be shared would be the activation status of OD-SIB1 in a given cell.

Observation-2: Information additional to WUS Configuration should be supported by RAN3 and the most important such information would be the OD-SIB1 activation status

We propose the sharing of OD-SIB1 activation status as given in Fig.2

Fig.2 : Proposed RAN3 Signaling for WUS Activation

Key Step Description

The proposed signalling for the OD-SIB1 activation is given in  in Fig.2

The first aspect is which node deciding activation and deactivation of the OD SIB1 mode. There are many possible considerations that would be required to decide if a given cell is to operate in OD-SIB1 or normal, periodic SIB1 mode. These could include measurements like the following

- The existing load conditions in the network (this includes NES cell, Cell A and neighbouring cells)

- For an NES cell already in OD-SIB1 mode, the number of UEs requesting OD-SIB1 or number of slots that transmit OD-SIB1 in a configurable time period could be used to disable OD-SIB1 mode.

As can be observed, these measurements are not limited only to DU level ones, but could include CU level measurements and even measurements from neighbouring gNBs.

Observation-3: The measurements required for taking the decision could be at the DU-level/CU-level and network level too (involving neighbouring gNBs). Such measurements are available at the gNB-CU and hence, the decision also is more appropriate at the gNB-CU of the NES cell.

Additionally, the gNB-CU is in charge of both connected mode and idle mode load balancing, and would need to take into account that a UE not supporting OD-SIB1 mechanism (e.g. a pre-Rel-19 UE) can not camp on a cell in OD-SIB1 mode. This means that the decision to enable/disable OD-SIB1 mode is better done at the gNB-CU of the NES cell rather than at the gNB-DU.

Proposal-2: The gNB-CU of the NES Cell decides to enable OD-SIB1 mode in one or more of its NES cells.

For what concerns the associated signalling, in the example provided in Figure 2, the gNB-CU (NES) sends the activation indication along with the cell IDs to the gNB-CUs of the associated Cell As over Xn. This triggers those gNB-CUs to transmit the corresponding UL-WUS configuration (that was shared earlier in Fig.1) over the air. The gNB-CUs then confirms this action to the gNB-CU of NES Cells in Step 5. Note that the UEs are expected to store the WUS configuration in order to be used when they required SIB1 in the NES cell later.

In Step 6, the gNB-CU (NES Cell) then requests the corresponding gNB-DU over F1 to take the following actions

- Update MIB to indicate that the SIB1 broadcast is stopped

- Stop SIB1 broadcast

It is important to note that the stopping of OD-SIB1 broadcast shall be done in the NES cell only after the Cell A confirms that WUS configuration is being transmitted. Otherwise, there could be a period where UEs have no way of accessing the NES cell at all (ie SIB1 stopped and no WUS configuration available). The acknowledgement messages over Xn and F1 ensure that this situation is prevented.

Observation-4: F1 and Xn signalling are required to ensure that the OD-SIB1 activation is done in a split architecture in a safe manner and UEs have a way of accessing the NES cell at all times.

Proposal-3: The gNB-CU of the NES cell communicates the activation and deactivation of the OD-SIB1 mode with the gNB-CU of Cell A over Xn and its own gNB-DUs over F1.

As can be seen in the earlier discussion, there are multiple scenarios which could trigger the enabling or disabling of OD-SIB1 mode and these triggers could be extended in the future, both in terms of the reason and the handling. This calls for a separate activation/deactivation procedure rather than just SIB1 status indication for making the solution future-proof.

Observation-5: Separate activation/deactivation signalling over Xn/F1 is more future proof to support the different, related scenarios.

Proposal-4: The Xn/F1 signalling could be based on activation/deactivation indication.

We now look at the cardinality rules between NES Cells and Cell As and their impacts on RAN3.

RAN2 has already agreed that the following 2  cardinality options should be supported in RAN2#126 [3]

- A given Cell A could support multiple NES cells.

- A given NES cell could be mapped to multiple Cell As.

In the case where a given Cell A is mapped to multiple NES Cells, the question arises as to which entity (CU or DU) stores and manages this mapping.

For each mapped NES Cell, the corresponding gNB-CU shall send the WUS Configuration and Activation Status over Xn as given in Fig.1 and Fig.2. It could then be appropriate that the gNB-CU (Cell A) then stores and manages the list of mapped NES Cells for each of its Cell A.

Proposal-5: In case of a given Cell A being mapped to multiple NES Cells, the mapping is maintained at the gNB-CU (Cell A).

In the case where a given NES cell is mapped to multiple Cell As, referring to Fig.1 and Fig.2, this would mean that a single F1 message carrying the WUS Configuration and/or OD-SIB1 activation status will have to be forwarded to potentially multiple gNB-CUs over Xn. This could be a potential cell configuration and part of the Served Cell Information IE. Accordingly, the gNB-DU (NES Cell) could maintain this mapping and inform the same to its gNB-CU which then decides on forwarding to the respective gNB-CUs.

Proposal-6: In case of a given NES cell being mapped to multiple Cell As, the mapping is maintained at the gNB-DU (NES Cell).

# 3	Conclusions

Observation-1 : Since the basic UL WUS configuration signalling across F1 and Xn have been agreed, we may move forward and see how this could be realized using actual RAN3 messages.

Proposal-1: The UL WUS Configuration could be part of the Served Cell Information IE and could reuse the F1 Setup, F1 gNB-DU Configuration Update, Xn Setup and Xn RAN Configuration Update procedures.

Observation-2: Information additional to WUS Configuration should be supported by RAN3 and the most important such information would be the OD-SIB1 activation status

Observation-3: The measurements required for taking the decision could be at the DU-level/CU-level and network level too (involving neighbouring gNBs). Such measurements are available at the gNB-CU and hence, the decision also is more appropriate at the gNB-CU of the NES cell.

Proposal-2: The gNB-CU of the NES Cell decides to enable OD-SIB1 mode in one or more of its NES cells.

Observation-4: F1 and Xn signalling are required to ensure that the OD-SIB1 activation is done in a split architecture in a safe manner and UEs have a way of accessing the NES cell at all times.

Proposal-3: The gNB-CU of the NES cell communicates the activation and deactivation of the OD-SIB1 mode with the gNB-CU of Cell A over Xn and its own gNB-DUs over F1.

Observation-5: Separate activation/deactivation signalling over Xn/F1 is more future proof to support the different, related scenarios.

Proposal-4: The Xn/F1 signalling could be based on activation/deactivation indication.

Proposal-5: In case of a given Cell A being mapped to multiple NES Cells, the mapping is maintained at the gNB-CU (Cell A).

Proposal-6: In case of a given NES cell being mapped to multiple Cell As, the mapping is maintained at the gNB-DU (NES Cell).

# References

[1] RP-234065, Rel-19 NES WID

[2] RP-242354, Revised Rel-19 NES WID

[3] R2-2405701 Report from session on V2X/SL, R19 NES and MOB