3GPP TSG-RAN WG3 Meeting #125bis	R3-245562

Hefei, China, 14 – 17 October, 2024

Agenda item:	19.2

Source:	Nokia

Title:	Initial discussion on RAN3 work on Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Document for:	Discussion and Decision

# 1	Introduction

This work is based on work item description “Evolution of NR duplex operation: Sub-band full duplex (SBFD)” in [1] and corresponding “LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation” received from RAN1 at the present meeting in [2].

We here provide initial discussion on RAN3’s part of this work, with focus on stage 2 and signalling impacts in non-split architecture (Xn).

# 2	Information provided to RAN3

A preceding study on evolution of NR duplex operation was done in Rel-18 time frame with outcome captured in TR 38.858, with RAN1 as primary responsible WG and RAN4 as secondary responsible WG. The TR was approved by RAN#102 (December 2023).

The TR provides background information and detailed performance analysis relative to SBFD (sub-band full duplex) operation, gNB-to-gNB CLI (cross-link interference) measurements and inter-gNB CLI handling schemes. The gNB-to-gNB CLI in SBFD can be illustrated as in Figure 1.

Figure 1: gNB-to-gNB cross-link interference in sub-band full duplex operation

For inter-gNB CLI handling, RAN1 has identified coordinated scheduling and spatial coordination as the main areas for discussion. Among the spatial coordination, 2 schemes are considered, namely beam nulling and beam pairing. A description of each of the schemes is as found below:

- Coordinated scheduling: gNBs rely on the time and frequency SBFD configuration from neighboring cells to adjust its scheduling decisions. For instance, a TDD gNB might decide to not use for DL transmissions those resources that overlap with the UL subband of a neighbour SBFD gNB. Among SBFD-capable gNBs, this information is also useful to ensure that the SBFD subbands are correctly configured to avoid potential intra-subband gNB-to-gNB CLI. Additionally, the time/frequency SBFD configuration information is also needed to correctly assign the CSI-RS resources for CSI-RS-based inter-cell UE mobility as well as for CSI-RS-based gNB-to-gNB CLI measurements.

- Beam nulling (FR1): Beam nulling is a spatial coordination technique for gNB-to-gNB CLI mitigation intended for FR1. The intention is that the aggressor gNB estimates the aggressor-victim channel and uses it for the DL precoder design, trying to create a null towards the victim gNB. The inter-gNB CLI handling scheme based on beam nulling implies that the aggressor gNB measures CSI-RS from victim gNB and when detected applies precoder adaptation.

- Beam pairing (FR2): Beam pairing is a spatial coordination technique for gNB-to-gNB CLI mitigation intended for FR2. The intention is that victim gNB identifies the Tx beam that creates the highest CLI and notifies the aggressor gNB. The inter-gNB CLI handling scheme based on beam pairing implies that the victim gNB measures CSI-RS from aggressor gNB, victim gNB reports the results and the aggressor gNB then ensures that the reported beams are not used in SBFD slots.

In their LS [2], RAN1 includes an extract of the WID [1] with RAN3 relevance:

    - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

      - Information exchange among gNBs, including [RAN3]

        - Semi-static cell-specific SBFD time and frequency location configuration

        - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

        - Strongest DL beam information

        - CLI-mitigation request

      - …

      - Note: Without dedicated optimization for dynamic/flexible TDD.

  - …

  - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

  - Notes: Following are assumed for information exchange among gNBs

    - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3

    - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

RAN1 also provides in their LS their agreements in clarification of the RAN3 part of the work item description, and below we also provide our related views.

According to the WID, measurement resource configuration for gNB-to-gNB CLI includes SSBs and/or periodic NZP CSI-RS.

Concerning NZP CSI-RS configuration, RAN1 indicates:

“From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.”

The purpose of the exchanged information described by RAN1 is to allow gNBs to use CSI-RS to perform:

- Power measurements for aggressor gNB identification and/or beam pairing

- Channel measurements for beam nulling

For these reasons, the gNB that performs the measurements must be aware of the CSI-RS configuration used by the potential victim or aggressor gNBs. Currently Xn interface supports the exchange of CSI-RS for UE mobility purposes but there are certain limitations: single port CSI-RS, “no-CDM”.  The existing information could be enough to perform RSRP measurements at the gNB for aggressor gNB identification, but not for channel measurements. It therefore seems needed to support exchange of similar NZP-CSI-RS resource set information over the Xn interface as the one used at the UE to perform CSI measurements for the gNB, specifically for beam nulling purposes.

Furthermore, RAN1 indicates:

“Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB. “

For beam pairing, as a result of the measurements, the measuring (victim) gNB should inform about which beams created the highest gNB-to-gNB CLI (this is our interpretation of “strongest”). The CSI-RS Resource Indicator (CRI) can be used for that. This additional information could be included in the measurement configuration.

Concerning SSB related information, RAN1 indicates:

“Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.”

Similar as for the CSI-RS-based measurements, the SSB index is used to report the SSBs with the highest measured gNB-to-gNB CLI. However, in order to perform measurements on neighbour cell SSBs, own SSB may need to be muted and hence NCD-SSB seems preferable for this purpose.

# 3	Framework and scenarios

A framework for CLI management was introduced in the NG-RAN in Rel-16. The framework is described as follows for non-split architecture in TS 38.300:

## 17.2	Cross-Link Interference Management

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI).

To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations over Xn and F1 interfaces; and the victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements:

-	SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

-	CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

Layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported.

TS 38.401 describes CLI management in split architecture:

## 7.3	Cross-Link Interference Management

The Cross-Link Interference Management function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, the gNB-CU forwards the TDD DL/UL patterns received from neighboring nodes to each concerned gNB-DU. The gNB-DU reports the TDD DL/UL patterns of its serving cells to the gNB-CU if Cross-Link Interference is detected.

The Rel-16 framework applies to dynamic TDD. It can be observed that corrective actions in this framework consists in exchange and coordination of the intended TDD DL-UL configurations over Xn and F1 interfaces, as well as configuration of UE-to-UE CLI measurements resources using SRS-RSRP and CLI-RSSI measurements.

For CLI management in SBFD operation a new framework is needed in order to introduce support for the new techniques of beam nulling and beam pairing described in section 2, and which require inter-gNB exchange of configuration information for CLI-RS resources. Beam pairing additionally requires gNB measurement of Strongest DL beam. According to the WID [1], the information exchange over network interfaces is subject to RAN3’s assessment of feasibility and specification impact.

Within the new framework, there are multiple options possible for transmission of reference signals used for the CLI measurements (SSB or NZP-CSI-RS, which may go under the common name of CLI-RS), corresponding exchange CLI-RS resource information over network interfaces as well as inter-gNB signalling required to trigger CLI mitigation (named CLI Mitigation Request in the WID). RAN1 introduced the concept of CLI mitigation request for beam nulling and it is seen as is a backhaul message initiated by the victim gNB to signal the aggressor gNB when to start applying the beam nulling. Regarding this latter topic, we see the following two main options:

- Option 1 (explicit CLI Mitigation Request): Victim gNB first exchanges the CLI-RS configuration (either for SSB or NZP-CSI-RS or both) with the aggressor gNB. The aggressor gNB measures the channel on the corresponding resources. Once inter-cell CLI is detected, the victim gNB sends a CLI Mitigation Request to the aggressor gNB containing cell id. Upon the reception, the aggressor gNB starts applying the CLI mitigation scheme (i.e., beam nulling).

- Option 2 (implicit CLI Mitigation Request): There is no initial CLI measurement resource exchange. When the victim gNB detects increased CLI level based on UL performance degradation, it triggers CLI measurement resource exchange and activation of CLI-RS transmission. Upon the reception of the CLI-RS configuration, the aggressor gNB is expected to perform measurements and also apply the beam nulling scheme. In this way the signalling of CLI measurement resource information is an implicit trigger of CLI mitigation.

With option 1, CLI-RS are allocated and exchanged between the impacted gNBs in a first step. Then in a second step the victim gNB will explicitly request CLI mitigation. Option 2 merges these two steps, so comes with the advantage of avoiding pre-allocation and periodical transmission of CLI measurement resources prior to the occurrence of the CLI issue. We therefore think option 2 should be prioritized.

Proposal 1: RAN3 to discuss explicit vs. implicit CLI Mitigation Request and prioritize support of the implicit CLI Mitigation Request (option 2).

# 4	Inter-gNB signalling

Based on discussion in the preceding sections, we propose the following signalling flow based on option 2 (implicit CLI Mitigation Request) as possible starting point for RAN3 discussion on XnAP support of beam nulling (Figure 2).

Figure 2: Example of inter-gNB signalling in support of beam nulling (used in FR1). Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originating in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of SBFD time and frequency configuration information.

Step 2: gNB 1 detects CLI based on UL performance degradation.

Step 3: gNB 1 sends its CLI-RS configuration to gNB 2.

Step 4: gNB 1 starts transmitting periodic CSI-RS.

Step 5: gNB2 estimates the victim-to-aggressor channel by measuring the CLI-RS of the victim gNB

Step 6: gNB 2 adjusts the DL precoder if DL transmission uses the identified aggressor beam.

For beam pairing, we propose the following signalling flow as possible starting point for RAN3 discussion (Figure 3). Also this signalling flow is based on option 2 (implicit CLI Mitigation Request):

Figure 3: Example of inter-gNB signalling in support of beam pairing. Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originating in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of: SBFD time and frequency configuration information; SSB information for CLI measurements.

Step 2: If gNB1 detects CLI based on UL performance degradation, the gNB measures SSB-RSRP and identifies one or more aggressor cells / SSB beams.

Step 3: gNB 1 indicates cell ID and SSB index with highest CLI to gNB 2 (potential aggressor gNB).

Step 4: gNB 2 sends CLI-RS configuration to gNB1.

Step 5: gNB 2 starts transmitting periodic CSI-RS.

Step 6: gNB1 performs per beam CLI measurements according to the (potential) aggressor CLI-RS

Step 7: gNB2 indicates the CRI with highest measured gNB-to-gNB CLI to the gNB1

Step 8: gNB2 avoids DL scheduling in the identified CSI-RS beam during all SBFD slots

The message flows in Figure 2 and Figure 3 can be used to identify new solution elements to be introduced in the NG-RAN in Rel-19:

- Exchange over Xn of SBFD configuration

- Exchange over Xn of SSB configuration information for CLI measurements: Our current assumption is that legacy information is sufficient.

- CLI mitigation request information over Xn:

  - Beam nulling: Measurement resource indication (CLI-RS configuration)

  - Beam pairing: Indication of CRI with highest measured gNB-to-gNB CLI

Based on the above, at this initial step, RAN3 may need to discuss and confirm introduction of the following XnAP signalling:

Proposal 2: Introduce Xn signalling for exchange of SBFD configuration information.

Proposal 3: Introduce new XnAP class 2 signalling procedure containing CLI mitigation request information.

# 5	Conclusion

We have made the following proposals:

Proposal 1: RAN3 to discuss explicit vs. implicit CLI Mitigation Request and prioritize support of the implicit CLI Mitigation Request (option 2).

Proposal 2: Introduce Xn signalling for exchange of SBFD configuration information.

Proposal 3: Introduce new XnAP class 2 signalling procedure containing CLI mitigation request information.

# References

[1]		RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), approved at RAN#104

[2]		R1-2407533 - R3-245006, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation, from RAN1 to RAN3 cc RAN2