3GPP TSG-RAN WG3 Meeting #125bis	R3-245566

Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0423 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 correction on LTM with gNB-CU-UP change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 38.401 Section 8.2.1.6 LTM with gNB-CU-UP change shows the procedure used for LTM with the change of gNB-CU-UP within a gNB. The Steps 10-13 are used for CU-CP to initiate the change of CU-UP together with LTM cell switch command and notification procedures, however, such change shall be executed only after the cell switch is successfully confirmed, i.e. after CU-CP receives ACCESS SUCCESS from the target DU. The LTM execution attempt from the UE could fail and another LTM candidate cell under the source DU may be selected for access (called “failure-reattempt”), where the UE would remain served by the source CU-UP. The change of CU-UP before cell switch is successfully confirmed could result in additional unneccessary change of CU-UP back to the source CU-UP (and additional data forwarding if applicable). Moreover, the target DU, once the UE successfully accessed, informs the UE’s access to the target CU-UP via sending initial DDDS frame. This was specified in Rel-15 to fastly inform CU-UP of the UE’s access during legacy mobility, and later extended for conditional mobility family (CHO, CPAC, subsequent CPAC) as a way to initiate DL data delivery from the target CU-UP. As LTM shares the same principle with conditional mobility family from CU-UP funtional point of view, this part was also missing in the figure. Furthermore, the last agreed R3-243090 changed the Step 10-11 from mandatory to optional given that data forwarding is a optional feature. However, according to TS 38.401 Section 8.9.4 Steps 12c-d, during conditional mobility, the Bearer Context Modification procedure toward the source CU-UP after the UE successfully accessed is always executed to indicate the source CU-UP to stop packet delivery to the UE (and while doing so, may retrieve PDCP UL/DL status or exchange TNL address for data forwarding). Given that LTM shares the same principle with conditional mobility, those steps needs to be reverted back to mandatory with the correct descriptions. Lastly, early data forwarding can be applied in LTM from the source CU-UP to the target CU-UP, which is currently missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Revert the optional Step 10-11 back to mandatory with the correct description including indicating the source CU-UP to stop packet delivery. 2. Swap Step 10-13 and Step 15-16, for CU-CP to initiate Step 10-13 after receiving ACCESS SUCCES from the target DU. 3. Add the step of the target DU sending DDDS to the target CU-UP. 4. Add the optional steps (new step 7x) and the description for early data forwarding from the source CU-UP to the target CU-UP. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on LTM with gNB-CU-UP change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure is incomplete. Moreover, unnecessary change of CU-UP may occur when LTM execution attempt from the UE fails and another candidate cell associated with different CU-UP is selected for access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Updated against v18.3.0 and re-submitted to RAN3-125bis. |  |  |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 8.2.1.6	LTM with gNB-CU-UP change

Figure 8.2.1.6-1 shows the procedure used for LTM with the change of gNB-CU-UP within a gNB.

Figure 8.2.1.6-1 LTM with the change of gNB-CU-UP

0.	The source gNB-DU forwards the Measurement Report to the gNB-CU-CP.

1.	The gNB-CU-CP decides to initiate LTM configuration.

2.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for NG-U to setup the bearer context in the target gNB-CU-UP.

3.	The target gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, DL TNL address information for NG-U, and the TNL address information for data forwarding to the target gNB-CU-UP.

4.	LTM configuration procedures are performed between gNB-CU and candidate gNB-DUs, and between gNB-CU and source gNB-DU as specified from step 3 to step 8 in section 8.2.1.5.

5 - 6.	The gNB-CU-CP sends the RRC Reconfiguration message to the source gNB-DU via DL RRC MESSAGE TRANSFER message, and receives the the RRCReconfigurationComplete message from the source gNB-DU via UL RRC MESSAGE TRANSFER message.

7. If early data forwarding is applied, the Bearer Context Modification procedure may be performed to the source gNB-CU-UP with the TNL address information for data forwarding. The COUNT related information is retrieved from the source gNB-CU-UP and provided to the target gNB-CU-UP via the Bearer Context Modification procedure.

8.	The UE sends the lower layer measurement result to the source gNB-DU, and the source gNB-DU decides to execute LTM to a target cell.

9.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU-CP with the selected target cell ID.

10.	The gNB-CU-CP forwards the selected target cell ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message.

11.	The target gNB-DU detects the UE in the target cell.

12.	The target gNB-DU sends an ACCESS SUCCESS message to inform the gNB-CU-CP of which cell the UE has successfully accessed. The target gNB-DU also sends a Downlink Data Delivery Status frame to inform the target gNB-CU-UP.

13.	The gNB-CU-CP performs the Bearer Context Modification procedure to indicate the source gNB-CU-UP to stop packet delivery, which may also retrieve the PDCP UL/DL status and exchange the TNL address information for data forwarding for the bearers.

14.	The gNB-CU-CP performs the Bearer Context Modification procedure to send the DL TNL address information for F1-U and the PDCP UL/DL status to the target gNB-CU-UP.

NOTE 1:	The DL TNL address information for F1-U may be configured to the target gNB-CU-UP, as soon as the gNB-CU-CP knows which cell the UE will access.

15.	Data Forwarding may be performed from the source gNB-CU-UP to the target gNB-CU-UP.

15.	The target gNB-DU detects the UE in the target cell.

16.	The target gNB-DU sends an ACCESS SUCCESS message to the gNB-CU-CP.

16-18.	Path Switch procedure is performed to update the DL TNL address information for the NG-U towards the core network.

19-20.	Bearer Context Release procedure may be performed to release the UE context in the source gNB-CU-UP.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////