3GPP TSG-RAN WG3 Meeting #125bis	R3-245568

Hefei, China, October 14th – 18th, 2024

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	Continuing discussions on Inter-CU LTM signaling design

Document for:	Discussion and Decision

# Introduction

The discussions for Rel-19 Inter-CU LTM have consumed 3 meetings so far. The related RAN3 and RAN2 agreements are provided in Annex A and B, respectively.

In the last meeting, RAN3 achieved the basic stage-2 signalling flow [1] (with some FFSes of course), which is reproduced below.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s).

Editor’s Note: FFS on whether one candidate cell or a list of candidate cells is contained in the message.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB.

6.	The source gNB requests LTM configuration update to the candidate gNB(s).

Editor’s Note: Details are FFS on step 6.

7.	The candidate gNB(s) sends the response (LTM CONFIGURATION UPDATE ACKNOWLEDGE) to the source gNB.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional and FFS on the message name.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early TA acquisition to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) send the TA INFORMATION TRANSFER message to the source gNB.

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Editor’s Note: Details are FFS on above note.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Details are FFS on step 20.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

This was indeed a good progress, but still far from completion. In this contribution, we continue providing our views for Rel-19 Inter-CU LTM signalling design, and highlighting important aspects for further discussions.

# Discussion

## Inter-CU LTM preparation details

With respect to preparing Inter-CU LTM, the agreements reached so far in RAN2 and RAN3 suggest that the Rel-18 Intra-CU LTM designs will be reused as a baseline for Rel-19 and will not differ at this point.

Having this in mind, we provide our view for each of the aspects marked as FFS one by one:

3. The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). Editor’s Note: FFS on whether one candidate cell or a list of candidate cells is contained in the message. 4. Admission Control may be performed by the candidate gNB(s). 5. The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB. 6. The source gNB requests LTM configuration update to the candidate gNB(s). Editor’s Note: Details are FFS on step 6. 7. The candidate gNB(s) sends the response (LTM CONFIGURATION UPDATE ACKNOWLEDGE) to the source gNB. Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional and FFS on the message name.

HO REQ: per cell vs multiple

Regarding the step 3 FFS on the first round-trip of the HO procedure, we still think that it is appropriate to trigger HO per candidate cell. The reason is as follows:

- As already agreed in RAN2, the maximum number of LTM candidate cells is 8, regardless of intra-CU or inter-CU. This number 8 is the same to that of CHO (maxnoofCellsinCHO = 8), that of (S-)CPAC (maxnoofPSCellCandidates = 8), and that of the combination of both (i.e. CHO+CPAC). From Rel-16, the parallel CHO preparations per cell basis have been working well.

- With that, updating the existing HO procedure messages to cater for multiple candidate cells by a single procedure (and for Inter-CU LTM only) would require significant makeshift in stage-3.

- There was a concern on how a C-CU knows that the different Source NG-RAN node UE XnAP ID reference IE included in each HO REQ message for different candidate cells map to the same UE [2]. But we see no problem – even if they are different, it can be known based on the mandatorily included UE Context Information > NG-C UE associated signalling reference, which will be the same for the same UE.

On the other hand, we see the point from [2] is also valid in a sense that there will be complications in error handling if multiple UE-associations are established for the same UE over Xn. In previous releases, initiating multiple UE associations was allowed by the source by including different Source NG-RAN node UE XnAP ID references into HO REQ messages for different candidate cells [3]:

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. The procedure uses UE-associated signalling.

It was also assumed that the target should treat those different UE associations as if they are for separate UEs. The management of their subsequent signaling (actually belong to the same UE) was left up to the source's implementation to handle.

This was acceptable in the past because conditional handover was designed for one-time cell switch, and thus its configuration is released in the UE and NW after the successful execution. However, more complex interactions over XnAP are expected for Inter-CU LTM, not only during preparation phase, but also during execution phase that would not end one-time. For the ease of XnAP signalling management, we see the benefit of enforcing a single UE association for Inter-CU LTM.

Proposal 1: The first round-trip (the existing HO procedure) is triggered per each candidate cell.

Proposal 2: In order to ease the XnAP signalling management of Inter-CU LTM between S-CU and a C-CU, enforce only one UE association for parallel transactions. That is, parallel transactions are allowed using the same source UE AP ID only, each identified by the target cell ID, and one UE-associated signaling connection is established and maintained (as shown below):

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

The procedure name of the second round-trip

When multiple entities are involved, a two-round-trip interaction is inevitable for subsequent LTM because admission control and finalizing LTM configurations cannot be done simultaneously from C-CU. The first round-trip involves the initial context setup and retrieval of necessary configurations, allowing S-CU to collect configurations and information from all the admitted candidate cells to generate a common lower-layer configuration for these cells. The second round-trip is then used to exchange the collected information and finalize the LTM configurations to the UE from each C-CU.

In this context, we believe the current name suggested in our current 38.300 BL CR [1], “LTM Configuration Update,” serves its purpose well. The purpose of the second round-trip is not about e.g. modifying radio bearer configuration details that have already been admitted. Instead, it is more like a post-LTM-preparation step to finalize the LTM configuration to the UE with each C-CU, based on the common information collected from all the admitted candidate cells. As well known, the essence of LTM design relies on a common CSI resource configuration and cell-specific CSI report configurations. Any update or change to the common information collected at the S-CU (e.g. any modification to a set of candidate cells) may require updating the LTM configuration generated by each C-CU and configured to the UE, which may require additional RRC reconfiguration if it occurs after the initial configuration phase. From this sense, we think the current name “LTM Configuration Update” is appropriate.

Proposal 3: Confirm the name of the second round-trip procedure as “LTM Configuration Update”, as suggested in our current 38.300 BL CR [1].

The second round-trip (“LTM Configuration Update”) should be marked mandatory or optional?

We previously discussed a similar issue at Rel-18 Inter-DU LTM, where we decided to mark the second round-trip to a candidate DU (TS 38.401 Figure 8.2.1.5-1, Steps 7-8) as optional, even though it was considered necessary for subsequent LTM. This decision was based on the fact that LTM configurations for subsequent cell switch could be prepared and configured to the UE at a later stage (i.e. after the initial LTM configuration, with the expense of additional RRC reconfiguration).

We believe the same logic applies here in Rel-19 Inter-CU LTM. It is true that the steps 6-7 with each C-CU can be omitted if the S-CU chooses to address subsequent LTM at a later stage. Another consideration is that subsequent LTM without steps 6-7 is also possible, only in a special situation when there is only one C-CU involved and final LTM configurations can be completely prepared with the first round-trip.

Proposal 4: The second round-trip (“LTM Configuration Update”) is marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM.

Details of information exchange between S-CU and C-CU

Given the Rel-18 Intra-CU LTM designs are re-used as baseline, the information exchanged during the first round-trip (the existing HO procedure, triggered per candidate cell) and during the second round-trip (new UE associated class-1 “LTM Configuration Update”) can mirror what we have done in Rel-18 and also from S-CPAC (for which we can borrow some inter-node subsequent mobility operations of data forwarding):

Proposal 5: During the first round-trip (the existing HO procedure), the below information can be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - LTM initiation indication

  - LTM reference configuration (already agreed as single), or its request.

  - SSB / CSI resource configuration of candidate cells (if available)

  - Early Sync Information Request (already agreed), which can contain a list of DU ID(s) associated with S-CU (including S-DU) for LTM.

- (per-cell) HO REQ ACK message to include:

  - LTM reference configuration if requested

  - SSB / CSI resource configuration allocated from the admitted cell

  - Early Sync Information (already agreed), which contains DL/UL early sync info allocated from the admitted candidate cell

  - Data Forwarding proposal to be used for subsequent LTM

  - LTM candidate configuration (i.e. RRCReconfiguration) for the admitted cell (which is via HO CMD) + its complete configuration indicator (if LTM reference configuration is provided)

Proposal 6: During the second round-trip (new UE associated class-1 “LTM Configuration Update”), the below information can be exchanged:

- LTM Configuration Update REQ message to include:

  - LTM reference configuration (already agreed as single)

  - SSB / CSI resource configuration of all the candidate cells admitted by other CU(s)

  - Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell.

  - LTM Configuration ID Mapping List (each admitted cell is with a unique LTM candidate ID)

  - Data Forwarding TNLs assigned by other involved CU(s)

- LTM Configuration Update RESP message to include the following information:

  - L1 measurement reporting configuration from each admitted candidate cell

  - (Updated) LTM candidate configuration (i.e. RRCReconfiguration) for each admitted candidate cell + their complete configuration indicator (if LTM reference configuration is provided)

Other information could be further discussed, but we believe that the above information exchange over the first/second round-trip procedures are required and essential for the Inter-CU LTM configuration preparation across S-CU and C-CU(s).

## UE-associated Xn signalling connections between C-CUs

In the last meeting, some companies raised an important and good discussion point regarding the support of subsequent Inter-CU LTM after the initial LTM execution from S-CU to one of C-CUs [4][5][6]:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

R3-244042 [4] Handling of XnAP Identifiers:

When inter-CU LTM is firstly configured, a UE-associated connection will be established between the Source gNB and every Candidate gNB (e.g., when XnAP HANDOVER REQUEST and HANDOVER REQUEST ACKNOWLEDGE messages are exchanged). Hence, the XnAP Identifiers corresponding to a specific UE context will always point toward the Source gNB that firstly initiated the inter-CU LTM configuration. In other words, one pair of identifiers is linked by a parameter given by the Source gNB and another given by the Candidate gNB.

However, in case of Subsequent inter-CU LTM there is issue that after mobility from Source gNB to Target gNB, XnAP identifiers for the UE context become outdated, since the Source gNB has changed. Therefore, the UE-association should be updated with parameter that points toward the new Source gNB.

One alternative for carrying out this task is that after inter-CU LTM, the new Source gNB sends a UE-associated message toward all Candidate gNBs. This message will serve the purpose of updating the XnAP identifiers for the UE context.

Proposal 15: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM MODIFICATION REQUEST message to update the information related to a given UE context.

R3-244483 [5] UE associated Xn signalling connection setup between candidate gNBs:

During LTM, the LTM Cell Switch Notification from the source gNB-CU to the target gNB-CU can be successfully transmitted for the first time cell switch, because the source gNB-CU and the target gNB-CU already have the NG-RAN node UE XnAP ID of each other.

However, in subsequent inter-CU LTM, it is unclear how the new source gNB sends the LTM Cell Switch Notification to the target gNB and how the target gNB identify the correct UE, as there is no any Xn signalling connection setup procedure between these two gNBs beforehand.

One possible solution is that each candidate gNB allocates UE XnAP ID and send to other candidate gNBs in the preparation step.

Another solution is that after each cell switch, the new serving gNB allocates the source node UE XnAP ID and send it to other candidate gNB-CUs, and then the candidate gNB-CUs reply with the target node UE XnAP ID.

Proposal 13: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case.

R3-244055 [6] LTM cell switch completion:

After completion of the LTM, new UE associated signalling connection should be established between the new serving gNB and candidate gNB(s) in order to perform subsequent LTM. To establish the new signaling connections, the new serving gNB needs to know the information of other candidate gNB(s), e.g. the gNB ID and corresponding Xn AP ID allocated for the UE. With these information, the new serving gNB can initiate procedure to setup signaling connection with other candidate gNB(s).

The old serving gNB has such information of all candidate gNB(s).The old serving gNB can transfer such information to the new serving gNB in one of following procedures:

- Handover modification procedure after path switch.

- Cell switch notification

Proposal 11: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

During the Inter-CU LTM preparation phase, S-CU will establish the UE-associated XnAP connection with each C-CU involved. This setup allows S-CU to interact directly with any C-CU for the UE, regardless of which one is selected for initial PCell switch (or even at a later stage).

However, after the initial LTM execution, the new serving CU (one of the C-CUs to which the initial Inter-CU PCell switch was executed) maintains the UE-associated XnAP connection only with the S-CU. It currently lacks direct UE-associated signaling connections with other C-CUs for the same UE.

This means that any subsequent LTM procedure triggered from this new serving C-CU toward other C-CU(s) has to be routed through the S-CU. While routing via the S-CU can be made to work for subsequent LTM – as we have demonstrated in our paper [7], showing how early TA acquisition, cell switch notification, and HO SUCCESS across C-CUs can be handled without direct UE-associated XnAP connections between them, and how their stage-3 details can be captured in XnAP [8] – we observe that this approach complicates the specifications and may not be tractable:

Figure 1: An example signalling flow for Inter-CU LTM cell switch notification, TA information transfer, and HO SUCCESS across CUs (and routed via S-CU if necessary) [7]

Moreover, it is important to note that the complicated routing via the S-CU is outstanding only when more than two CUs are involved. In the scenario where only one C-CU and S-CU are involved (i.e., a total of two CUs), which is most likely the scenario for Inter-CU LTM in practice considering the large coverage area of a single CU, the Inter-CU PCell switch does not require complex handling nor routing via the S-CU. Only when the number of CUs involved exceeds two, the routing via S-CU may be required, which introduces additional complexities and inconsistencies in the specifications. From a specification standpoint, such lack of the harmonized or unified way of describing procedures could be problematic.

In fact, Inter-CU LTM can be viewed as a series of NW-triggered conditional handovers executed sequentially. In Rel-16, CHO was designed only for single execution from the source currently serving the UE toward one of the candidate targets. If executed serially, the selected target becomes the new radio access management point of the UE, who can determine, as a “source”, the next handover of the served UE toward another RAN node, and so on. From an XnAP point of view, this nature allows the descriptions devised for conditional handover based on the "source" and "target" to be naturally applied successively for subsequent Inter-CU LTM. For that, all we need is allowing direct UE-associated signaling connections between C-CUs. With this, any procedure triggered by a C-CU at any point of time can be made to work as effective as from the S-CU for initial PCell switch. Moreover, our specification efforts can be greatly reduced since their descriptions can remain universally applied regardless of how many CUs are involved for LTM.

Furthermore, we also observe additional benefits, such as reduced signalling overhead and simplified error handling. The handling for HO Cancel (triggered from the source to a candidate target) and CHO Cancel (triggered from a candidate target to the source) can also be streamlined, because a C-CU currently serving the UE, as the “source”, can directly trigger HO Cancel toward any other CU (including S-CU that has UE-associated signalling connection by default), and a C-CU who is currently not serving the UE and remains as a candidate can bypass the S-CU and directly trigger CHO Cancel toward the C-CU currently serving the UE, i.e. the “source”.

While many benefits are observed, there is one pre-requisite for this approach (of allowing direct UE-associated signalling connection between C-CUs) to work flawlessly. After each and every Inter-CU PCell switch, the new serving CU must be known to all the other CU(s) involved. Otherwise, it may not work properly, because a CU may lose track of which CU is currently serving the UE. For example, a C-CU who wants to trigger cancellation of its own candidate cells may lose track of which CU it should trigger CHO CANCEL message to (i.e. the node currently serving the UE – acting as the source).

But we also observe that this pre-requisite can be easily achieved by the security key update process that should be triggered from the new serving CU to all the other candidate CU(s). New security keys for candidate cells shall be derived from the new serving CU and shared to all the other candidate CU(s). Regardless of whether horizontal or vertical derivation is used, the master key shall be updated whenever the serving CU changes. This process (currently pending solution development from SA3) can be leveraged to let all the involved CU(s) aware of who became the new serving CU after an Inter-CU PCell switch.

Proposal 7: Discuss and consider allowing the UE associated XnAP signalling connections directly between C-CUs, for the unified/simplified specification handling and reduced signaling overhead.

## E1AP impacts

E1AP was promoted as one of the impacted specs in the previously updated WID [9], based on the following observations [10]:

E1-AP potential impacts

1.       Add the abbreviations of LTM/Conditional LTM.

2.       Clarify that some procedures can be reused for LTM (e.g. Early Forwarding SN Transfer).

3.       The enhancement on the BEARER CONTEXT SETUP procedure to indicate the request concerns a conditional LTM cell switch.

4.       The security handling for inter-CU LTM (Maybe, depends on RAN2 solution).

5.       Some enhancement on the UP handling to support early data forwarding for LTM (Maybe, depends on RAN3 solution).

While (4) and (5) are further to discuss, (1)-(3) are quite straightforward, given that

- LTM shares the similar design philosophy with conditional mobility, where serving cell switch does not happen immediately upon RRC configured. Moreover, the famous IE in the BEARER CONTEXT SETUP REQEUST message for one-bearer-context handling in CU-UP for multiple CHO requests applies the same.

- RAN3 already agreed early data forwarding for Inter-CU LTM:

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

- From CU-UP standpoint, no difference is observed between non-Conditional LTM and Conditional LTM for (1)-(3).

As a result, we just need to extend their usage in E1AP to cater for LTM as well (which was neglected in Rel-18), and their impact is small.

As an E1AP BLCR rapporteur, we have provided the corresponding E1AP BL CR in [11].

Proposal 8: Endorse the E1AP BL CR proposed in [11].

## Other consideration

Regarding TA handling, there were several additions worked out in Rel-18 that CU can share with a DU: (a) a known TA value (i.e. TA value = 0) between the current serving cell and a candidate cell; and (b) whether UE-based TA measurement can be applied (and configured to the UE) in-between the serving and candidate cells. The former was an optimization to cater a special deployment (co-located cells), while the latter was one of early UL synchronization mechanism specified that is utilized only when DU knows so.

While those TA handling is perfectly fine within a single CU (as specified in Rel-18), the question is whether they can be applicable between cells lying under different CUs. It is hard to imagine that either co-located cells are managed under different CUs, or the 260ns TAE (timing alignment error) requirement could be satisfied between cells under different CUs [12]. Before extending, the usage of those TA handlings across CUs shall be validated.

Proposal 9: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

Moreover, the WID was revised in RAN-104 (Shanghai) and the second priority works of Inter-CU LTM when DC is configured has been further clarified [9]:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

RAN2-126 (Fukuoka) also agreed as follows:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

In Rel-18, when DC is configured, independent “Intra-CU” LTM operations from either MN or SN were allowed for their respective cells. The RRC signalling specified in Rel-18 allowed NW to configure LTM over MCG and over SCG, as long as it does not incur simultaneous PCell and PSCell changes to the UE. Given that Rel-18 LTM was limited to “Intra-CU” mobility, the serving MN-CU or the serving SN-CU was able to independently decide LTM over their respective PCells or PSCells for a UE, without having to coordinate with each other.

In Rel-19, on the other hand, such independent operations become limited when Inter-CU LTM is involved in MN or SN. As clarified above in [9], Inter-CU LTM shall not be configured in both MN and SN at the same time for a UE. As a result, some MN-SN coordination is expected to prevent simultaneous LTM configurations in both MCG and SCG in Rel-19 Inter-CU LTM scenarios. For example, when Inter-MN LTM (or Inter-SN LTM) is being prepared and configured to the UE, the other peer, i.e. SN (or MN) needs to know such so that it does not initiate Inter-SN LTM (or Inter-MN LTM) for the same UE. Given that those MN-SN coordination efforts are related to the second priority, we think RAN3 should start their works after the first priority (non-DC standalone scenario, Xn-based) is sufficiently progressed.

Proposal 10: Any necessary coordination between MN and SN to prevent simultaneous Inter-CU LTM configuration in both MCG and SCG in Rel-19 should be studied and worked out, only after the first priority (non-DC standalone scenario, Xn-based) is sufficiently progressed.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Inter-CU LTM preparation details

Proposal 1: The first round-trip (the existing HO procedure) is triggered per each candidate cell.

Proposal 2: In order to ease the XnAP signalling management of Inter-CU LTM between S-CU and a C-CU, enforce only one UE association for parallel transactions. That is, parallel transactions are allowed using the same source UE AP ID only, each identified by the target cell ID, and one UE-associated signaling connection is established and maintained (as shown below):

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

Proposal 3: Confirm the name of the second round-trip procedure as “LTM Configuration Update”, as suggested in our current 38.300 BL CR [1].

Proposal 4: The second round-trip (“LTM Configuration Update”) is marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM.

Proposal 5: During the first round-trip (the existing HO procedure), the below information can be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - LTM initiation indication

  - LTM reference configuration (already agreed as single), or its request.

  - SSB / CSI resource configuration of candidate cells (if available)

  - Early Sync Information Request (already agreed), which can contain a list of DU ID(s) associated with S-CU (including S-DU) for LTM.

- (per-cell) HO REQ ACK message to include:

  - LTM reference configuration if requested

  - SSB / CSI resource configuration allocated from the admitted cell

  - Early Sync Information (already agreed), which contains DL/UL early sync info allocated from the admitted candidate cell

  - Data Forwarding proposal to be used for subsequent LTM

  - LTM candidate configuration (i.e. RRCReconfiguration) for the admitted cell (which is via HO CMD) + its complete configuration indicator (if LTM reference configuration is provided)

Proposal 6: During the second round-trip (new UE associated class-1 “LTM Configuration Update”), the below information can be exchanged:

- LTM Configuration Update REQ message to include:

  - LTM reference configuration (already agreed as single)

  - SSB / CSI resource configuration of all the candidate cells admitted by other CU(s)

  - Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell.

  - LTM Configuration ID Mapping List (each admitted cell is with a unique LTM candidate ID)

  - Data Forwarding TNLs assigned by other involved CU(s)

- LTM Configuration Update RESP message to include the following information:

  - L1 measurement reporting configuration from each admitted candidate cell

  - (Updated) LTM candidate configuration (i.e. RRCReconfiguration) for each admitted candidate cell + their complete configuration indicator (if LTM reference configuration is provided)

UE-associated Xn signalling connections between C-CUs

Proposal 7: Discuss and consider allowing the UE associated XnAP signalling connections directly between C-CUs, for the unified/simplified specification handling and reduced signaling overhead.

E1AP impacts

Proposal 8: Endorse the E1AP BL CR proposed in [11].

Other consideration

Proposal 9: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

Proposal 10: Any necessary coordination between MN and SN to prevent simultaneous Inter-CU LTM configuration in both MCG and SCG in Rel-19 should be studied and worked out, only after the first priority (non-DC standalone scenario, Xn-based) is sufficiently progressed.

The corresponding XnAP TP is also proposed in [13].

# Reference

[1] R3-244855, “(BL CR to 38.300) Support for Inter-CU LTM”, Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung

[2] R3-244355, “Further discussion on XnAP signaling for inter-CU LTM”, Ericsson

[3] TS 38.423, NG-RAN; XnAP (Release 18), v18.3.0

[4] R3-244042, “Discussion on Inter-gNB-CU LTM Procedure”, Nokia

[5] R3-244483, “Discussion on Inter-CU LTM procedure”, Huawei

[6] R3-244055, “Discussion on inter-CU LTM”, ZTE Corporation

[7] R3-244476, “Further discussion on Inter-CU LTM signaling design”, LG Electronics Inc.

[8] R3-244478, “(TP for NR_Mob_Ph4 TS 38.423)”, LG Electronics Inc.

[9] RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[10] RP-241516, “Discussion on rel-19 mobility enhancements WID revision”, Apple Inc, China Telecom

[11] R3-245569, “Baseline CR for introducing Rel-19 Mobility enhancement in E1AP”, LG Electronics Inc.

[12] R3-235019/R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM, RAN4

[13] R3-245570, “(TP for NR_Mob_Ph4 TS 38.423)”, LG Electronics Inc.

# Annex A (RAN3 agreement for Inter-CU LTM)

RAN3-125 (Maastricht)

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

RAN3-124 (Fukuoka)

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

RAN3-123bis (Changsha)

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

# Annex B (RAN2 agreement for Inter-CU LTM)

RAN2-127 (Maastricht)

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

RAN2-126 (Fukuoka)

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

RAN2-125bis (Changsha)

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

- Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.