3GPP TSG-RAN WG3 Meeting #125-bis	R3-245573

Hefei , China, 14 - 18 October, 2024

Title:	(TP to TR 38.769) Protocol Stack Option for Topology 2

Agenda Item:	16.2

Source:	Nokia

Document for:	other

# Introduction

In last meeting, the following were listed as solutions for topology 2:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

The detailed protocol stack was not captured due to lack of time and not enough discussion. This paper continues the discussion.

# Discussion

The currently discussed proposals are limited because they do not consider differences in uplink and downlink.

In fact, the constraint for downlink and uplink traffic can be quite different.

For example, for the downlink it is beneficial to send the inventory request using the RRC based protocol stack in order to have gNB refine the inventory area and list of involved UE readers. More generally, some information in any AIOT request need to be visible and interpreted by the gNB such as the request type, the area, etc... The following advantages can be achieved by the RRC based stack in the downlink:

- Cell visibility in the gNB,

- Harmonization of topology 1 and topology 2,

- Optimized management of radio resources.

But for the uplink, one example is that a simple and short inventory request from AIoT CN can generate tens of responses from AIoT devices which can be aggregated by each UE reader. In this case, it may instead be beneficial to have the AIoT responses aggregated by the UE readers and sent over NAS protocol or PDU session UP protocol depending on the amount of responses are expected by the AIoT CN.

Therefore, there is no “one-size-fits-all” protocol stack applicable for both downlink and uplink and flexible choice may be beneficial for the uplink response method.

For the uplink, this also means that an entity needs to control which protocol stack is used for the responses, and because this would need consistent handling by all the readers, this entity should be a central entity involved in the request which is the AIoT CN node. The AIoT CN node can better decide which method it wants to UE readers to use in their responses, based for example based on the expected number of responses and/or the maximum time allowed for answering.

As a result, we think that a valid alternative option for protocol stack could be to use an RRC based stack for the downlink while for the uplink make the AIOT CN decide which method the UE should use.

Proposal 1: introduce a fourth protocol stack option whereby the RRC based protocol stack is used for the downlink enhanced with a “preferred response method” for the uplink.

Proposal 2: The AIOT CN indicates in the request which response method the UE reader should use between RRC, NAS or PDU session.

This is proposed below to be captured as solution X in the below TP for TS 38.769.

# Text Proposal for TS 38.769

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

#### 6.4.2.1	Solutions for Topology 2

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Solution X: Enhanced RRC based solution in Downlink, flexible for uplink

The A-IoT CN transmits the AIoT request to the A-IoT-enabled gNB with an optional list of UEs and optional Target Area and a preferred response method between (RRC based, NAS based or PDU session based). The gNB transmits the related request towards the selected A-IoT-enabled UEs via NR Uu RRC.

Figure 6.4.2.1-x-1: Enhanced RRC based solution of Topology 2 for Downlink

The UEs aggregate the answers and transmit the response according to the preferred method indicated in the request either to the gNB (RRC based), to the AIoT CN control plane entity (NAS based) or to the AIoT CN user plane entity (PDU session based) as follows:

RRC based solution for uplink response requested by AIoT CN

If requested by AIoT CN, the A-IoT-enabled UEs aggregate the answers and transmit the response to the A-IoT-enabled gNBs over RRC:

Figure 6.4.2.1-x-2: RRC based uplink response for Topology 2

NAS based solution for uplink response requested by AIoT CN

If requested by AIoT CN, the A-IoT-enabled UEs aggregate the answers and transmit the response to the A-IoT CN over NAS:

Figure 6.4.2.1-x-3: NAS based uplink response for Topology 2

UP based solution for uplink responses requested by AIoT CN

If requested by AIoT CN, the A-IoT-enabled UEs aggregate the answers and transmit the response to the A-IoT CN user plane entity over the corresponding PDU session.

Figure 6.4.2.1-x-4: UP Session based uplink response for Topology 2

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>