3GPP TSG-RAN WG3 Meeting #125bis	R3-245581

Hefei, China, 14th – 18th October 2024

Agenda Item:	16.3

Source:	Ericsson

Title:	[TP for TR 38.769] Elements for AIoT RAN – AIoT CN communication and overall signalling

Document for:	Agreement

# 1	Introduction

This document provides a TP on further details on elements for AIoT RAN to AIoT CN signalling based on discussions and principles worked on in [3], covering aspects for inventory and command.

Further content for TR 38.769 on protocol elements for communication on the XX interface is provided.

# 2	References

[1]	RP-240826 "Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR".

[2]	TS 38.848 "Study on Ambient IoT (Internet of Things) in RAN".

[3]	R3-245579 "Further discussionis on AIoT RAN architecture ".

# 3	Text Proposal for TR 38.769 v.1.0.0

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

#### 6.5.1.x	XX Information common to AIoT transactions

(x.1)	A-IoT Device Identification

-	Dependent on the use case, it may concern only a single device or multiple devices.

Note 1:	The definition of this identification is out of RAN3 scope.

(x.2)	Scope of the AIoT Transaction

-	Allows NG-RAN to select the readers necessary to perform the AIoT transaction

-	Contains either geographical information, an AIoT Transaction Area ID (associated with a reader ID), or, in the context of topology 2 deployments an NR Cell ID.

Editor’s Note 1: Further definition of the Scope of AIoT Transaction and AIoT Transaction Area ID is FFS.

(x.3)	Reader ID

-	Allows AIoT CN and AIoT RAN to communicate latest know location of an AIoT device on reader ID granularity

Editor’s Note 2: Further definition of the Reader ID is FFS.

(x.4)	AIoT Transaction ID

-	Allows AIoT RAN and AIoT CN to address different, potentially concurring AIoT transactions

-	If the AIoT transaction is initiated by the AIoT CN, the AIoT Transaction ID is allocated by the AIoT CN and should be uniquely identifiable for a certain amount of time at least among the readers involved in AIoT transactions initated by the AIoT CN.

-	If the AIoT transaction is initiated by the AIoT RAN, the AIoT Transaction ID is allocated by the AIoT RAN and should be uniquely identifiable for a certain amount of time at least among the readers involved in AIoT transactions initated by readers served by the AIoT RAN.

(x.5)	AIoT upper layer PDU

-	Transparent Container, passed unaltered from the AIoT CN to the AIoT device.

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, includes the following:

(1)	AIoT Transaction ID (CN allocated)

(2)	A-IoT Device Identification

-	contains the ID of a single device ID, multiple IDs, a group of devices, or an indication that all devices are concerned

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(3)	Scope of AIoT transaction

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

The Inventory Response from the A-IoT RAN to the A-IoT CN, includes the following:

(1)	AIoT Transaction ID (AIoT CN allocated)

The Inventory Report from the A-IoT RAN to the A-IoT CN, includes the following:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification

(3)	Reader ID(s) involved in the Inventory transaction of the device indicated in (2)

#### 6.5.1.2	Command

##### 6.5.1.2.1	Inventory and Command

Inventory and Command may be performed on XX as follows:

-	Command related AIoT upper layer PDU contained in Inventory Request

-	Command related AIoT upper layer PDUs contained in subsequent AIoT CN initiated XX communication:

Command Request from AIoT CN to AIoT RAN:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification

(3)	AIoT upper layer PDU

Command Response from AIoT RAN to AIoT CN:

(1)	AIoT Transaction ID (AIoT CN allocated)

(2)	A-IoT Device Identification

(3)	AIoT upper layer PDU or Reception Confirmation

##### 6.5.1.2.2	Command-only

Command-only may be performed on XX as follows:

-	AIoT CN initiated: same as Inventory and Command

-	AIoT device initiated: Command related AIoT upper layer PDUs contained in XX communication, with the following communication elements on XX:

Command Required from AIoT RAN to AIoT CN:

(1)	AIoT Transaction ID (AIoT RAN allocated)

(2)	A-IoT Device Identification

(3)	Reader ID (s) involved in AIoT device-initiated Command-only transaction

(4)	AIoT upper layer PDU

Command Confirm from AIoT CN to AIoT RAN:

(1)	AIoT Transaction ID (AIoT RAN allocated)

(2)	A-IoT Device Identification

(4)	AIoT upper layer PDU or Reception Confirmation

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory)

#### 6.5.1.3	XX protocol elements for AIoT resource allocation in case of NAS/UP based solutions

NAS/UP based solutions require the possibility for requesting AIoT resources in advance to the NAS/UP based communication with the AIoT device.

XX AIoT resource allocation does not need to carry AIoT device specific information, but only sufficient information in order to configure the AIoT enabled UEs with respective AIoT resources.

The AIoT Resource Request and the AIoT Resource Response message shall carry the same AIoT Transaction ID (CN allocated) as communicated by the AIoT CN directly to the AIoT enabled UE(s).

Editor’s Note 1: Further details are FFS.

#### 6.5.1.4	Termination of AIoT transactions

Any kind of AIoT transaction needs to be terminated on AIoT RAN and AIoT CN side. This may be done either explicitly or implicitly:

-	Both, AIoT RAN and AIoT CN are assumed to terminate the transaction based on a configured timer.

-	Both, AIoT RAN and AIoT CN are allowed to request or command the termination of the transaction by means of explicit XXAP procedures.

-	In case an AIoT transaction consists of several XXAP procedures (e.g. multiple Inventory Report messages) the respective node provides in the last message an “end of transaction” indication.

Editor’s Note 1: Further details are FFS.

### 6.5.2	Signaling and Procedures for Topology 1

#### 6.5.2.0	General

Editor’s note 1: XX communication depicted in the following chapters uses protocol elements (messages and information elements) detailed in section 6.5.1 and are not repeated, unless additional description is necessary. The termination of AIoT transactions is not explicitly depicted.

#### 6.5.2.1	Candidate procedures for A-IoT Inventory for Topology 1

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

1a.	The AIoT RAN node attempts to allocate AIoT radio resources along the Scope of AIoT transaction indicated in step 1 to the readers.

2.	AIoT CN is notified about the successful initiation of the Inventory.

4./4b	Successful inventories are reported to the AIoT CN in Inventory report messages, which may contain information concerning multiple AIoT devices.

#### 6.5.2.2	Candidate procedures for A-IoT Command-only for Topology 1

Editor’s note 1: The (device initiated) command-only scenario depicted below attempts to estimate XX interface impacts only. Dependencies on ongoing discussions in RAN2 (AIoT radio details) and SA2 (support of the scenario and expected AIoT CN behaviour) need to be reviewed at a later stage.

Editor’s note 2: This example procedure for Command-only aims at initiating future work for topology 2 and respective approaches (RRC/NAS/UP).

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

0.	The AIoT device performs random access and the AIoT RAN node allocates AIoT radio resources.

1a/1b.	AIoT device-initiated command related signalling takes place, providing respective information to the AIoT CN via the AIoT RAN node.

2a/2b.	If necessary/required, the AIoT CN confirms the reception of the information and may provide additional information to the AIoT device.

Editor’s note 2: The conditions under which further signalling, subsequent to steps 1a/1b is possible, and the requirements to acknowledge information exchange by the peer-entity (AIoT device or AIoT CN) are FFS.

### 6.5.3	Signaling and Procedures for Topology 2

#### 6.5.3.1	Candidate procedures for A-IoT Inventory for Topology 2

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

#### 6.5.3.1.1	Candidate procedures for A-IoT Inventory for Topology 2 – RRC based solution

Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 (if RRC-based solution is used)

1a.	AIoT CN issues an XX Inventory request.

1b.	The AIoT enabled gNB performs AIoT radio admission control and allocates resources for the AIoT enabled UE(s) to be involved in the inventory transaction.

1c/2a. The AIoT enabled gNB configures the AIoT enabled UE(s) to be involved in the inventory transaction.

2b.	The AIoT CN is notified about the successful initiation of the inventory transaction

4.	results of the inventory transaction are provided by the AIoT enabled UE(s) via the AIoT enabled gNBs to the AIoT CN.

#### 6.5.3.1.2	Candidate procedures for A-IoT Inventory for Topology 2 – RRC based solution

Figure 6.5.3.1.2-2: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

Editor’s note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

0a.	Direct communication between the AIoT CN and and the AIoT enabled UE (as of the NAS/UP solution) requires the AIoT CN to first request AIoT resources for for the transaction.

0c/d. If AIoT radio resources are admitted by the AIoT enabled gNB, the AIoT enabled UE is configured with AIoT resources it is allowed to use. Respective RRC signalling needs to carry an AIoT transaction ID.

1a.	The AIoT RAN node attempts to allocate AIoT radio resources along the Scope of AIoT transaction indicated in step 1 to the readers.

2.	AIoT CN is notified about the successful initiation of the Inventory.

4./4b	Successful inventories are reported to the AIoT CN in Inventory report messages, which may contain information concerning multiple AIoT devices.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>