3GPP TSG-RAN WG3 Meeting #125bis	R3-245582

Hefei, China, 14th – 18th October 2024

Agenda Item:	16.4

Source:	Ericsson

Title:	[TP for TR 38.769] On locating AIoT devices

Document for:	Discussions & Approval

# 1	Introduction

This document provides a text proposal for TR 38.769 which contains reasoning for concluding that “reader ID” is not sufficient for actual use cases.

# 2	Text Proposal for TR 38.769 (currently based on agreed TP in R3-243972)

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 6.9	Locating ambient IoT devices

### 6.9.1	General

Editor’s note: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

A-IoT device location information may be used for the following purposes:

(a) improving the A-IoT operation itself, e.g. by A-IoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the A-IoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes, but is likely not sufficient for purpose (b).

For topology 1, A-IoT RAN node ID can be considered as a location of A-IoT device. For topology 2, UE ID of the A-IoT-enabled UE can be considered as a location of A-IoT device.

Editor’s Note 1:	How to know the “reader” location is FFS. Whether to use more than one “readers” within one A-IoT RAN for location purposes is FFS.

Editor’s Note 2:	Signalling details on how to provide the location information of the A-IoT device to the A-IoT CN needs further study.

Editor’s Note 3:	Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4:	Analysing the gap between the positioning requirements in the SID and the feasibility for A-IoT devices.

### 6.9.2	Locating AIoT devices with higher accuracy

It can be assumed that the Common reader function’s location is available to the AIoT enabled 5GS.

Further, it can be expected that for the scenarios in the ongoing study, the a reader’s location in “Reader ID granularity” is known with an accuracy in the range of the projected communication range of AIoT devices, i.e. within several tens of meters. Locating AIoT devices should also take into account requirements to cope with a related AIoT device density of ~1,5 devices per square meters (see TS 22.369 [10]), some of the indoor deployment scenarios may need to cope with readers deployed in a 3D setup.

It can be concluded that providing an AIoT device location with “Reader ID granularity” may only give a rough approximation of the AIoT device’s location, not suitable for avoiding (human) labour intensive and time-consuming operations and not suitable ofr performing subsequent automated operations, e.g. retrieval of goods for further transportation by means of roboters.

For purpose (b), i.e. providing location information to the consumer of the A-IoT service, further improvement of the locating accuracy may be achieved by involving multiple readers for e.g. measuring the power of the backscattered signal. With regards to necessary RAN functions, in topology 1, this approach can be realized by means of AIoT RAN node internal functions, in topology 2, this approach would require at least specification work on RRC.

Editor’s Note 1: System architecture aspects are FFS.

### 6.9.x	Proximity determination

Editor’s note: Proximity determination may be in a 6.9.x sub-clause, or another arrangement, depending on how the study proceeds.

Proximity determination is feasible with either of the two following solutions. Potential specification impact or not will not be determined in this study item.

Solution 1:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader.

Solution 2:	If the reader successfully receives D2R transmission from the device in response to R2D transmission, then the device is determined as near to the reader based on measurements at the reader side.

Proximity determination based on device-side measurements is not considered.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>