3GPP TSG-RAN WG3 Meeting #125bis	R3-245583

Hefei, China, 14 – 18 October 2024

Agenda Item:		19.3

Source:		Charter Communications

Title:			RAN3 issues related to SBFD

Document for:		Discussion

# Introduction

According to the WID RP-241614 [1] the following SBFD related issues are to be handled by RAN3:

- Specify enhancement for CLI handling [RAN1, RAN2, RAN3]

  - Information exchange among gNBs, including [RAN3]

    - Semi-static cell-specific SBFD time and frequency location configuration

    - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

    - Strongest DL beam information

    - CLI-mitigation request

RAN3 has recently received liaison R3-2450006 / R1-2407533 from RAN1, which indicates:

- From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic NZP CSI RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported.

- Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1..N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

- Strongest DL beam information in the context of SSBs is defined as an SSB index.  SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

- Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

Meanwhile, this meeting is the first one in which CLI mitigation in the context of SBFD is discussed in RAN3, while other groups (RAN1, RAN4, RAN2) already have ongoing work on the topic for some time. In this contribution, we discuss some of the issues that relate to RAN3 impacts.

# Discussion

  - CLI-mitigation request

While some of the topics are still in discussion, we believe that the topic of CLI mitigation is one that RAN3 can start working immediately, so that if any communication back to RAN1 is necessary / desirable, it can be done as soon as possible.

Proposal 1: RAN3 initiates discussion on gNB-gNB CLI mitigation, so that progress on this topic can be advanced, and any issues that need clarification from RAN1 can be communicated back to RAN1 at the earliest time.

We believe that CLI mitigation / testing procedures, must be flexible enough to allow the action to be started / stopped at any particular point in time, to provide whatever testing parameters are necessary, and to provide the result of testing to the requesting entity. The existing Xn messages RESOURCE STATUS REQUEST, RESOURCE STATUS UPDATE and the corresponding F1 RESOURCE STATUS REQUES, RESOURCE STATUS UPDATE, with appropriate modifications specific to the CLI mitigation / testing would support that functionality.

The existing RESOURCE STATUS REQUEST message already contains the infrastructure to:

- start / stop a particular test;

- which cell should report the results,

- which SSB should report the results;

- a bitmap to request different types of report / measurements, so one more category of tests such as CLI mitigation/testing could be easily added.

Proposal 2: The existing Xn RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE, appropriately modified as suggested in the annex (A.1 and A.2), should be used to request CLI mitigation / testing, exchange the appropriate parameters for the testing (such as particular CSI-RS, SSB, SRS, etc), and to report the results (such as strongest downlink beam information, any measured interference power, etc) of the completed CLI mitigation / testing, back to the requesting gNB.

Proposal 3: In case of a disaggregated gNB architecture, the existing F1 RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE, appropriately modified and similar (in the sense that they are for F1, instead of Xn) to what is suggested in the annex (A.1 and A.2), should be used to request CLI mitigation / testing, exchange the appropriate parameters for the testing (such as particular CSI-RS, SSB, SRS, etc), and to report the results (such as strongest downlink beam information, any measured interference power, etc) of the completed CLI mitigaton / testing, back to the requesting CU.

  - Strongest DL beam information

As communicated in the LS from RAN1, the strongest DL beam information should be exchanged as an index into the NZP-CSI-RS or SSB set of resources.  That is clear enough, and we have added an IE to support that capability in the proposed modifications to the Xn and F1 RESOURCE STATUS UPDATE message.

  - Measurement resource configuration

The RP-241614 WID refers to “Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS”.

In the LS, RAN1 indicated that the use of NZP-CSI-RS is agreed and supported.  But it seems that there are still ongoing discussions about SSB.  In our understanding the discussion seems to be on whether the requirement is a requirement for “SSB and NZP CSI-RS” or a requirement for “SSB or NZP CSI-RS”.  If our understanding is correct about this “and/or” problem, it is not clear to us why this would have an impact in RAN3, since whether it is a “and”, or a “or”, RAN3 would have to support both options.  If the issue, however, is one whether SSBs could possibly be dropped from consideration, then certainly RAN3 does not want to waste time on that discussion.

Proposal 4: RAN3 waits for LS from RAN1, indicating that the use of SSB as a measurement resource is confirmed.

## Semi-static cell-specific SBFD time and frequency location configuration

LS R1-2407533 sent by RAN1 to RAN3, has no information regarding the semi-static cell specific SBFD time and frequency location configuration. Our understanding is that there are still a number of issues that depend on progress in RAN4 and RAN2

Proposal 5:RAN3 should wait for a subsequent LS from RAN1 concerning semi-static cell-specific SBFD time and frequency location configuration.

# Proposals

Proposal 1: RAN3 initiates discussion on gNB-gNB CLI mitigation, so that progress on this topic can be advanced, and any issues that need clarification from RAN1 can be communicated back to RAN1 at the earliest time.

Proposal 2: The existing Xn RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE, appropriately modified as suggested in the annex, should be used to request CLI mitigation / testing, exchange the appropriate parameters for the testing (such as particular CSI-RS, SSB, SRS, etc), and to report the results (such as strongest downlink beam information, any measured interference power, etc) of the completed CLI mitigation / testing, back to the requesting gNB.

Proposal 3: In case of a disaggregated gNB architecture, the existing F1 RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE, appropriately modified as suggested in the annex, should be used to request CLI mitigation / testing, exchange the appropriate parameters for the testing (such as particular CSI-RS, SSB, SRS, etc), and to report the results (such as strongest downlink beam information, any measured interference power, etc) of the completed CLI mitigaton / testing, back to the requesting CU.

Proposal 4: RAN3 waits for LS from RAN1, indicating that the use of SSB as a measurement resource is confirmed.

Proposal 5:RAN3 should wait for a subsequent LS from RAN1 concerning semi-static cell-specific SBFD time and frequency location configuration.

# Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD).

[2] R1-2407533, LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation.

# Annex A

# A.1 Current existing Xn RESOURCE STATUS REQUEST

## Proposed modifications to Xn RESOURCE STATUS REQUEST to support CLI Testing

- Add a bit 7 to IE “Report Characteristics” to indicate the measurement object NG-RAN node 2 is request to report is “CLI Testing”

- Add a new IE “CLI Test Parameters”, which is conditionally present if the “Report Characteristic” IE indicates “CLI Testing”, and the existing “Registration Request” IE indicates “start” or “add”.

IE/Group Name; Presence; Range; IE type and reference; Semantics Description. CLI Test Parameters; O. >Type of CLI Test; M; ENUMERATED (gNB-to-gNB, …). >RS Configuration Type; M; ENUMERATED (NZP-CSI, ZP-CSI, CSI-IM, SSB, SRS…). >RS Configuration Transparent Container; M; OCTET STRING; Contains the definition of the RS, as coded in RRC. >Requesting Cell ID; M; TS38.423. 9.2.2.7.

## A.2 Current existing Xn RESOURCE STATUS UPDATE

## Proposed modifications to Xn RESOURCE STATUS UPDATE to support CLI Testing

Current RESOURCE STATUS UPDATE message needs following modification to support coordinated CLI measurement test results

- Add a new IE “CLI Test Result” to the existing “Cell Measurement Result Item” IE

IE/Group Name; Presence; Range; IE type and reference; Semantics Description. CLI Test Result; O. >gNB-to-gNB CLI test; O. >> Strongest DL Beam Information; M; INTEGER. >>CLI Power; M; INTEGER(0..20); Power in dBm. >>CLI Power delta; M; INTEGER(0..20); Power delta in dBm from previous environment.

A.3 Message Flows

gNB1 / CU

gNB2 / DU

RESOURCE_STATUS_REQUEST (start)

RESOURCE_STATUS_UPDATE (includes testing results)

RESOURCE_STATUS_RESPONSE (implicit acceptance)

RESOURCE_STATUS_REQUEST (stop)

A.4 No changes

Xn RESOURCE STATUS RESPONSE and RESOURCE STATUS FAILURE

- No changes to these messages.

- In the case of RESOURCE STATUS FAILURE, perhaps a new Cause value specific to CLI Testing may be added.

F1 RESOURCE STATUS REQUEST, RESOURCE STATUS UPDATE

## Similar changes to the Xn RESOURCE STATUS REQUEST and RESOURCE STATUS UPDATE