3GPP TSG-RAN WG3 #125bis	R3-245584

Hefei, China, 14 – 18 Oct. 2024

Agenda Item:	11.4. R18 leftovers

Source:	NTT DOCOMO INC.

Title:	Discussion on AIML for split architecture

Document for:	Discussion, agreement

Introduction

At the last RAN3 meeting [1], we captured TR 38.743 and had following description for split architecture use case.

5.2	Split architecture support for Rel-18 use cases

5.2.1	Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU-DU architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

-	AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

5.2.2	Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18:

-	The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase.

-	Measured Energy Cost from gNB-DU to gNB-CU.

In this contribution, we have WI phase discussion regarding energy saving use case in split architecture.

Discussion

In the TR support of measured energy cost from gNB-DU to gNB-CU was captured. In the split architecture, CU-CP and CU-UP can be considered as nodes from which energy cost can be reported. However, we see that DU energy consumption is more important so we think RAN3 should focus on DU to CU energy cost reporting and if we see benefit on reporting of CU-UP energy consumption, it can also be discussed later.

We think we can reuse rel-18 AI/ML energy saving supported over Xn can be a baseline, and we need to have a procedure in which the CU is requesting node and the DU is requested node. Whether we reuse existing procedure (e.g. Resource Status Reporting procedure) should be further discussed.

Proposal 1:	RAN3 should focus on support of measured energy cost reporting from DU to CU over F1. FFS on what F1 procedure should be reused or introduced.

For another point, whether per-DU energy cost should be sent to other gNB should also be discussed. Even if we only support per-DU energy cost transfer, it can be used for energy saving within the gNB. However, per-DU energy cost transferred over Xn would improve energy saving between gNBs because per-DU energy cost reporting can isolate the energy cost of the target DU from undesired effect by other DUs in the target gNB. For this reason, we also propose to exchange per-DU energy cost over Xn.

Proposal 2:	RAN3 to support signalings carrying measured energy cost per-DU both F1 and Xn.

Lastly, we think we should clarify how to define and configure the energy cost mapping rule for per-DU energy cost to the DU. In our standing, optimal mapping rule for per-DU may be different from one for per-gNB, so operator should have responsibility to define energy cost mapping rule suitable for all DUs across gNBs within a specific area.

Proposal 3:	RAN3 to discuss how to configure energy cost mapping rule for per-DU granularity.

Proposal 4:	Energy cost mapping rule for per-DU granularity is also provided from OAM to DUs and it is determined by operator same as one for per-gNB granularity.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN3 should focus on support of measured energy cost reporting from DU to CU over F1. FFS on what procedure should be reused or introduced.

Proposal 2:	RAN3 to support signalings carrying measured energy cost per-DU both F1 and Xn.

Proposal 3:	RAN3 to discuss how to configure energy cost mapping rule for per-DU granularity.

Proposal 4:	Energy cost mapping rule for per-DU granularity is also provided from OAM to DUs and it is determined by operator same as one for per-gNB granularity.

References

- RAN3#125 Chair Notes