3GPP TSG-RAN WG3 #125bis	R3-245585

Hefei, China, 14 – 18 Oct. 2024

Agenda Item:	13.2. Support for inter-CU LTM

Source:	NTT DOCOMO INC.

Title:	Discussion on inter-CU LTM non-DC case

Document for:	Discussion, agreement

Introduction

At the previous meeting, RAN3 reached the following agreements on inter-CU LTM[1].

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

And also, inter-gNB LTM without PDCP anchor change proposed by Qualcomm [2] was discussed in RAN3, but no agreement was reached.

Based on the above, this paper discusses the open points of Inter-CU LTM and without PDCP anchor change.

Discussion

Inter-CU LTM without PDCP anchor change

Qualcomm is proposing solution for LTM that avoids security key change and PDCP reestablishment in inter-CU LTM by switching the UE to the DU under the target gNB without changing the PDCP anchor in the C/U-plane. This method could maximize the benefit of low interruption of LTM with minimum signaling overhead in the following scenarios, for example.

- When a UE moves in an area near a single gNB and repeatedly performs intra-CU LTM, and sometimes ping-pong occurs between inter-gNB cells when crossing the boundary with neighbor gNB.

- When a single gNB covers a large area and a UE that moves out of the area to an neighbor gNB is expected to return to the original gNB within a certain period of time.

- When a UE is expected to move between the coverage boundaries of two gNBs

On the other hand, if intra-CU cell switch occurs frequently after moving to a neighboring gNB, Xn signaling may increase compared to normal inter-CU LTM. Therefore, both method of inter-CU LTM with and without PDCP anchor change should be supported in order to allow gNB to select better one based on situations.

Besides, SA3 is discussing security mechanism for inter-CU LTM based on the LS from RAN2. If SA3 would not confirms the solution from RAN2, conclude there is no solution, or only support security key change with L3 signaling at every cell switch, it means that subsequent inter-CU LTM is not supported. This heavily degrades the benefit of LTM, and PDCP anchor solution would be good way to solve it. However, we may not be able to receive an answer from SA3 soon. So we think we should not ruled out that solution.

On the other hand, the solution has some unclear points, and we think most important one is how we can support existing procedures in case the PDCP is anchored by other gNB. If it requires another procedure for all existing functions for PDCP anchor case, it may not be feasible.

Observation 1:	Inter-CU LTM without PDCP anchor change has significant advantages in terms of low interruption in certain scenarios.

Observation 2:	Security mechanism for inter-CU LTM is under discussion in SA3, and subsequent inter-CU LTM could not be supported if SA3 would not confirm the solution RAN2 raised or propose new solution.

Proposal 1:	As well as normal inter-CU LTM with security key change, inter-CU LTM without PDCP anchor change should also be discussed.

Proposal 2:	For LTM without PDCP anchor change, how to support existing mechanism (e.g. MRO, load balancing, energy saving) in case that PDCP is anchored by other gNB should be discussed.

LTM preparation

In order to support the Xn signaling for the preparation phase similar as Rel-18 intra-CU LTM F1AP, the following information needs to be sent from the source CU to the target CU in the preparation phase.

- Target candidate cell

In legacy L3 handover, the source CU determines the target cell and includes it in the Handover Request message. In inter-CU LTM, source CU should also determine the target/candidate cell and include it in the Handover Request message. The source CU should also decide which beam of each target/candidate cell to include as a candidate. If the candidate CU decides candidate beams, since the candidate beams are included in the common CSI resource configuration, this method increases the round trip of Xn signaling in the preparation phase.

Proposal 3:	Target candidate cell ID should be included in Handover Request message.

- LTM Configuration ID of target candidate cells

The LTM configuration ID assigned to the target candidate cell can be determined by the source CU or the candidate CU. However, if the candidate CU assigns the LTM configuration ID, same as the case of target candidate cell decision, it increases the round trip of Xn signaling. So that source CU should decide it.

Proposal 4:	Allocation of LTM configuration ID for the target candidate cell should be done by the source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

- LTM CSI Resource Configuration

The source CU can generate the LTM CSI Resource Configuration based on the cell/beam information associated with each gNB obtained in the Xn Setup procedure or the NG-RAN node configuration update procedure. The candidate CU generates the lower layer configuration based on this information.

Observation 3:	Source CU can generate LTM CSI Resource Configuration based on the information received in Xn Setup procedure or NG-RAN node Configuration Update procedure.

Proposal 5:	LTM CSI Resource Configuration could be generated by source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

- Candidate gNB-DU ID List

Similar to Rel-18 intra-CU LTM, when the candidate DUs allocate RACH resources, the CU should send a Candidate gNB-DU ID List in order to allocate the same RACH resource to the same source DU to save resources. Since the uniqueness of gNB-DU ID is guaranteed only within one gNB, candidate CU which receives a list of candidate gNB-DU IDs may not be able to distinguish candidate DUs from different gNBs. So that this list should also include Global NG-RAN Node ID which the DU is corresponding to.

Proposal 6:	Candidate gNB-DU List should be sent to the all candidate CU(s) via Handover Request message.

Proposal 7:	Candidate gNB-DU List should include gNB-DU ID associated with Global NG-RAN Node ID.

- Request for generation of reference configuration

In RAN2, there are two options to configure reference configuration: 1) only one reference configuration in one LTM Config, and 2) one reference configuration for each candidate CU. RAN3 impact depends on which option is supported, however, since RAN2 is currently under discussion, RAN3 should wait for the RAN2 conclusion.

Proposal 8:	How to generate and configure reference configuration should be discussed in RAN3 after RAN2 conclusion.

Subsequent inter-CU LTM

In the offline discussion in the last meeting, following three options were raised for inter-gNB coordination for subsequent LTM but not treated.

- Option 1: Send the collected LTM information to the candidate gNB(s) after the initial handover preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB send the LTM Cell Switch Command to the UE (e.g., via LTM Cell Switch Notification message).

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

This mechanism is needed because we agreed to support LTM cancellation in Xn, but if this happens in subsequent LTM (i.e. after initial cell switch), candidate CUs don’t know which one is serving gNB which Xn cancellation message should be sent to. For this reason, all candidate CUs should be informed which gNB is serving gNB. Also, if the gNB receiving LTM cancel message is not initial serving gNB, the gNB needs to acquire the RRC configuration configured to the UE.

Regarding the message carrying the notification, we think the notification of changes of serving gNB should be sent via Xn Cell Switch Notification or new class-2 message. This is because Handover Success is used in legacy L3 handover and the aim of this message is to notify the success of handover from target gNB to source gNB. Cell Switch Notification will be introduced in this release and this is for LTM so that we think Cell Switch Notification or new message is suitable in this case. However, the timing to send this notification is different from F1 Cell Switch Notification. F1’s one is sent at a time the DU sends Cell Switch Command to the UE, and this is for the target DU to start UL scheduling. On the other hand, in this case, the notification should be sent only after cell switch is completed, i.e. at a same time as Handover Success is sent.

Regarding the message carrying the RRC configuration, we think LTM modification message which would be used as 2nd round trip in preparation phase can be used. This configuration could also be sent via Cell Switch Notification or Handover Success after every cell switch, but this make these notification message bigger, so that we think it should be done prior to execution phase, i.e. preparation phase.

Proposal 9:	LTM configuration sent to the UE is shared among source CU and all candidate CU(s) in preparation phase (i.e. 1st or 2nd round trip btw source and candidate CU(s).)

Proposal 10:	Changes of source CU should be notified all candidate CU(s) at every cell switch completion.

Proposal 11:	Xn Cell Switch Notification or new class-2 message can be used to notify candidate CU(s) of change of serving CU.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	Inter-CU LTM without PDCP anchor change has significant advantages in terms of low interruption in certain scenarios.

Observation 2:	Security mechanism for inter-CU LTM is under discussion in SA3 and subsequent inter-CU LTM could not be supported if SA3 would not confirm the solution RAN2 raised or propose new solution.

Proposal 1:	As well as normal inter-CU LTM with security key change, inter-CU LTM without PDCP anchor change should also be discussed.

Proposal 2:	For LTM without PDCP anchor change, how to support existing mechanism (e.g. MRO, load balancing, energy saving) in case that PDCP is anchored by other gNB should be discussed.

Proposal 3:	Target candidate cell ID should be included in Handover Request message.

Observation 2:	Source CU can generate LTM CSI Resource Configuration based on the information received in Xn Setup procedure or NG-RAN node Configuration Update procedure.

Proposal 4:	Allocation of LTM configuration ID for the target candidate cell should be done by the source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

Observation 3:	Source CU can generate LTM CSI Resource Configuration based on the information received in Xn Setup procedure or NG-RAN node Configuration Update procedure.

Proposal 5:	LTM CSI Resource Configuration could be generated by source CU before sending Handover Request message to the target CU, and it should be included in Handover Request message.

Proposal 6:	Candidate gNB-DU List should be sent to the all candidate CU(s) via Handover Request message.

Proposal 7:	Candidate gNB-DU List should include gNB-DU ID associated with Global NG-RAN Node ID.

Proposal 8:	How to generate and configure reference configuration should be discussed in RAN3 after RAN2 conclusion.

Proposal 9:	LTM configuration sent to the UE is shared among source CU and all candidate CU(s) in preparation phase (i.e. 1st or 2nd round trip btw source and candidate CU(s).)

Proposal 10:	Changes of source CU should be notified all candidate CU(s) at every cell switch completion.

Proposal 11:	Xn Cell Switch Notification or new class-2 message can be used to notify candidate CU(s) of change of serving CU.

References

- RAN3#124 Chair Notes

- R3-244228, “Inter-gNB LTM with no change of RRC/PDCP anchor”