3GPP TSG-RAN WG3 Meeting #125-bis	R3-245591

Hefei , CN, 14 - 18 Oct, 2024

Title:	Consideration on SDT signalling optimization for partial context transfer [SDTnetworkSolution]

Agenda Item:	8.3

Source:	Huawei

Document for:	Discussion

# 1	Introduction

In previous meetings, RAN3 sent LS [1] to RAN2 to ask RAN2 to confirm whether this network-based solution has any RAN2 impact or issue. In this meeting, we received the reply LS from RAN2 in [2].

In this contribution, we further discussed the network-based solution based on RAN2 reply.

# 2	Discussion

The following information is provided by RAN2 [2]:

With regards to the network-based solution discussed in RAN3, RAN2 would like to inform RAN3 that: - If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost. - If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

Firstly, RAN2 mentioned two issues, one is the potential UL data loss, another one is the AS context and AS security context discarding and establishment, these two issues have already discussed in RAN3 before sending the LS, no need to re-discuss them in RAN3.

Then, RAN2 mentioned about the MAC reset “issue”, but based on the reply, there seems no consensus on whether it is an issue, as “Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.”.

Conclusion 1: There is no consensus on whether there is RAN2 impact in RAN2 discussion.

The following agreement was made in RAN3#123 meeting:

Agree with network based solution in the next meeting, unless the showstopper is found for network solution.

Based on the reply LS from RAN2, it is unclear whether there is a showstopper for the network-based solution. From our view, since it is the same gNB that will be moving the UE to the connected state so it can know the MAC state and can continue without reseting the MAC, it might be possible to avoid MAC reset by proper network implementation.

Based on these, RAN3 need to mak decision on whether to introduce the network-based solution, to provide another potential method to be used by the anchor gNB, especially when the UE does not support RRCRelease with resume indication.

Conclusion 2: RAN3 need to mak decision on whether to introduce the network-based solution.

As it was discussed in [3], the network-based solution can only be used in case of case 1 and 2, not in case 3 and 4:

- Case 1: DL non-SDT data or DL non-SDT signalling arrival

- Case 2: Large size of DL SDT data or DL SDT signalling arrival

- Case 3: UE assistance information (UL non-SDT data arrival indication) is received from the UE

- Case 4: Receiving the indication about large uplink SDT data from the BSR from the receiving gNB

Conclusion 3: Network-based solution can only be used in case of DL non-SDT data/signalling or DL large size data/signalling arrival.

In case the network-based solution needs to be introduced, it should only include one round of Retrieve UE Context procedure, i.e., the last serving gNB directly sends RETRIEVE UE CONTEXT RESPONSE message towards the Receiving gNB, to relocate UE Context and invoke the Receiving gNB to send RRCSetup towards the UE.

Figure 1. Cal flow of the network-based solution

Based on this call flow, the following RAN3 impacts are foreseen:

Impact #1: new indication in XnAP: RETRIEVE UE CONTEXT RESPONSE to trigger the receiving gNB to send RRCSetup towards the UE, and update stage 3 procedural text accordingly.

Impact #2: update stage 2 call flows in TS 38.300 and TS 38.401 to enable network-based solution.

Conclusion 4: In case RAN3 decide to enable network-based solution, it is needed to add new indication in XnAP: RETRIEVE UE CONTEXT RESPONSE to trigger the receiving gNB to send RRCSetup towards the UE, and update procedural text and stage2 call flows accordingly.

# 3	Conclusion

In this contribution, we further discussed the network-based solution based on RAN2 reply and get the following conclusions:

Conclusion 1: There is no consensus on whether there is RAN2 impact in RAN2 discussion.

Conclusion 2: RAN3 need to mak decision on whether to introduce the network-based solution.

Conclusion 3: Network-based solution can only be used in case of DL non-SDT data/signalling or DL large size data/signalling arrival.

Conclusion 4: In case RAN3 decide to enable network-based solution, it is needed to add new indication in XnAP: RETRIEVE UE CONTEXT RESPONSE to trigger the receiving gNB to send RRCSetup towards the UE, and update procedural text and stage2 call flows accordingly.

The corresponding CRs are provided in [4] [5] and [6], the draft Reply LS is provided in [7].

# 4	Reference

[1] R3-242198 Reply LS on SDT signalling optimization for partial context transfer, Source: RAN3, To: RAN2

[2]	R2-2407816 Reply LS on SDT signalling optimization for partial context transfer, Source: RAN2, To: RAN3

[3] R3-241668 Consideration on SDT RAN2 solution and Network based solution, Huawei

[4] R3-245592 Introduction of SDT network based solution, 38.300 draftCR, Huawei

[5] R3-245593 Introduction of SDT network based solution, 38.423 XnAP CR, Huawei

[6] R3-245594 Introduction of SDT network based solution, 38.401CR, Huawei

[7] R3-245595 [Draft] Reply LS on SDT signalling optimization for partial context transfer, Huawei