3GPP TSG-RAN WG3 Meeting #125bis	R3-245600

Hefei, China, 14 – 18 October, 2024

Agenda item:	8.1

Source:	Nokia

Title:	Discussion on the LS reply from SA5 on AI/ML for NG-RAN Energy Saving Energy Cost Index

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has received in this meeting a reply LS [1] from SA5. In the reply LS two correction CRs to TS 28.541 are attached, namely:

- S5-245163: Rel-18 CR to TS 28.541 Align with TS 38.423 to introduce NRM definitions to support NR energy cost Reporting

- S5-245164: Rel-19 CR to TS 28.541 Align with TS 38.423 to introduce NRM definitions to support NR energy cost Reporting

In the CRs, SA5 introduced NRM definitions for the NR Energy Cost mapping rule. There has been a sequence of LSs between RAN3 and SA5 on the topic of Energy Cost index and the current reply LS from SA5 [1] is related to an earlier LS sent by RAN3 to SA5 [3] in which RAN3 provided some requirements to SA5 and addressed earlier questions from SA5 on this topic. In this paper we point out some unclear aspects in the resolutions provided by SA5 and propose to send a reply LS to SA5 with our clarifications.

# 2	Discussion

In S5-245164, there is an attribute defined as nRECMappingRuleRef with a condition on Energy Cost reporting being supported. However, Energy Cost Reporting is related to reporting between neighbours over the Xn interface and, therefore, we think that it is out of SA5 scope.

Observation 1: Reporting of Energy Cost is described in TS 38.423 and is out of SA5 scope.

Related to a similar aspect, Figure 4.2.1.2-X illustrates an NRM fragment for NR Energy Saving based on Energy Cost Report. However, RAN3 hasn’t introduced an “Energy Cost Report” and therefore this terminology is confusing with respect to what it means.

In an earlier LS from RAN3 [2] RAN3 indicated to SA5 the following: “RAN3 assumes that these rules shall be the same at least for all neighbouring NG-RAN nodes within the area where a request on Energy Cost reporting is triggered by a NG-RAN node. Based on this, it is required that NG-RAN nodes are configured with unified rules to map the Energy Cost value to a measurement of consumed energy and ensure normalization of the exchanged Energy Cost information.”.

Also, in [3] RAN3 responded to the following question from SA5:

Q6: What are the requirements for the mapping rule? Should the mapping rule be same for all the gNBs in a given area?

Answer 6: Yes, the mapping rule should be the same across all gNBs in a given area where an AI/ML Energy Saving action is initiated as described in answer 3. Requirements of the mapping rule have been described in previous answers.

Observation 2: In the CRs attached by SA5, it is not clear that the mapping rule should be the same across all gNBs in a given area.

Observation 3: RAN3 has not introduced an “Energy Cost Report” and it is unclear what it means.

Also, S5-245164 describes the minimum and maximum Energy Cost values, ecMRInputMinimumValue and ecMRInputMaximumValue, as follows:

| ecMRInputMinimumValue | This attribute specifies, for the attribute considered in the mapping rule, the minimum value of to be applied for mapping from this attribute to the energy cost. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| --- | --- | --- |
| ecMRInputMaximumValue | This attribute specifies, for the attribute considered in the mapping rule, the maximum value of to be applied for mapping from this attribute to the energy cost. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

However, these definitions of the minimum and maximum values of an Energy Cost index are not clear and do not reflect RAN3’s response to SA5, quoted below from [3]:

Q2: Do ‘the Energy Consumption values corresponding to the minimum and maximum Energy Cost index values’ for a given gNB, correspond to its own minimum and maximum energy consumption values? If not, then what do these correspond to?

Answer 2: The energy consumption values corresponding to the minimum and maximum Energy Cost index values do not necessarily correspond to the given gNB’s own minimum and maximum energy consumption values. Instead, they depend on the minimum and maximum energy consumption values among all gNBs within a certain area.

It is important to capture in the SA5 specifications that the minimum and maximum energy consumption values do not correspond to a given gNB’s own Energy Consumption values but depend on the minimum and maximum energy consumption values among gNBs in a certain area (or within a group of gNBs).

Observation 4: There is no clear description in S5-245164  that the minimum and maximum Energy Cost values correspond to the minimum and maximum energy consumption values within a certain area of gNBs, which was what RAN3 indicated to SA5 in their Reply LS.

The same is true for the defined time interval which is defined as follows in S5-245164:

| ecTimeInterval | This attribute specifies the time interval (in seconds) that should be applied for collecting values of mapping rule attribute to then be used for computing the energy cost. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| --- | --- | --- |

In RAN3’s response to SA5’s question Q7 related to the time interval, RAN3 had responded the following:

Q7: Should the ‘time interval’ have the same value for all gNBs in a defined area or can the gNBs in the defined area have different values for the ‘time interval’?

Answer 7: The time interval selection is up to operator to define, but RAN3 would assume that the same time interval is configured by OAM for all gNBs within the defined area. The time interval is configured to limit, within specific time boundaries, the choice of an implementation-specific averaging window size for energy consumption measurements.

So it seems to us that the definition of Time Interval in S5-245164 is also incorrect. It is defined as an interval that should be applied for the collection of mapping rule attributes, instead of a time interval within which averaging of the energy consumption takes place.

Observation 5: Time Interval is in S5-245164 described as an interval that should be applied for the collection of mapping rule attributes, but should, according to RAN3’s LS, have been described as an interval within which the values of energy consumption are being averaged as per RAN3’s response.

Also, it seems that the Energy Cost mapping rule in Figure 4.2.1.1-X from S5-245164 is configured in gNB-CU-CP function as shown below:

Figure 4.2.1.1-X: NRM fragment to support energy saving based on Energy Cost Reporting

This may be valid under the assumption that this mapping rule is applicable at the gNB level in monolithic architecture, but is insufficient in case of split architecture support where a gNB-DU needs to measure an Energy Consumption and map it to an Energy Cost index. Thus, even though this works for the purpose of Rel-18 NG-RAN AI/ML work, SA5’s description is insufficient to enable a gNB-DU report an Energy Cost measurement to gNB-CU, according to RAN3 Rel-19 agreements for split architecture support.

Observation 6: Energy Cost mapping rule configuration in gNB-CU-CP is valid under the assumption that this mapping rule is applicable at the gNB level in monolithic architecture, but is insufficient in case of split architecture support where a gNB-DU needs to measure an Energy Consumption and map it to an Energy Cost index.

We, therefore, propose to send an LS to SA5 to ask SA5 to clarify the issues described above. A draft Reply LS can be found in [4].

Proposal: Agree the Reply LS in [4].

# 3	Conclusion

In this paper we make the following observations:

Observation 1: Reporting of Energy Cost is described in TS 38.423 and is out of SA5 scope.

Observation 2: In the CRs attached by SA5, it is not clear that the mapping rule should be the same across all gNBs in a given area.

Observation 3: RAN3 has not introduced an “Energy Cost Report” and it is unclear what it means.

Observation 4: There is no clear description in S5-245164  that the minimum and maximum Energy Cost values correspond to the minimum and maximum energy consumption values within a certain area of gNBs, which was what RAN3 indicated to SA5 in their Reply LS.

Observation 5: Time Interval is in S5-245164 described as an interval that should be applied for the collection of mapping rule attributes, but should, according to RAN3’s LS, have been described as an interval within which the values of energy consumption are being averaged as per RAN3’s response.

Observation 6: Energy Cost mapping rule configuration in gNB-CU-CP is valid under the assumption that this mapping rule is applicable at the gNB level in monolithic architecture, but is insufficient in case of split architecture support where a gNB-DU needs to measure an Energy Consumption and map it to an Energy Cost index.

We also propose the following:

Proposal: Agree the Reply LS in [4].

# References

[1]	R3-245026 - S5-245177	LS reply on AI/ML for NG-RAN Energy Saving Energy Cost index, SA5

[2]		R3-233515	LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[3]		R3-240998 –S5-243808 Reply LS on the NG-RAN Energy Saving Energy Cost index, RAN3

[4]	R3-245601	[Draft] Reply LS to LS reply on AI/ML for NG-RAN Energy Saving Energy Cost index, Nokia