3GPP TSG-RAN WG3 Meeting #125bis	R3-245603

Hefei, China, 14 – 18 October, 2024

Agenda item:	11.2

Source:	Nokia, Jio

Title:	Extension of Data Collection Procedures for AI/ML Slicing

Document for:	Discussion and Decision

# 1	Introduction

In TR 38.743 we have identified the standardization impacts for AI/ML Network Slicing use case. In particular, the TR identifies the following input data  for AI/ML-based Network Slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy predicted UE trajectory

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

In addition the following is agreed as output data of AI/ML based Network Slicing:

-	Predicted radio resource status per slice

-	Predicted slice available capacity

-	Resource management decisions for resources within RRM policies (used by gNB internally)-	Slice aware mobility decisions (used by gNB internally)

Finally, the following is agreed as feedback of AI/ML-based network slicing:

-	Measured Radio resource status per slice

-	Measured Slice available capacity

It was also agreed that the possible standard impacts involve enhancements of Xn interface to collect the predicted information between gNB pertaining to:

-    Predicted radio resource status per slice

-	Predicted slice available capacity

Thus, according to TR 38.743, Predicted Radio Resource Status per slice and Predicted Slice Available Capacity from neighbouring gNBs is agreed as one of the input data for AI/ML-based Network Slicing. In this contribution, we discuss our views on how this information may be requested and exchanged between neighbours.

# 2	Discussion

In Rel-18, we introduced 2 procedures, a Class 1 and a Class 2 procedure, over Xn interface for the reporting of information to support e.g., AI/ML in NG-RAN. The procedures enable a subscription-based mechanism in the sense that a node can subscribe to measurements from its neighbours and the latter will provide those measurements when available. The Class 1 procedure, Data Collection Reporting Initiation procedure, is used for the configuration of measurements at a neighbouring node. In Rel-18, those measurements comprise Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted Number of RRC Connections, measured Energy Cost, as well as UE-associated information related to a measured UE Trajectory and UE Performance measurements such as Average UE Throughput in DL and UL, Average Packet Delay in DL and UL and Average Packet Loss in DL. The Class 2 procedure,  Data Collection Reporting, is used for the actual reporting of the collected measurements by the configured node.

These procedures were designed taking as a baseline another subscription-based procedure over Xn, namely the Resource Status Reporting Initiation and Resource Status Reporting procedures. To this end, the Report Characteristics for Data Collection IE of Data Collection Reporting Initiation procedure is a bitmap where each of the bits defines a measurements (mainly predictions) to be started, similarly to the defined bits in the Report Characteristics IE for Resource Status Reporting Initiation procedure for 3 load measurements: Radio Resource Status, Number of Active UEs and number of RRC Connections.

When it comes to Radio Resource Status, a requesting node for Radio Resource Status information may request that the reported load contains information pertaining to specific slices. In that case it will include in the request a Slice To Report List IE to indicate only the subset of slices for which it is interested to obtain Radio Resource Status information. Such list of slices enables efficient reporting, since the number of the supported slices may be very high (with a maximum specified value equal to 1024).

We propose to enable Predicted Radio Resource Status per slice similarly to the legacy framework by introducing a Slice to Report List for Data Collection IE in the DATA COLLECTION REQUEST message.

Proposal 1: Introduce in the DATA COLLECTION REQUEST message a Slice to Report List for Data Collection IE to request Predicted Radio Resource Status per slice over the set of slices indicated in the list.

The legacy Slice Available Capacity IE, configured and reported through Radio Resource Status procedures over Xn, indicates the amount of resources per network slice that are available per cell relative to the total NG-RAN resources per cell. The Slice Available Capacity information (per slice) is reported together with the Composite Available Capacity (per cell). This is because different slices may take resources from the same resource pool, and the total available capacity over the reported slices may therefore exceed the available cell capacity. By including the Composite Available Capacity information, the receiver will have a better view of actual available capacity in the cell, e.g. for the case where it needs to make a decision involving multiple UEs being served by different slices.

Observation 1: In legacy load reporting, the Composite Available Capacity information is needed to correctly interpret the Slice Available Capacity information because some slices may use resources from the same resource pool.

When it comes to Predicted Slice Available Capacity we support to extend the legacy Data Collection Procedures following the scheme used for legacy Slice Available Capacity reporting described above. This will imply to additionally predict Composite Available Capacity, and report the Predicted Composite Available Capacity per cell together with Predicted Slice Available Capacity per slice.

Proposal 2: Introduce a prediction of Composite Available Capacity and report the Predicted Composite Available Capacity per cell together with Predicted Slice Available Capacity per slice.

For this, in terms of XnAP protocol impact, we propose to define a new bit in the Report Characteristics for Data Collection IE of Data Collection Reporting Initiation procedure that can be used to configure reporting of Predicted Slice Available Capacity. However, in legacy Resource Status Reporting Initiation procedure Slice Available Capacity is reported together with the Composite Available Capacity Group IE, when a requesting node sets the third bit “Composite Available Capacity Periodic” in the Report Characteristics IE. To keep alignment with the legacy framework, we propose to introduce a new bit “Predicted Composite Available Capacity” in Report Characteristics for Data Collection IE of Data Collection Reporting Initiation procedure, through which predicted Slice Available Capacity can also be reported.

Thus, we support to report Slice Available Capacity if the cell for which Composite Available Capacity Group IE is requested supports more than one slice, and if the Slice To Report List for Data Collection IE is included for a cell.

Proposal 3: Introduce a new bit “Predicted Composite Available Capacity” in Report Characteristics for Data Collection IE of Data Collection Reporting Initiation procedure, through which Predicted Slice Available Capacity can also be reported if the cell for which Predicted Composite Available Capacity Group IE is requested supports more than one slice, and if the Slice To Report List for Data Collection IE is included for a cell.

The Composite Available Capacity Group IE indicates the overall available resources per cell and per SSB area in the cell in Downlink, Uplink, and Supplementary Uplink. Even though indicating the available capacity in Supplementary Uplink has merits because it enables more accurate determination of the uplink resources, we believe it is very hard to predict. This is because for instance it is not possible to predict the number of UEs in the future that will be consuming the available capacity in the SUL.

Proposal 4: Introduce in the DATA COLLECTION UPDATE message a Predicted Composite Available Capacity Group IE, that indicates the overall predicted available capacity levels per cell and per SSB area in the cell in Downlink and Uplink.

Predicted Slice Available Capacity may comprise two components per S-NSSAI, Predicted Slice Available Capacity Value Downlink and Predicted Slice Available Capacity Value Uplink IEs. Those can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE.

Proposal 5: Introduce in the DATA COLLECTION UPDATE message a Predicted Slice Available Capacity IE for a cell that includes the requested Predicted Slice Available Capacity Value Downlink IE and Predicted Slice Available Capacity Value Uplink IE, providing the predicted slice available capacity with respect to the Cell Capacity Class Value.

We also provide an XnAP CR in [1], reflecting the above proposals.

# 3	Conclusion

In this paper we make the following observation and proposals:

Proposal 1: Introduce in the DATA COLLECTION REQUEST message a Slice to Report List for Data Collection IE to request Predicted Radio Resource Status per slice over the set of slices indicated in the list.

Observation 1: In legacy load reporting, the Composite Available Capacity information is needed to correctly interpret the Slice Available Capacity information because some slices may use resources from the same resource pool.

Proposal 2: Introduce a prediction of Composite Available Capacity and report the Predicted Composite Available Capacity per cell together with Predicted Slice Available Capacity per slice.

Proposal 3: Introduce a new bit “Predicted Composite Available Capacity” in Report Characteristics for Data Collection IE of Data Collection Reporting Initiation procedure, through which Predicted Slice Available Capacity can also be reported if the cell for which Predicted Composite Available Capacity Group IE is requested supports more than one slice, and if the Slice To Report List for Data Collection IE is included for a cell.

Proposal 4: Introduce in the DATA COLLECTION UPDATE message a Predicted Composite Available Capacity Group IE, that indicates the overall predicted available capacity levels per cell and per SSB area in the cell in Downlink and Uplink.

Proposal 5: Introduce in the DATA COLLECTION UPDATE message a Predicted Slice Available Capacity IE for a cell that includes the requested Predicted Slice Available Capacity Value Downlink IE and Predicted Slice Available Capacity Value Uplink IE, providing the predicted slice available capacity with respect to the Cell Capacity Class Value.

# References

[1]	R3-245604	AI/ML Slicing Support in Data Collection Reporting procedures, Nokia, Jio