3GPP TSG-RAN WG3 #125bis								R3-245614

14th – 18th Oct 2024, Hefei, China

Agenda Item:	9.2

Source:	China Telecom, ZTE

Title: 	Discussion on Transmission Bandwidth less than 5MHz

Document for:	Discussion

Introduction

In Rel-18, several market requests from vertical industry with the operating bandwidth less than 5 MHz were raised, such as Future Railway Mobile Communication System (FRMCS) in Europe, Smart Grids in USA and Public Safety in Europe. Based on this consideration, the new 3 MHz channel bandwidth, as well as the 12 and 20 PRB transmission bandwidths allow for gradual migration from the present GSM-R systems to 5G NR, while leaving room for a necessary number of GSM-R carriers on the same band (n100).

In RAN3#125 meeting, CRs on supporting of 12PRB or 20PRB transmission bandwidth were submitted [2][3]. During the online discussion, there was no consensus on the necessity of introducing 12PRB or 20PRB transmission bandwidth over Xn/F1 interface. This contribution tries to clarify the the necessity and provide our views on how to capture new transmission bandwidth in RAN3 specifications.

Discussion

Background

With the WID NR support for dedicated spectrum less than 5MHz for FR1 the work has been initiated on bandwidths <5 MHz (e.g., from around 3 MHz upwards) to enable during the migration phase the parallel operation of Future Railway Mobile Communication System (FRMCS) and GSM-R and massive infrastructure reuse. As shown in Figure 1, the (E)R-GSM bands are re-farmed for FRMCS to share 5.6MHz x 2 by NR-R and GSM-R to support NR and simultaneously legacy GSM-R (to enable transition). NR-R is deployed with collocated towers for GSM-R without guard bands, the UE performance is not affected by unwanted emission outside the 3.6MHz but within the 5MHz (as long as the relative transmit power between the two channels is not too high). When introducing 5G NR for FRMCS in band n100 of 2x5.6MHz FDD, the railway infrastructure managers need to maintain the network quality level of GSM-R while reducing the available bandwidth or number of GSM-R channels.

Figure 1 3.6MHz FDD Bandwidth for NR-R

The above case illustrated in Figure 1 is a special case for n100 which NR signal is strictly restricted to 20 PRBs in the 5 MHz channel bandwidth, and GSM-R signal occupies at least 5 PRBs in in the 5 MHz channel bandwidth. The SSB of 20RBs with SCS=15kHz can be reused without any puncturing and other physical channels are limited within the 3.6MHz.

Given the GSM-R network needs to support up to 14 GSM-R channels in high traffic density areas [4][5], another special use case of 12 PRB transmission bandwidth in a 3MHz channel bandwidth need to be considered. RAN4 specified an additional synchronization raster point at 920.73 MHz for 12 resource blocks transmission within 3 MHz channel.

Observation 1: 12PRBs and 20PRBs are very special use cases for n100. NR signal is strictly restricted to 12 PRBs and 20 PRBs in the 3MHz and 5 MHz channel bandwidth, respectively.

Per Table 5.4.3.1-3 in TS38.101-1, the transmission bandwidth for n100, can support 12 PRB, or 20 PRB transmission bandwidth configuration.

Table 5.4.3.1-3: Additional GSCN parameters for band n100

| SS Block frequency position SSREF (MHz) | GSCN | Note |
| --- | --- | --- |
| 920.73 | 41637 | Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |
| 921.45 | 41638 | Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |

Table 5.4.3.3-4: Applicable SS raster entries per operating band (FR1) for 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n26 | 15 kHz | Case A | 30937 – <1> – 31100 |
| n28 | 15 kHz | Case A | 30432 – <1> – 30644 |
| n31 | 15 kHz | Case A | 28955 – <1> – 28967 |
| n72 | 15 kHz | Case A | 28947 – <1> – 28959 |
| n85 | 15 kHz | Case A | 30282 – <1> – 30359 |
| n100 | 15 kHz | Case A | 31240 – <1> – 31242, 31244 – <1> – 31253, 416372 |
| n106 | 15 kHz | Case A | 31317 – <1> – 31329 |
| NOTE 1: SS Block pattern is defined in clause 4.1 in TS 38.213 [10]. NOTE 2: Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [9] clause 7.4.3.1. |  |  |  |

Per TS38.104 Table 5.3.2-1, the transmission bandwidth configuration NRB is still 15PRB and 25PRB for 3 and 5 MHz channel bandwidth with SCS=15KHz, respectively. In RANP#99, two agreements were reached on the number of PRBs for 3MHz and 5MHz channel bandwidth for band n100[4], which are:

RAN Plenary has discussed the possible transmission bandwidth options for 3 MHz and 5 MHz channel bandwidths for the spectrum allocations on the bands of interest in this work item, and concluded the following:

- For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4):

  - PBCH transmission bandwidth is 12 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands.

- For the 5MHz channel bandwidth:

  - PBCH transmission bandwidth is 20 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

And 12 PRBs and 20 PRB are due to the very special use case for n100, where the GSM-R system deployed on this band is migrating to 5G NR FRMCS system. During the transition the available PRB number could be reduced to 12 or 20 in some scenarios to ensure there are sufficient GSM-R channels, though maximum transmission bandwidth configuration is still 15 or 25 PRBs.

Observation 2: Maximum transmission bandwidth configuration or transmission bandwidth configuration NRB is still 15 PRBs or 25 PRBs for 3 and 5 MHz channel bandwidth.

In the summary of WI on NR support for dedicated spectrum less than 5MHz for FR1 s, the following were agreed and introduced in specifications [1]:

- a new 3 MHz channel bandwidth with 15 PRB

- support for 12 and 20 PRB transmission bandwidths for 3 and 5 MHz channel bandwidth, respectively, on band n100 .

Moreover, we should point out that 12 and 20 PRBs also require changes to some of the physical channels, that would otherwise occupy a larger number of PRBs:

- The bandwidth of PBCH is reduced to 12 PRBs for 3 MHz channel bandwidth by puncturing the lower and the upper 4 RBs of the 20 RB PBCH.

- The bandwidth of CORESET#0 is reduced to match the transmission bandwidths of 12, 15 and 20 PRB. Table 13-0 in TS 38.213 [1] includes the supported CORESET#0 sizes for dedicated spectrum less than 5MHz.

  - The 12 PRB CORESET#0 is directly mapped to 12 PRBs.

  - The 15 PRB CORESET#0 is obtained by puncturing the nine highest PRBs of the 24 PRB CORESET#0. Both interleaved and non-interleaved REG-to-CCE mapping is supported for the 15 PRB CORESET#0.

  - The 20 PRB CORESET#0 is obtained by puncturing the four highest PRBs of the 24 PRB CORESET#0.

Therefore, 12 and 20 PRBs are new feature for NR system require changes to some of the physical channels.

Observation 3: 12 and 20 PRB transmission bandwidths for 3 and 5 MHz channel bandwidth are new feature for NR system require changes to some of the physical channels.

RAN3 impact

The concept of transmission bandwidths is already used in Xn/F1 interface. We take the Transmission Bandwidth IE in TS38.473 as example which consists of NR SCS IE and NRB IE. The NRB IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104). Therefore, the CRs[2][3] submitted in last meeting are incorrect as 12 and 20 PRBs are not specified in "NRB". Since the NR signal is strictly restricted to 12 PRBs and 20 PRBs in the 3MHz and 5 MHz channel bandwidth, it is need to specify two numerical value over Xn/X2/F1 interface as some functions, i.e, load balance, need to take the cell total PRB number to evaluate some metrics and 15 PRB and 25 PRB are not the actual cell total PRB number.

| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage in percentage of the cell total PRB number. |
| --- | --- | --- | --- | --- |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage in percentage of the cell total PRB number. |

Based on the above analysis, there are two options to address this issue:

- Option 1: add 12 PRB and 20 PRB transmission bandwidth in existing NRB IE and update semantics descriptions for 12PRB and 20PRB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NRB | M |  | ENUMERATED (nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242, nrb15, nrb12, nrb20) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [17]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. The nrb12 or nrb20, this IE is used to indicate the 12PRB transmission bandwidth within 3MHz channel or 20PRB transmission bandwidth within 5MHz channel (TS 38.104, Table 5.4.3.1-3). |

- Option 2: Add a new IE, namely Actual Transmission Bandwidth, in existing Transmission Bandwidth IE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NRB | M |  | ENUMERATED (nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242, nrb15) | This IE is used to indicate the UL or DL maximum transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [17]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. |
| Actual transmission bandwidths | O |  | ENUMERATED ( nrb12, nrb20,…) | This IE is used to indicate the 12PRB transmission bandwidth within 3MHz channel, or 20PRB transmission bandwidth within 5MHz channel (TS 38.104, Table 5.4.3.1-3) |

We are also fine with the above two options and slightly prefer Option 1 as it does not impact ASN.1 part.

Proposal 1: to add a new IE, namely Actual Transmission Bandwidth, in existing Transmission Bandwidth IE.

The new codepoints need to be introduced in X2/Xn/F1 interface. Therefore, we kindly ask RAN3 to agree the corresponding CRs [6][7][8].

Proposal 2: to agree the corresponding CRs [6][7][8] for X2/Xn/F1 interface.

Proposal

Based on the above analysis, we have the following proposals:

Observation 1: 12PRBs and 20PRBs are very special use cases for n100. NR signal is strictly restricted to 12 PRBs and 20 PRBs in the 3MHz and 5 MHz channel bandwidth, respectively.

Observation 2: Maximum transmission bandwidth configuration or transmission bandwidth configuration NRB is still 15 PRBs or 25 PRBs for 3 and 5 MHz channel bandwidth.

Observation 3: 12 and 20 PRB transmission bandwidths for 3 and 5 MHz channel bandwidth are new feature for NR system require changes to some of the physical channels.

Proposal 1: to add a new IE, namely Actual Transmission Bandwidth, in existing Transmission Bandwidth IE.

Proposal 2: to agree the corresponding CRs [6][7][8] for X2/Xn/F1 interface.

Reference

- RP-234062，Summary for WI NR support for dedicated spectrum less than 5MHz for FR1, Nokia

- R3-244175, Correction on Transmission Bandwidth less than 5MHz, ZTE Corporation, China Telecom, CATT, China Unicom

- R3-244176, Correction on Transmission Bandwidth less than 5MHz, ZTE Corporation, China Telecom, CATT, China Unicom

- RP-230780, Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz

- RP-230189, UIC response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz, UIC

- R3-245047,Correction on Transmission Bandwidth less than 5MHz in TS38.423,ZTE Corporation, China Telecom, CATT, China Unicom, Qualcom, Huawei, Nokia

- R3-245048, Correction on Transmission Bandwidth less than 5MHz in TS38.473,ZTE Corporation, China Telecom, CATT, China Unicom, Qualcom, Huawei, Nokia

- R3-245049, Correction on Transmission Bandwidth less than 5MHz in TS36.423,ZTE Corporation, China Telecom, CATT, China Unicom