3GPP TSG RAN3 meeting #125bis  	         	                          R3-245618

14th – 18th Oct 2024  Hefei, China

Agenda Item:	10.3.1

Source:	China Unicom

Title:	Discussion on SON/MDT enhancements for NTN mobility

Document for:	Discussion

Introduction

The Work Item on SON/MDT enhancement has been agreed in RAN#102 (RP-234038) with the following objectives:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

This paper discusses the SON/MDT enhancements for NTN mobility.

Discussion

Condition Handover Optimization for NTN network

Time-based CHO Optimization for NTN network

NTN supports the time-based trigger condition upon which UE may execute CHO to a candidate cell. The CHO condition is configured from the gNB to UE via the RRCReconfiguration message, and the preparation and execution phase of the conditional handover procedure is performed by the UE without involvement of the 5GC.

In RAN3#125 meeting, RAN3 had the following agreements:

MRO for CHO with a time-based trigger condition: Case 1 Too Late CHO Execution: RLF happens in the source cell before UE executes CHO (before or after T1 threshold starts), and the UE re-connects the cell other than the source cell. Distinguish if the RLF happens before or after T1 Case 2 Too Early CHO Execution/CHO Execution to Wrong Cell: HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell.

It has the agreements for the cases about too Late CHO execution, too early CHO execution and CHO execution to wrong cell.

For the case about too late CHO execution, the RLF may happens in the source cell and re-connects to the cell other than the source cell, but the RLF may happen before of after T1 threshold starts, and the time-based trigger condition and/or the measurement-based trigger conditions may need to be optimized;

In the current specification, the timeSinceCHO-Reconfig can be used for the optimization about the time-based trigger condition, and the measResultLastServCell and measResultNeighCells can be used for the optimization about the measurement-based trigger conditions.

For the case about too early CHO execution and CHO execution to wrong cell, both the time-based trigger condition and the measurement-based trigger conditions are need to be optimized, and the the UE should record the handover time and the measurement result of the reference signal of the serving cell, handover target cell and other neighbor cells, and these information can be send to the gNB when the RLF happen, and the gNB can optimize the CHO condition and the measurement-based trigger conditions according to these information.

Proposal 1: For the case about too early CHO execution and CHO execution to wrong cell, the UE can record the handover time when the time-based CHO happen, and report this information to the gNB when the radio link failure happen.

The time-based CHO will configure the t1-Threshold and the duration for the handover, and the UE will execute the handover during the configured time duration. Even if the handover is successful, and there is no radio link failure happened after the CHO, the CHO may also need to be optimized. The CHO may happen at the beginning of the t1-Threshold, or nearly at the end of the t1-Threshold + duration, therefore the gNB also needs to get the CHO related information to further optimize the CHO configuration to avoid the potential handover failure. The condition for the UE to report the CHO related information can be configured by the gNB.

Proposal 2: The gNB can configure the CHO successful report condition for the UE to report the time-based CHO related information.

Proposal 3: The UE can record and report the handover time and time-based CHO related information to the gNB when the condition for the UE to report the time-based CHO related information if satisfied.

Location-based CHO Optimization for NTN network

In RAN3#124 meeting, RAN3 had the following agreements:

MRO for intra-NTN mobility shall consider the following trigger conditions: - Measurement-based trigger condition; - A time-based trigger condition; - A location-based trigger condition; - A time-based and Measurement-based combined trigger condition; - A location-based and Measurement-based combined trigger condition;

For the location-based CHO, the UE should measure the distance to the reference location of the serving cell and the target cell, and it will execute the handover after the distance threshold for the serving cell and the target cell are satisfied. There are also two cases for the handover too early or too late, or even handover to the wrong cells. RLF happens in the source cell before UE executes CHO, and the UE re-connects the cell other than the source cell, then too late CHO happens. When  HOF happens or RLF happens shortly after successful CHO, and the UE re-connects in the source cell or a third cell, then too early CHO execution or CHO execution to wrong cell happen. In these cases, the location-based CHO configuration may need to be optimized. The handover too early or too late, or handover to the wrong cells may be detected by the radio link failure procedure, the UE can record the information about the CHO when the RLF happen, and the UE can report the CHO information to the gNB for CHO configuration optimization.

The location-based CHO condition is configured by the gNB according to the network planing of the NTN network, and the  measurement result of the reference signal of the serving cell, handover target cell and other neighbor cells can be used for the network to optimize the distance thresh from reference for serving cell and neighbor cells. When the location-based CHO happen, these information also need to be recorded, and  these information can be send to the gNB when the RLF happen.

Proposal 4: The UE can record the distance from the reference point when the location-based CHO happen, and report  these information to the gNB when the radio link failure happen.

The location-based CHO will configure location-based CHO condition for the handover, and the UE will execute the handover during the configured condition. Even if the handover is successful, and there is no radio link failure happened after the CHO, the CHO may also need to be optimized. The CHO may happen near the location of distanceThreshFromReference1 or distanceThreshFromReference2, therefore the gNB also needs to get the CHO related information to further optimize the CHO configuration to avoid the potential handover failure. The condition for the UE to report the CHO related information can be configured by the gNB.

Proposal 5: The gNB can configure the CHO successful report condition for the UE to report the location-based CHO related information.

Proposal 6: The UE can record and report the location-based CHO related information to the gNB when the condition for the UE to report the location-based CHO related information if satisfied.

MDT Measurement Enhancement for NTN Network

The UE in NTN network can support mobility in RRC IDLE/INACTIVE states, and can support CHO in RRC CONNECTED state.

For UE in RRC IDLE/INACTIVE states, the network will configure the reference location and the moving reference location to UE via SIB19. UE will execute the cell re-selection according to the reference location and the moving reference location configured by the network.

If distance Thresh and reference Location are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN quasi-Earth-fixed system and has obtained its location information, the UE will check the distance between UE and the serving cell reference location, if the reference Location is shorter than distance Thresh, the UE may not perform intra-frequency measurements; otherwise, the UE shall perform intra-frequency measurements.

If distance Thresh and moving Reference Location are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving system and has obtained its location information, the UE will check the distance between UE's location and the serving cell reference location determined based on moving Reference Location is shorter than distance Thresh, the UE may not perform intra-frequency measurements; otherwise, the UE shall perform intra-frequency measurements.

The UE will execute the cell re-selection after the measurement, and the reference location configuration is critical for the UE cell re-selection.

For UE in RRC CONNECTED state, the network can configure the location based CHO for UE, and the UE will execute the conditional handover when the condition configured by the network is satisfied.

In the location based CHO configuration, the referenceLocation1 is associated to serving cell and referenceLocation2 is associated to candidate target cell. When the distance from a fixed reference location configured with referenceLocation1 or referenceLocation2 or a moving reference location determined by the UE based on referenceLocation1 or referenceLocation2 satisfy the configured threshold, the UE will execute the conditional handover.

Observation 1: The reference point is used to determine the cell re-selection measurement and conditional handover during UE mobility in RRC IDLE/INACTIVE states and RRC CONNECTED state.

The reference location configuration is critical for the UE to execute cell re-selection and handover, and how to configure the reference location and the distance threshold for UE and reference location relies on the network deployment for NTN network. When the reference location and the distance threshold are not configured properly, there will be coverage hole or too much overlap coverage, which will lead to UE handover fail or unnecessary measurements for UE. The reference location and distance threshold configuration need to be optimized according to the measurements based on the reference location.

Proposal 7: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

For the cell re-selection and CHO case, the distance between the reference location is the critical judgment parameters according the condition configured by the gNB. How to optimize the configuration for cell re-selection and CHO relies on the measurements of the reference signal when the cell re-selection and CHO happen. The MDT measurement can be used for the measurements of the reference signal for certain distance to the reference point, and the reference point can be the reference point configured or will be configured for the cell re-selection and CHO. There are two scenarios for the use of the MDT configuration configured according to the reference point:

Case 1: Because the large coverage of the NTN cell, only the edge of the cell need to be measured for the NTN network to optimize the CHO and cell re-selection configuration.

Case 2: The NTN cell may cover two countries, or operated by different operators, and the MDT measurement should be executed in part of the areas.

Proposal 8: The network can configure the MDT measurement for the reference point, and the area scope can be configured by the network with the MDT configuration.

Conclusion

Proposal 1: The UE can record the handover time when the time-based CHO happen, and report this information to the gNB when the radio link failure happen.

Proposal 2: The gNB can configure the CHO successful report condition for the UE to report the time-based CHO related information.

Proposal 3: The UE can record and report the time-based CHO related information to the gNB when the condition for the UE to report the time-based CHO related information if satisfied.

Proposal 4: The UE can record the distance from the reference point when the location-based CHO happen, and report  these information to the gNB when the radio link failure happen.

Proposal 5: The gNB can configure the CHO successful report condition for the UE to report the location-based CHO related information.

Proposal 6: The UE can record and report the location-based CHO related information to the gNB when the condition for the UE to report the location-based CHO related information if satisfied.

Observation 1: The reference point is used to determine the cell re-selection measurement and conditional handover during UE mobility in RRC IDLE/INACTIVE states and RRC CONNECTED state.

Proposal 7: The reference location and distance threshold configuration need to be optimized for UE cell re-selection and CHO.

Proposal 8: The network can configure the MDT measurement for the reference point, and the area scope can be configured by the network with the MDT configuration.