3GPP TSG RAN3 meeting #125bis  	         	                         R3-245619

14th – 18th Oct 2024  Hefei, China

Agenda Item:	10.3.2

Source:	China Unicom

Title:	Discussion on SONMDT enhancements for network slicing

Document for:	Discussion

Introduction

The Work Item on SON/MDT enhancement has been agreed in RAN#102 (RP-234038) with the following objectives:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

This paper discusses the SON/MDT enhancements for network slicing.

Discussion

MDT Enhancement for Network Slicing

In RAN3#125 meeting, an FFS is listed below:

The slice ID will be added as a filter to the area scope (for the both, management- and signalling-based immediate MDT related to slice). The intention is to limit the number of reports sent to the OAM, thus the UE selection will not be amended.

Regarding adding slice ID in the area scope IE, we consider that the motivation is for OAM to choose to only monitor the UEs for one or some slices, and then those UEs using a certain slice shall report their MDT measurements results for further optimization for slicing reason. If the UE does not use the slices in the configuration slice list, then MDT measurement should not be initiated. Once the UE uses the corresponding slice in the configuration, UE should report its MDT measurement to the network along with which of the slice are actually been used.

Proposal 1: OAM can configure MDT measurement per slice.

Proposal 2: The slice(s) ID which the network wants to measure should be configure in the MDT measurement  in the area scope IE.

Proposal 3: UE should initiate MDT measurement only if configured slicing has been used.

Proposal 4: The slice(s) ID which UE has been used should report to the network.

MRO enhancement for Network Slicing

Slicing is introduced in 5G network, and the slice information can be configured together with the PDU session setup procedure. The network can provide differentiated services for different slices, but the optimization for slicing coverage is not together with the optimization of the radio signaling coverage. There may be slice coverage hole even if the radio coverage is well, when the UE already configured with a slice service may HO to a target cell, which cannot support this slice service and thus causing service discontinuity. The slice coverage may need to be enhanced.

In the UE Information report, the history of UE visited cells will be reported to the network, and the slice information associated with the PDU session of the UE can be reported to the network. When the handover happen, the History Information from the UE will be transferred from the source cell to the target cell in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store the collected information and use it for future handover preparations. The target cell may analyse which future candidate cells support the services of certain slices, and which future candidate cells will have high success rate of handover, then it will make the final handover decision considering about this.

Proposal 5: The slices information associated with the PDU sessions of the UE can be reported to the network for slice optimization.

Conclusion

Proposal 1: OAM can configure MDT measurement per slice.

Proposal 2: The slice(s) ID which the network wants to measure should be configure in the MDT measurement  in the area scope IE.

Proposal 3: UE should initiate MDT measurement only if configured slicing has been used.

Proposal 4: The slice(s) ID which UE has been used should report to the network.

Proposal 5: The slices information associated with the PDU sessions of the UE can be reported to the network for slice optimization.