3GPP TSG-RAN WG3 Meeting #125bis	R3-245626

Hefei, China, 14 - 18 Oct, 2024

Title:	Discussion on L3 measurement based LTM support over F1

Agenda Item:	8.1

Source:	Huawei

Document for:	discussion

1	Introduction

RAN2 sent an LS [1] to RAN3 to support L3 measurement based LTM, as shown below:

1. Overall Description: In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

In this contribution, we discuss how to design the F1 procedure and signalling to support L3 measurement based LTM.

2	Discussion

In Rel-18 LTM, a fundamental design target is to enable the gNB-DU trigger the LTM cell switch with the received L1 measurement reports.

According to the incoming LS [1], RAN2 asks RAN3 to design F1 signalling to support the L3 measurement based LTM. In the LS, RAN2 proposes the gNB-CU to request the gNB-DU to send the MAC CE cell switch command to the UE.

However, the RAN2 proposed method may have much more impacts on Rel-18 LTM basic designs. Considering that Rel-18 LTM has completed for a long time, we propose RAN3 to make a complete analysis and comparison among all the feasible methods and choose one with minimal legacy LTM function impact.

In general, we foresee the following three methods for the L3 measurement based LTM.

- Method 0: The gNB-CU does not initiate LTM preparation for the UE not capable of L1 measurement reporting. The gNB-CU always triggers L3 handover. This can be supported by implementation.

- Method 1: The gNB-CU forwards the received L3 measurement results to the gNB-DU via F1 signalling. The gNB-DU then can initiate LTM to the UE by the L3 measurement results with legacy procedures. This method requires the gNB-DU to understand the L3 measurement results. This was submitted to last RAN3 meeting in [2].

- Method 2a: The gNB-CU decides to trigger LTM cell switch directly, i.e., selecting a target cell and/or a target beam. The CU needs to send the target cell/beam indication to the gNB-DU, and the gNB-DU generates the LTM cell switch MAC CE and sends it to the UE. This method seems the one that RAN2 proposes, but it changes the LTM decision from the gNB-DU to the gNB-CU. This method may also have impacts on many legacy LTM functions, including Early DL/UL synchronization, MAC CE triggered CFRA cell switch and Cell switch notification, etc.

- Method 2b: similar to Method 2a, but before the LTM cell switch is triggered, the gNB-DU requests the gNB-CU to provide a target cell for LTM cell switch. This method enables the gNB-DU to request the gNB-CU to trigger LTM immediately in case that the gNB-DU detects that the UE performance is declined lower than the Qos profile requirement. This method is in line with the method that RAN2 proposes while keeping the LTM decision at the gNB-DU.

A brief summary table for the above methods is given below:

|  | Method 0 | Method 1 | Method 2a | Method 2b |
| --- | --- | --- | --- | --- |
| LTM for UEs not capable of L1 measurement report? | NO. Always using L3 handover | YES | YES | YES |
| In line with RAN2 proposal? | NO | NO | YES | YES |
| Which node makes the LTM decision? | NA | gNB-DU | gNB-CU, different from L1 measurement triggered LTM. | gNB-DU |
| F1 signalling impact | NO | one new class 2 UE associated signalling | Depends on the study on the impact on the legacy functions in below cell. | one new class 1 UE associated signalling |
| Legacy LTM functions impacted | NO | NO. The gNB-DU just needs to interpret L3 measurement results | The following functions needs to be revisited: - Early DL/UL synchronization - MAC CE triggered CFRA cell switch - Cell switch notification | NO. |

From above comparison, we may conclude that with method 2a, the LTM cell switch decision is made by the gNB-CU, which aligns with the proposal of the LS [1], but with lots of impacts on the legacy LTM functions, for example, how and when does the DU trigger the PDCCH order to the UE for early TA acquisition? Which node, the gNB-CU or the gNB-DU set the IEs in the PDCCH order? Whether and how to support CFRA resource in LTM cell switch MAC CE? In addition, whether the source DU still needs to send the DU-CU CELL SWITCH Notification when it sends LTM cell switch MAC CE to the UE? None of the issues above can be concluded by RAN3 itself, and all have to cooperate with RAN2.

According to the above analysis, although method 2a align with the incoming LS, they bring more new issues and impacts on legacy LTM functions. At this late stage, we think that method 1 is the best choice for RAN3 to support the L3 measurement based LTM.

Proposal 1: The gNB-CU forwards the L3 measurement results to the gNB-DU via a new class 2 F1 signalling for support of L3 measurement based LTM.

Proposal 2: If the gNB-CU providing LTM target cell is preferred, RAN3 could agree on method 2b to minimize the impact on legacy LTM functions.

3	Conclusion

In this contribution, we discuss how to support L3 measurement based LTM and give the following proposal.

Proposal 1: The gNB-CU forwards the L3 measurement results to the gNB-DU via a new class 2 F1 signalling for support of L3 measurement based LTM.

Proposal 2: If the gNB-CU providing LTM target cell is preferred, RAN3 could agree on method 2b to minimize the impact on legacy LTM functions.

The reply LS and corresponding CRs are in [3] ~[6].

4	Reference

[1] R2-2407606	L3 measurement based LTM support over F1, RAN2

[2] R3-244090	LTM interwork with L3 measurements

[3] R3-245627	Support of L3 measurement based LTM over F1

[4] R3-245631	Support of  L3 measurement based LTM over F1

[5] R3-245629	Support of L3 measurement based LTM over F1

[6] R3-245630	[draft] Repy LS on L3 measurement based LTM support over F1