3GPP TSG-RAN WG3 Meeting #125bis	R3-245637

Hefei, China, 14th – 18th October 2024

Agenda Item:	12.2

Source: 				LG Electronics Inc.

Title:			Access control and location information in WAB

Document for:		Discussion and Decision

1.	Introduction

In R3-244019/S2-2407345 [1], the SA2 sent a LS on questions regarding FS_VMR_Ph2 as follows:

… Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06. …

In this LS, SA2 asks the RAN3 to provide any feedback on two issues related to the WAB support. In this contribution, we will discuss the open issues raised by the SA2 in the LS and provide our view on it.

2.	Discussion

SA2 study on MWAB is now completed. SA2 had captured the conclusions on MWAB in TR 23.700-06 [2]. Regarding two issues related to the additional ULI format and the access control to guarantee single-hop backhauling (to restrict WAB-MT accessing WAB-gNB cell), SA2 sent a LS to ask RAN3 to provide any feedback on these issues [1].

First issue in [1] is how to the access control (to restrict WAB-MT accessing WAB-gNB cell) for below two cases:

- WAB-MT accessing any WAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

- WAB-MT accessing WAB-gNB belonging to same WAB.

According to the SID in RP-234041, it is assumed that single-hop backhauling is sufficient for WAB. Hence, RAN3 captured the following requirement in TR 38.799:

- A WAB-gNB cannot serve WAB-MT(s).

In Case a), this means that the serving gNB of the WAB-MT should be a stationary gNB providing the PDU Session for backhauling. Regarding the WAB-MT in IDLE mode, SA2 considers one candidate solution in clause 6.2.3.6 of TS 23.700-06 [2]. In this solution, during the NG Setup procedure, the WAB-gNB provides to the UE’s AMF a new indication that this is a WAB (i.e., WAB-gNB indication). If the WAB-MT accesses to the UE’s AMF via the WAB-gNB, the UE’s AMF can reject the WAB-MT to prevent the multi-hop backhauling. Note that in [2], SA2 already agreed to introduce a new WAB-gNB indication in NGAP signaling as below excerpted from clause 8.1 of [2].

- The MWAB-gNB indicates to the AMF(s) it is a MWAB when it establishes the N2 connection to the AMF(s).

NOTE 2:	This requires RAN3 to define this indication.

Proposal 1: The WAB-gNB indication should be defined in the NG SETUP REQUEST message.

However, in order to support this solution, the AMF needs to be aware of whether the UE intends to operate as the WAB-MT (i.e., WAB-MT indication). According to the conclusion on SA2 study [2], there is no AS level indication that the WAB-MT intends to operate as WAB. For now, SA2 considers that dedicated S-NSSAI(s)/DNN for WAB operation is used for a WAB-MT to indicate the intension to BH-5GC. In this case, the AMF can know that the UE is a WAB-MT, based on the dedicated S-NSSAI(s)/DNN.

Observation 1: The AMF is aware of whether the UE intends to operate as the WAB-MT based on dedicated S-NSSAI(s)/DNN.

Based on above observations, we think that SA2 solution can prevent the multi-hop backhauling.

Proposal 2: It is proposed to confirm that SA2 solution (i.e., combination of WAB-gNB indication in NG SETUP REQUEST message and dedicated S-NSSAI(s)/DNN for WAB operation) guarantees single-hop backhauling (to restrict WAB-MT accessing WAB-gNB cell).

In the SA2 solution, since the WAB-MT does not have a priori knowledge on other WAB-gNB(s), it can try to access to the WAB-gNB, thus resulting in AMF rejecting the registration request from the WAB-MT. In order to prevent unnecessary access towards the WAB-gNB (even this unnecessary access trial may happen multiple times to different WAB-gNBs around), we think that the approach in Rel-18 mobile IAB can be used as a baseline. That is, the WAB-gNB may broadcast the WAB cell indication in SIB1. Then, since the WAB-MT in IDLE mode is aware of the WAB cell, the multi-hop backhauling can be prevented.

Proposal 3: The WAB-gNB should broadcast a new indication that this is a WAB cell.

Proposal 3a: Proposal 3 needs to consult with RAN2.

Note that the BH-gNB serving the WAB-MT may be not upgraded to support the WAB functionality. In other words, legacy gNB may provide the BH PDU Session for the WAB node without any knowledge on the WAB. During the mobility of the WAB-MT in CONNECTED mode, therefore, the BH-gNB (i.e., source node) cannot know whether the UE is a WAB-MT and whether the target node is a WAB-gNB. In this scenario, the BH-gNB may select the WAB-gNB as a target. If the NG handover for the WAB-MT is triggered, the AMF can reject the handover under the assumption that it already knows that the UE is the WAB-MT and the target node is the WAB-gNB. However, if the Xn handover is triggered, the target WAB-gNB cannot reject the handover of the WAB-MT because there is no information on whether the UE operates as a WAB-MT. Hence, in order to prevent the multi-hop backhauling, one possible option in this case is to always trigger the NG handover for the WAB-MT instead of the Xn handover. To this end, the WAB-gNB should not establish the Xn interface with the neighbouring stationary gNB(s) when these gNB(s) are not upgraded to support the WAB functionality. In this case, when the BH-gNB serving the WAB-MT selects the WAB-gNB as a target, the BH-gNB always triggers NG handover for the WAB-MT because there is no Xn interface between the BH-gNB and WAB-gNB. Then, the AMF can reject the handover. We think that OAM can have knowledge of whether the neighbouring gNBs are upgraded at WAB-node’s current location. Therefore, the WAB-gNB may be (re-)configured via OAM to know that the neighbouring gNB(s) are not upgraded and not aware of the WAB functionality, or that Xn setup is not needed in the location the WAB is at.

Proposal 4: When the neighbouring stationary gNBs are not upgraded to support the WAB functionality, the Xn interface between the neighbouring gNBs and WAB-gNB is not established.

Proposal 4a: When the neighbouring stationary gNBs are not upgraded to support the WAB functionality, the NG handover is always triggered for the mobility of the WAB-MT.

It is possible that the BH-gNB serving the WAB-MT is enhanced to support the WAB functionality (i.e., BH-gNB can be aware of the WAB functionality). In this case, the Xn interface may be established between the stationary gNBs and WAB-gNB. Also, the AMF may provide to the BH-gNB the WAB-MT indication during the Initial Context Setup procedure for the WAB-MT. Therefore, the BH-gNB can be aware of whether the UE operates as the WAB-MT. When the Xn handover for the WAB-MT is triggered, the BH-gNB just forwards the WAB-MT indication to the target gNB. If the target node is the WAB-gNB and it receives the WAB-MT indication from the BH-gNB, the Xn handover can be rejected by the target WAB-gNB. Based on the information on dedicated network slices instead of WAB-MT indication, the BH-gNB can be aware of whether the UE operates as the WAB-MT. It is also possible that if the WAB-gNB indication is received from the WAB-gNB during the Xn Setup procedure, the BH-gNB does not consider the WAB-gNB as a candidate target RAN node during the Xn handover. For the NG handover, the AMF can reject the handover as well.

Proposal 5: When the neighbouring stationary gNBs are upgraded to support the WAB functionality, the AMF provides the WAB-MT indication to the BH-gNB during the Initial Context Setup procedure.

Proposal 5a: When the neighbouring stationary gNBs are upgraded to support the WAB functionality, the BH-gNB provides the WAB-MT indication to the target gNB during the Xn handover.

Proposal 5b: The target gNB rejects the Xn handover for the WAB-MT based on the WAB-MT indication from the source BH-gNB.

For Case b), it seems that the WAB-MT accesses to the co-located WAB-gNB. For the WAB-MT in IDLE mode, it cannot access to the co-located WAB-gNB because the co-located WAB-gNB is not yet initialized. For the WAB-MT in CONNECTED mode, the BH-gNB may determine to initiate the handover of the WAB-MT towards the co-located WAB-gNB. However, this can be avoided if the BH-gNB already discovers the co-location of the WAB-MT and the WAB-gNB. Also, our understanding is that the WAB-MT may not report to the BH-gNB the measurement results for the co-located WAB-gNB. This is because the WAB-MT can obtain the information (e.g., gNB ID, TAC, PCI, NCGI, and so on) related to the co-located WAB-gNB by using internal interface between WAB-MT and WAB-gNB.

Proposal 6: The BH-gNB should discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 6a: It is considered that the WAB-gNB is not candidate target RAN for the co-located WAB-MT's handover.

Next question in [1] is how to define the Additional ULI for the WAB. In [2], SA2 considered the following options used for the Additional ULI in normative phase:

- Option 1: mapped TAC/Cell ID based on geo-location of the WAB based on input from OAM;

- Option 2: geo-location of the WAB;

- Option 3: TAC/Cell ID of the cell serving the WAB-MT in other PLMN

For Option 1, we think that how to determine mapped TAC/Cell ID based on geo-location of the WAB needs to be configured by OAM, thus may cause unnecessary complexity. For example, whenever the geo-location of the WAB node is changed, the WAB-gNB may contact the OAM server to obtain the mapped TAC/Cell ID related to the WAB’s current location. In order to support Option 2, it seems that a new IE needs to be defined to contain the geo-location of the WAB in Additional ULI. However, from the RAN3 point of view, this is not a critical specification impact. Also, Option 3 just follows the approach in Rel-18 mobile IAB. We think that this option has a minor RAN3 impact as well.

Proposal 7: Option 2 and 3 in clause 8.5 of TR 23.700-06 can be supported as Additional ULI format.

The above proposals are captured into a draft reply LS in Appendix. Then, the following proposal is suggested to RAN3:

Proposal 8: It is proposed to send a replay LS in Appendix to SA2.

3.	Conclusion

In this contribution, we focused on the open issues raised by the SA2 in the LS and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: The WAB-gNB indication should be defined in the NG SETUP REQUEST message.

Proposal 2: It is proposed to confirm that SA2 solution (i.e., combination of WAB-gNB indication in NG SETUP REQUEST message and dedicated S-NSSAI(s)/DNN for WAB operation) guarantees single-hop backhauling (to restrict WAB-MT accessing WAB-gNB cell).

Proposal 3: The WAB-gNB should broadcast a new indication that this is a WAB cell.

Proposal 3a: Proposal 3 needs to consult with RAN2.

Proposal 4: When the neighbouring stationary gNBs are not upgraded to support the WAB functionality, the Xn interface between the neighbouring gNBs and WAB-gNB is not established.

Proposal 4a: When the neighbouring stationary gNBs are not upgraded to support the WAB functionality, the NG handover is always triggered for the mobility of the WAB-MT.

Proposal 5: When the neighbouring stationary gNBs are upgraded to support the WAB functionality, the AMF provides the WAB-MT indication to the BH-gNB during the Initial Context Setup procedure.

Proposal 5a: When the neighbouring stationary gNBs are upgraded to support the WAB functionality, the BH-gNB provides the WAB-MT indication to the target gNB during the Xn handover.

Proposal 5b: The target gNB rejects the Xn handover for the WAB-MT based on the WAB-MT indication from the source BH-gNB.

Proposal 6: The BH-gNB should discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 6a: It is considered that the WAB-gNB is not candidate target RAN for the co-located WAB-MT's handover.

Proposal 7: Option 2 and 3 in clause 8.5 of TR 23.700-06 can be supported as Additional ULI format

Proposal 8: It is proposed to send a replay LS in Appendix to SA2.

4.	Reference

- R3-244019/S2-2407345, “LS on questions regarding FS_VMR_Ph2,” Samsung, May, 2024.

- TR 23.700-06, “Study on architecture enhancements for vehicle-mounted relays - Phase 2,” v19.0.0.

5.	Appendix: Reply LS for S2-2407345

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #125bis R3-24xxxx Hefei, China, 14th – 18th October 2024 # Title: [Draft] Reply LS on questions regarding FS_VMR_Ph2 # Response to: R3-244019/S2-2407345 # Release: Rel-19 # Work Item: FS_VMR_Ph2, NR_WAB_5GFemto-Core Source: RAN3 To: SA2, RAN2 Cc: - Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: none # 1 Overall description RAN3 thanks SA2 for the LS on questions regarding FS_VMR_Ph2. RAN3 would like to provide the following feedback to SA2 regarding two open issues. Also, RAN3 has captured the conclusions on WAB in TR 38.799. The study item on WAB is now completed in RAN3. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. RAN3’s Answer for Case a): RAN3 has discussed the aspect on how to prevent multi-hop backhauling and then confirms that SA2 solution (i.e., combination of MWAB-gNB indication in NG SETUP REQUEST message and dedicated S-NSSAI(s)/DNN for MWAB operation) can be used to prevent multi-hop backhauling. To support this SA2 solution, it is also agreed to newly introduce a MWAB-gNB indication in NG SETUP REQUEST message. In order to prevent unnecessary access trials of the MWAB-UE towards the MWAB-gNB, RAN3 also considers that it is beneficial for the MWAB-gNB to broadcast a new indication that this is a MWAB cell in SIB. Hence, RAN3 would like to request RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. RAN3’s Answer for Case b): RAN3’s view is that this can be managed by e.g. MWAB implementation, information about co-location of MWAB-gNB and MWAB-UE to BH-gNB. RAN3 will further discuss on this issue during normative phase. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06. RAN3’s Answer: Option 2 and 3 in clause 8.5 of TR 23.700-06 can be supported as Additional ULI format. 2. Actions: To SA2: ACTION: RAN3 kindly asks SA2 to take the above into account. To RAN2: ACTION: RAN3 kindly asks RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#126 18 – 22 November 2024 Orlando, USA 3GPP TSG RAN3#127 17 – 21 February 2025 Greece, Athens

-----------------End of the First Change---------------