3GPP TSG-RAN WG3 #125bis								R3-245640

14th – 18th Oct 2024, Hefei, China

Agenda Item:	12.2

Source:	China Telecom

Title: 	Discussion on RAN2 impact of WAB

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. The objectives of the Wireless Access Backhaul (WAB) work are as follows:

-	Specifications for the support of WAB including [RAN3]:

-	Support of a WAB-node including a WAB-gNB and a WAB-MT.

-	Support of backhauling of the WAB-gNB’s NG, Xn and OAM traffic over the WAB-MT’s PDU session(s).

-	Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

-	Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes.

-	Network integration procedures for WAB nodes.

-	Handling of WAB-gNB’s traffic (including Xn, NG and OAM traffic) during WAB-node mobility, including the case where the WAB-MT’s BH PDU session changes.

-	Support the UE’s AMF change for UEs connected to, or camped on, a WAB-gNB.

-	UE’s ULI that reflect the WAB node’s location.

-	The handling of:

-	PCI collision avoidance.

-	Reconfiguration of TAC and RANAC on WAB-gNBs.

-	Mechanisms to avoid multi-hop WAB topology.

-	Radio-resource coordination between access and backhaul links.

-	NG connection management.

NOTE 1: For PCI collision avoidance and reconfiguration of TAC and RANAC on WAB-gNBs, follow the conclusion of mobile IAB.

NOTE 2: NG connection management should take the NTN conclusion into account, avoiding parallel discussions.

NOTE 3: No impact on the UE.

NOTE 4: Coordination with other WGs (e.g. SA2, RAN2) when needed.

NOTE 5: Backhaul link for WAB-MT can be TN or NTN.

NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

NOTE 7: The interface between the WAB-MT and the co-located WAB-gNB is out-of-scope for the normative phase.

NOTE 8: Split architecture of the WAB-gNB is out-of-scope for the normative phase.

NOTE 9: RAN2 impact should be identified as early as possible, and should be minimal.

In SI phase, there was rarely discussion on RAN2 impact as no impact on Uu interface was assumed in WAB. Since the WAB entity also comprises of two entities, namely WAB-MT and WAB-gNB, like IAB and NCR, it is need to specify the functionalities of WAB-MT and the behaviour of WAB-gNB during mobility procedure at least in stage 2. Based on this analysis, we believe it is need to study and identify RAN2 impact as NOTE 9 in WID.

Discussion

RRC States

According to SA2 CR[2] in TS23.501, WAB-MT needs to support all the RRC states.

The existing UE mobility procedures without MWAB-specific UE enhancements can be reused to handle a UE mobility to and from a MWAB-gNB while the UE is in CM-IDLE or CM-CONNECTED modes, irrespective of whether the MWAB is static or the MWAB is moving, i.e.:

-	For UEs in RRC_IDLE and RRC_INACTIVE state procedure for cell (re-) selection as specified in TS 38.304 [50] and TS 23.122 [17] for RRC_IDLE and RRC_INACTIVE is used as normal irrespective of cells are belonging to a MWAB-gNB or a gNB.

-	For UEs in RRC_CONNECTED state, the MWAB-gNB or gNB triggers the handover procedures to the neighbouring cells (irrespective of whether they are belonging to a MWAB-gNB or a gNB) for UEs in RRC_CONNECTED state.

The behaviour and functionalities of NCR-MT and IAB-MT are captured in existing stage 2 and stage 3 specifications. As is quoted in TS38.300, the NCR-MT supports all RRC states and the behaviour of NCR-Fwd during state transition. The WAB-MT also needs to support all the existing RRC states, namely Connected, Idle, and Inactive. In addition, WAB-MT also needs to support all SRBs as it may be configured with single carrier or MR-DC by BH gNB(s). Based on the above analysis, it is concluded that WAB-MT need to support all RRC States and all SRBs.

Proposal 1: it is concluded that WAB-MT need to support all RRC States and all SRBs.

In Connected state, the WAB-gNB could send/receive signal to/from UE and establish Xn and NG interface with BH gNB/5GC. Regarding the behaviour of WAB-gNB for Idle and Inactive state, there was not discussed in the past RAN3 and RAN2 meetings. Upon the WAB-MT transit from Connected state to Idle state, the WAB-gNB ceases to send/receive signal to/from UE and remove Xn/NG interface, and redirection all UEs to other NG-RAN nodes or E-UTRAN nodes. For Inactive state, whether WAB-gNB ceases to send/receive signal to/from UE need to be studied in RAN2. From the perspective of RAN3, the Xn/NG interface could be kept as the arrival of Xn-C/NG-C signalling or Xn-U/NG-U user plane data may trigger WAB-MT transit from Inactive to Connected state. In a word, the behaviour of WAB-gNB in Idle and inactive state of WAB-MT need to be discussed in RAN2.

Proposal 2: the behaviour of WAB-gNB in Idle and inactive state of WAB-MT need to be discussed in RAN2.

RLF and BFD/BFR

According to the NOTE6 in objective part of WID, the Mobility of the WAB-MTs reuses the legacy UE mobility procedures. This NOTE also implies that the WAB-MT shall supports RLF detection, Beam Failure Detection (BFD) and Beam Failure Recovery (BFR) procedure as normal UE.

NOTE 6: Mobility procedures to be used for the UEs served by a WAB-gNB are legacy UE mobility procedures. Mobility of the WAB-MTs is based on legacy UE mobility procedures.

Upon the RLF is detected during mobility procedure, WAB-MT performs the RRC re-establishment procedure as legacy UE. During RRC re-establishment procedure, the behaviour of need to be discussed in RAN2 as the WAB-gNB disconnects with BH gNB before the re-establishment procedure is completed. For NCR-MT, the behaviour of NCR-Fwd also need to be specified in stage 2 specification.

Proposal 3: The behaviour of WAB-gNB during RLF or Beam Failure need to be discussed in RAN2 WI phase.

Access Control

In order to avoid another WAB-MT tries to access the cell of WAB-gNB, it is need to specify a mechanism to avoid multi-hop. One potential solution is to introduce a new indicator in SIB1 to indicate the support of WAB. The WAB-MT shall consider the cell broadcast this new indicator as a barred cell while the legacy ignores this indicator. In addition, since IAB and NCR are also alternatives to extend coverage and enhance wireless topological, whether the WAB supports NCR and IAB also need to be decided by RAN2. In our view, WAB does not supports NCR and IAB to avoid multi-hop and stage 2 description on the conclusion is also needed.

Proposal 4: whether the WAB supports NCR and IAB also should be decided by RAN2.

Moreover, the following issues on IAB-MT and NCR-MT were discussed in the scope of RAN2, and we understand RAN2 also need to discuss whether they are also needed for WAB-MT:

- Cell status and cell reservations: Both IAB-MT and NCR-MT ignore the existing cell status and reservations in SIB1, e.g, cellBarred, cellReservedForOperatorUse, cellReservedForFutureUse, intraFreqReselection, and cellReservedForOtherUse.

- Unified Access Control: Unified Access Control does not apply to IAB-MTs and NCR-MTs

- additional allowed cell list and/or forbidden cell list: NCR-MT may be configured with additional allowed cell list and/or forbidden cell list. And The NCR-MT shall consider only the allowed cell list for reselection candidate cells.

Proposal 5: RAN2 needs to discuss whether the Cell status and cell reservations, Unified Access Control and additional allowed cell list and/or forbidden cell list are really needed in WAB

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: it is concluded that WAB-MT need to support all RRC States and all SRBs.

Proposal 2: the behaviour of WAB-gNB in Idle and inactive state of WAB-MT need to be discussed in RAN2.

Proposal 3: The behaviour of WAB-gNB during RLF or Beam Failure need to be discussed in RAN2 WI phase.

Proposal 4: whether the WAB supports NCR and IAB also should be decided by RAN2.

Proposal 5: RAN2 needs to discuss whether the Cell status and cell reservations, Unified Access Control and additional allowed cell list and/or forbidden cell list are really needed in WAB.

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T

- S2-2409245，Mobility of UEs served by MWAB,Huawei, HiSilicon, Nokia, Ericsson, ZTE