3GPP TSG-RAN WG3 #125bis								R3-245641

14th – 18th Oct 2024, Hefei, China

Agenda Item:	12.2

Source:	China Telecom

Title: 	On Xn and NG interface management

Document for:	Discussion

Introduction

The new WI on additional topological enhancements for NR was approved in RAN#105 meeting [1]. On Xn and NG interface management, the following objectives are captured in WID:

-	Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

-	Defining the behaviour of WAB-node in case the authorization status of WAB-MT and/or WAB-gNB changes.

-	The handling of:

-	PCI collision avoidance.

-	Reconfiguration of TAC and RANAC on WAB-gNBs.

-	Mechanisms to avoid multi-hop WAB topology.

-	Radio-resource coordination between access and backhaul links.

-	NG connection management.

This contribution aims to further discuss the issues on Xn and NG interface management.

Discussion

WAB Awareness for BH RAN

In last RAN3 meeting, one open issue needs to be further studied is whether a BH RAN node can be unaware of WAB. The WAB awareness information could provide useful information for BH RAN to configure dedicated resource for WAB-MT or WAB-gNB. However, from the perspective of signalling enhancement, we prefer to not introduce any new indicator over air interface. According to the objective of SI on WAB, WAB is expected to not impact on air interface. Additionally, SA2 had also concluded that there is no need to specify a new indication in RRC connection establishment that the MWAB-UE intends to operate as MWAB. Regarding whether new UE capability is needed for WAB-MT, we understand that BH RAN node does not have additional operation for WAB-MT and it also do not need to really know what slices are used for WAB operation although SA2 agreed to configure dedicated slices for a MWAB-UE to indicate the intension to BH 5GC. Moreover, the authorization information for WAB-MT can be send via NAS signalling. To sum up, BH RAN does not need to aware of WAB and no need to introduce a new indicator over air interface and NG interface.

Proposal 1: BH RAN does not need to aware of WAB and no need to introduce a new indicator over air interface and NG interface.

Since the WAB-gNB may be deployed in abroad ship, the Xn interface between between WAB-gNB and BH gNB may not always exists. Based on this, both WAB-gNB and BH-gNB may remove the cell relation and Xn interface after a period of time. In order to address this issue, a new indicator information can be contained in Xn Setup message to indicate the survive time of Xn interface.

Proposal 2: a new indicator information can be contained in Xn Setup message to indicate the survive time of Xn interface.

WAB authorization

According to the conclusion of SA2, the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs. Therefore, it is need to introduce a new indicator in NG Setup Request/RAN Configuration Update Message to indicate it is a WAB node. This indicator cannot be changed after NG interface was established. Without this new indicator contained in NG Setup Request/RAN Configuration Update Message, the AMF will take this node as legacy gNB.

Proposal 3: the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs.

According to RAN3 agreements in last meeting, authorization of the WAB-MT is different from the WAB-gNB service authorization/configuration/activation by e.g. OAM/SeGW.

If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. As regards whether SCTP connection for Xn and NG interface is still maintained, we understand it depends on pre-configuration or OAM configuration. If the WAB-MT authorization status changes from “not authorized” to “authorized” , since BH PDU session and its corresponding DRBs resource can be reallocated to WAB-gNB, it could perform the normal WAB-gNB setup procedure.

Proposal 4: If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection

If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the authorization information can be delivered via NGAP message., NG Setup Response/RAN Configuration Update ACK message. The WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. In this case, the SCTP connection for Xn and NG interface should be also removed. While the authorization information is triggered by OAM, the WAB-gNB could also not continue to provide service to any UEs and needs to ceases to receive/send signal from/to UE and remove Xn/NG connection. Since the BH PDU session can be still kept and, the SCTP connection can be maintained.

Proposal 5: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases to receive/send signal from/to UE and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: BH RAN does not need to aware of WAB and no need to introduce a new indicator over air interface and NG interface.

Proposal 2: a new indicator information can be contained in Xn Setup message to indicate the survive time of Xn interface.

Proposal 3: the WAB-gNB indicates to the AMFs it is a MWAB when it establishes the NG connection to the AMFs.

Proposal 4: If the WAB-MT authorization status changes from “authorized” to “not authorized”, the WAB-MT acts as a normal UE and WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection

Proposal 5: If SeGW trigger the WAB-gNB authorization status changes from “authorized” to “not authorized”, the WAB-gNB could not continue to provide service to any UEs and needs to ceases broadcast and remove Xn/NG connection. If the authorization status changes is triggered by CN, the SCTP connection shall be removed.

Reference

- RP-242395，New WID on additional topological enhancements for NR，NTT DOCOMO, INC., AT&T