3GPP TSG-RAN WG3 #125bis								R3-245644

14th – 18th Oct 2024, Hefei, China

Agenda Item:	16.3

Source:	China Telecom

Title: 	Discussion on RAN-CN interface for Ambient IoT

Document for:	Discussion

Introduction

The study item on solutions for Ambient IoT (Internet of Things) in NR was agreed in RAN#102 meeting [1]. The Ambient IoT aims to design ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The objectives of the Ambient IoT in RAN3 are as follows:

  - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable:

        - Paging

        - Device context management

        - Data transport

In RAN3#125 meeting, the following open issues were made:

For Topology2, whether AIoT RAN needs to be revised as AIoT enabled gNB needs to be further checked in next meeting.

RAN3 starts with inventory-only procedure. Discuss the content in Inventory/Command.

The AIoT CN to select suitable reader (AIoT enabled gNB or AIoT enabled UE reader) for both topology 1 and topology 2?

Assistance information from AIoT CN to AIoT RAN?

This contribution aims to the leftover issues on device context and content in inventory and command for Ambient IoT.

Discussion

NG Signalling for Ambient IoT

Since Ambient IoT is a very simple function, there is no need to specify a new protocol stack between RAN and CN to support Ambient IoT scenarios. From the perspective of RAN3, there is no need to extend the existing NGAP message to support Ambient IoT as it may be regarded as a new RAT. Therefore, for topology 1 and 2 we prefer to specify a few new procedures (e.g., paging) in NG interface to support Ambient IoT scenario rather than to extend the existing NGAP message.

Proposal 1: To specify a few new procedures (e.g., paging) in NG interface to support Ambient IoT scenario rather than to extend the existing NGAP message for both Topology 1 and 2.

Content in Inventory for Topology 1

In RAN3#124 meeting, whether the following information for topology 1 need to be contained in inventory message was discussed:

- [M] AIoT Device ID MASK (to find a single device, a group of devices, or all devices)

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

We prefer to not introduce an additional device ID mask to find a group of devices as device ID may comprise of multiple bits like gNB-ID or cell ID. If the CN decide to find a group of devices, it can only send part of device ID bits to NG-RAN. The A-IoT device with the same part of device ID bits need to treat this inventory message. Therefore, it is need to specify a variable length of A-IoT device ID in inventory message. Regarding the inventory session ID, we support to introduce this session ID as specifications need to allow multiple Inventory procedure instances to run in parallel over RAN-CN interface. Considering the A-IoT RAN system may cover multiple regions in practical deployment, the inventory area list needs to be contained in Inventory message. The estimated number of devices can be used to enable CFRA in case of single device inventory. With this information, the RAN node can configure an appropriate configuration for CFRA and timing configuration. Additionally, A-IoT system may also perform periodic inventory to allow the newly coming AIoT devices to be discovered. The AF may further provide the inventory strategy information (e.g. inventory period) to perform periodic inventory without further explicit requests from the AF. The inventory strategy information can be configured via Inventory message or Reconfigure/Modified via other NGAP message.

In conclusion, the following information can be included in A-IoT Inventory message:

- [M]a variable length of A-IoT device ID

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

Proposal 2: the following information can be included in A-IoT Inventory message:

- [M]a variable length of A-IoT device ID

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

Content in Command message

In addition, since the Commands refers to an instruction (e.g., Read,Write, Disable) sent by an AF to an Ambient IoT Device, are end-to-end protected between AF and Ambient IoT Device, the AMF may include one or more Commands information in Paging message. Therefore, the Command Information with String type can be also included in Command message. Additionally, the usage of Command (e.g., Read, Write, Disable) is also need to be contained in Command Message.

Proposal 3: the Command Information with String type and the usage of Command are contained in Command Message.

Device Context

In legacy NR mechanism, the UE context management consists of at least Initial UE context request/medication/release procedures. In these procedures, both AMF and RAN allocate or reallocate the corresponding UE ID, mobility restriction, security configuration, slice configuration, and etc. With this information, RAN may configure or reconfigure SRB or DRB configuration for UE. In RAN3#124 meeting, the definition of Device Context in RAN/reader side was not reached consensus. At least RAN/reader may respond inventory, NAS message, command, the temporarily device ID needs to be contained in corresponding message. Other information contained in device context depends on SA2 progress.

Proposal 4: At least temporarily device ID needs to be contained in corresponding message. Other information contained in device context depends on SA2 progress.

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: To specify a few new procedures (e.g., paging) in NG interface to support Ambient IoT scenario rather than to extend the existing NGAP message for both Topology 1 and 2.

Proposal 2: the following information can be included in A-IoT Inventory message:

- [M]a variable length of A-IoT device ID

- [M] Inventory Session ID

- [M] Inventory Area List

- [O] Estimated Number of devices

- [O] Periodicity

Proposal 3: the Command Information with String type and the usage of Command are contained in Command Message.

Proposal 4: At least temporarily device ID needs to be contained in corresponding message. Other information contained in device context depends on SA2 progress.

Reference

- RP-234058,New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair)