3GPP TSG-RAN WG3 Meeting #R3-125bis		R3-245645     Hefei, China, Oct 14-18, 2024

Title: 	Initial considerations on Inter-CU LTM

Source: 	Rakuten

Agenda item:	13.2

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution, we discuss the resource management issues associated with configuration and maintenance of multiple candidate cells, the impacts on gNB-DU and the importance of gNB-DU initiated reconfiguration of LTM target cell configuration.

# 2. Discussion

## 2.1 Resource management for multiple LTM target cells

The lead time (duration between LTM candidate cell preparation and execution) in an LTM cell switch could be unpredictably long. Secondly, not all prepared candidate cell configurations end up in LTM cell switches. Radio resources are scarce and considering that a UE can be configured with multiple LTM candidate cells, resource reservation is always going to be an issue. Reservation of radio resources in all the configured LTM candidate cells for a UE leads to sub-optimal performance and overhead on the network. Especially considering that this is applicable on a large scale, for all UEs configured with LTM in the network.

Rel18 LTM requires the UE to perform UL sync to acquire timing advance from the candidate/target cell before the serving cell switch with the objective of performing a RACH-less LTM cell switch and therefore reducing HO latency and reducing user service interruption.

Based on the UE’s L1 measurements, the serving gNB-DU may ask the UE to perform UL sync with the candidate/target cell to acquire UE’s candidate/cell timing advance. Hence, UE performing UL sync with the candidate/target cell can be considered to indicate a likelihood of UE undergoing LTM cell switch to the given target cell.

To address the resource reservation issues, the following could be considered - at the time of preparing an LTM candidate cell, the gNB-CU could indicate to the candidate/target gNB-DU whether or not the UE is going to be configured to perform RACH based TA acquisition, as there could be UEs who could be configured with UE capability-based TA acquisition. Based on this, the target gNB-DU shall not reserve all radio resources immediately at the time of LTM candidate/target cell configuration preparation.

Based on the UE performing TA acquisition with a candidate cell, the candidate/target gNB-DU can anticipate an incoming LTM cell switch and initiate resource reservation at the candidate/target cell including update of a RACH resource configuration corresponding to the UE’s best beam/beam-group at that point of time.

This introduces multiple benefits

- This ensures that the resource reservation at the target gNB-DU is performed very close to the LTM serving cell switch and hence optimized, as the likelihood of an LTM cell switch is strong. This also prevents unnecessary blocking of resources for a longer duration in case of long lead times.

- This also ensures that the right RACH resources corresponding to the best beam/beam-group are configured at the candidate/target gNB-DU.

- The UE performing UL sync can be a good indicator of the likelihood of a LTM serving cell change. This enables the candidate/target gNB-DU to issue a target cell re-configuration (if required) based on the prevalent resource conditions at that time.

The resource reservation changes may result in a modified candidate/target cell configuration at the candidate/target gNB-DU and could be delivered to the UE via the gNB-CU using a gNB-DU initiated F1: UE Context Modification Required procedure.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

However, in Rel18, gNB-DU initiated LTM candidate cell re-configuration is not permitted by RAN3. This is a limitation considering the fact that there could be changes in resource status and availability between LTM candidate cell preparation and execution. As an example, a simple RACH preamble modification based on the best beam or beam-group change isn’t possible after the candidate cell configuration has been prepared. Once the candidate cell configuration is prepared, it is either executed or cancelled. This, in our view, is too inflexible and limiting genuine cases requiring re-configuration.

Proposal 1: serving gNB-CU indicates to the candidate/target gNB-CU about the timing acquisition mechanism configured to the UE.

Proposal 2: RAN3 reconsiders gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the gNB-DU.

## 2.3 Assistance information for target cell preparation

L1/L2 triggered mobility could be associated with multiple flavours like Inter Cell Beam Management (ICBM), ICBM with dynamic switching, LTM Serving cell switch, LTM serving cell change with dynamic switching etc. Each of these variants need minor variation in resource reservation at the candidate/target cell.

For example, LTM serving cell switch could be considered as the primary form where the UE undergoes serving cell change by adopting the target cell configuration.

In Inter Cell Beam Management (ICBM), CSI measurement configuration and TCI states for PDCCH and PDSCH are modified to include target cell (intra DU or inter DU) aspects.

Configurations are extended to include resources of multiple cells.

- The reporting configuration for the UE is maintained as common across all cells associated with ICBM, the target cells may reserve resources at the same location in PUSCH/PUCCH for reporting.

- Modification of beam management related configuration to enable efficient beam switching operation without impacting cell specific beam refinement operation within same cell is needed.

- Paging and System Information Broadcast are performed from the serving cell.

Dynamic switching between two intra gNB-DU or inter gNB-DU cells is when ping-pong is expected between them, and the involved candidate cells have to be prepared for that without causing any additional damage to the user-plane interruption time.

Since gNB-CU is responsible for preparation of LTM target cells, we think two aspects are very important here.

- Awareness at gNB-CU regarding the technical feasibility of the desired target cell configuration at the target gNB-DU.

- Resource situation at the target gNB-DU to accede to the request made by gNB-CU.

For a), the gNB-CU may need to be aware of the UE’s current configuration at serving gNB-DU. On top of this, the gNB-CU needs to be able to decide if the same or a different configuration is feasible at the target gNB-DU. Some of this may need serving gNB-DU specific configuration information at the gNB-CU.

For b), each gNB-DU which could be a potential target gNB-DU, the gNB-CU needs awareness of resource availability of each variant before making a request, failing which there could be a lot of rejections.

Observation 2: Awareness of source configuration at gNB-DU is required at gNB-CU to determine technical feasibility of the desired LTM target cell configuration at candidate/target gNB-DU and awareness of resource situation at each target gNB-DU is required at gNB-CU before initiating candidate/target cell preparation.

Proposal 3: RAN3 discusses and agrees a method to enable selection of the appropriate variant of LTM during candidate cell preparation.

## 2.3 Security aspects of inter-CU LTM

## 2.3.1 Security impacts of inter-CU LTM on intra-CU LTM

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

In Rel 19, RAN2 #125-bis made the above agreement which is to support inter-CU LTM with security key change. This is because the source and target PDCP entitties located in the gNB-CU-UP are different in an inter-CU LTM scenario. However, there is an impact of this on intra-CU LTM also.

Let us consider the below figure which depicts a gNB with the following elements,

  - A gNB having a gNB-CU-CP and two gNB-CU-UPs serving a subset of gNB-DUs.

  - gNB-CU-UP1 serves gNB-DUs, DU1..DU5.

  - gNB-CU-UP2 serves gNB-DUs, DU6..DU10.

It should be noted here that when a UE undergoes LTM cell switch from a cell belonging to DU5 to a cell belonging to DU6, it also undergoes intra gNB inter-gNB-CU-UP relocation. When a UE undergoes intra gNB gNB-CU-UP relocation, it changes the PDCP anchor point and hence there is a need to refresh the security keys. This implies that new security keys generated have to be delivered to the UE. This also implies that the gNB-CU-UP has to be included while preparing such an LTM candidate cell.

This scenario was excluded in Rel 18 as no security change was supported for LTM. This has to be considered in Rel 19 along with inter-CU LTM with security key change.

Figure 1: Inter gNB-DU LTM HO with intra gNB gNB-CU-UP relocation

Observation 3: Security key refresh is required when a UE undergoes inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

Proposal 4: 	Security Key refresh is supported for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

## 2.3.2 Security handling in inter-CU LTM scenario

Security key refresh is required at inter gNB mobility as the PDCP anchor point i.e gNB-CU-UP is bound to change. There is no difference to this even when LTM solutions are applied for UE mobility.

The below agreement was made in RAN2 #125-bis.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

Here in this paper, we discuss the FFS: if there is an inter-CU LTM w/o security key change as it has impacts on RAN3.

A UE configured with LTM may have both intra and inter-frequency candidate cells. In scenarios where two different gNBs are on two different frequency layers (eg: one on FR1 and another on FR2), the inter-CU LTM is not limited just to the gNB edges (as in case of intra-frequency candidate cells), it may have to be configured at all locations inside a gNB.

Hence, the logic that inter-CU LTM is just limited to geographical boundaries of a gNB is not true. This also entails that inter-CU LTM may still be applicable when a UE is well inside one gNB and hence inter-CU LTM  without security key change i.e by anchoring the PDCP entity from the old serving gNB is not sustainable as it may cause significant signaling overhead and compromise on data transmission latency between CU-UP and the gNB-DU.

Figure 2: Exemplary deployment scenario for inter-CU LTM

In the above figure 2, the Red cells belong to a FR1 macro gNB and the Green cells are small cells of FR2 frequency range and belong to a different gNB. A UE is shown in one of the cells where it can be easily visualized to have inter-CU LTM candidate cells. In this case, the concept of PDCP anchoring from a previous serving gNB becomes unnecessarily complex.

Figure 3: Exemplary deployment scenario for inter-CU LTM without security key change

Another issue that we foresee is that the candidate/target gNB may need a direct Xn interface with the gNB hosting the anchor CU-UP to establish the UL/DL datapath. In such a case inter-CU LTM without security key change will not work and hence CU-UP has to be relocated.

For example, in the above figure 3, let’s consider the case where the UE moves from gNB1 (DU1) to gNB2 (DU2) to gNB3(DU3). In the scenario where the CU-UP is anchored from the first serving gNB, it may work when the UE moves under the coverage of gNB2, albeit with some unnecessary and complex signaling to establish data path between CU-UP1 and DU2. But when the UE moves to gNB3, this doesn’t work as there is no Xn interface between gNB3 and gNB1. In such cases, the CU-UP has to be relocated.

In summary

- Inter-CU LTM without security key change consumes additional signaling and latency between a serving gNB-CU-CP and anchor gNB-CU-CP (to setup and manage anchor gNB-CU-UP).

- Inter-CU LTM doesn’t provide any benefit wrt RLC handling, as the gNB-DU is going to be different in anycase.

- Inter-CU LTM without security key change has the benefit of not requiring data forwarding, as the PDCP entity at the gNB is not changed. Since data forwarding is over the backhaul, it cannot be considered as a gamechanger.

- Even with Inter-CU LTM without security key change, gNB-CU-UP relocation (PDCP + security key change) becomes mandatory when there is no direct Xn interface between the target gNB and the gNB hosting the anchor PDCP. This leads to dual solution implementation causing additional effort.

Therefore, we don’t think it is worth the trade-off having an inter-CU LTM solution without security key change, especially when Inter-CU LTM with security key change is already in place.

We also think that inter-CU LTM with security key change is inevitable, and hence we see no benefit in inter-CU LTM without security key change.

- Observation 4: Inter-CU LTM scenarios are not limited to geographical boundaries of a gNB, they could occur at any location in a gNB.

- Observation 5: gNB-CU-UP relocation (PDCP + security key change) becomes mandatory when there is no direct Xn interface between the target gNB and the gNB hosting the anchor PDCP.

- Observation 6: This handover without security key change is not specific to LTM. Any other handover, for eg L3 handover could also be impacted due to this.

- Observation 7: There is no visible benefit of inter-CU LTM without security key change compared to inter-CU LTM with security key change.

- Observation 8: Inter-CU LTM with security key change solution is sufficient to address all use-cases of inter-CU LTM.

- Proposal 5: Security Key refresh is always executed for inter-CU LTM.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Observation 1: gNB-DU initiated target cell re-configuration can be used to enable optimized resource reservation.

Observation 2: Awareness of gNB-DU specific information at gNB-CU to determine technical feasibility of the desired LTM target cell configuration at target gNB-DU and awareness of resource situation at each target gNB-DU is required at gNB-CU before initiating target cell preparation.

Observation 3: Security key refresh is required when a UE undergoes inter gNB-DU mobility that results in an intra gNB gNB-CU-UP relocation.

- Observation 4: Inter-CU LTM scenarios are not limited to geographical boundaries of a gNB, they could occur at any location in a gNB.

- Observation 5: gNB-CU-UP relocation (PDCP + security key change) becomes mandatory when there is no direct Xn interface between the target gNB and the gNB hosting the anchor PDCP.

- Observation 6: This handover without security key change is not specific to LTM. Any other handover, for eg L3 handover could also be impacted due to this.

- Observation 7: There is no visible benefit of inter-CU LTM without security key change compared to inter-CU LTM with security key change.

- Observation 8: Inter-CU LTM with security key change solution is sufficient to address all use-cases of inter-CU LTM.

Proposal 1: Serving gNB-CU indicates to the candidate/target gNB-CU about the timing acquisition mechanism configured to the UE.

Proposal 2: RAN3 reconsiders gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the gNB-DU.

Proposal 3: RAN3 discusses and agrees a method to enable selection of the appropriate variant of LTM during candidate cell preparation.

Proposal 4: 	Security Key refresh is supported for inter gNB-DU LTM, when gNB-CU-UP relocation occurs inside the same gNB.

- Proposal 5: Security Key refresh is always executed for inter-CU LTM.

# 4. Reference

RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024