3GPP TSG-RAN WG3 Meeting #R3-125-bis	                                                               R3-245647

Hefei, CH, Oct 14-18, 2024

Agenda Item:	17.3

Source: 	Rakuten

Title:  	Discussion on OD-SIB1

Document for:	Discussion and decision

# Introduction

The Rel-19 WI on enhancements of network energy savings for NR (RP-234065), with the objective to study on-demand SIB1 for idle/inactive UEs as highlighted below:

Study procedures and signalling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] Triggering method by uplink wake-up-signal using an existing signal/channel. Wake-up-signal configuration provisioning to UE Note: No modification of SSB will be discussed under this objective Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. Checkpoint for normative work in RAN#105

# Activation of Energy Saving (ES) mode on NES Cell

As per TS38.473, gNB-DU is the network entity responsible for encoding and broadcasting SIB1 for a given cell. Hence, the information related to all the fields of SIB1 is available at the gNB-DU.

gNB-DU is the network entity responsible for broadcasting SIB1.

NOTE: In this document, we call the gNB-DU that is associated with the NES cell, a “NES cell gNB-DU” and the gNB-CU that is associated with the NES cell, a “NES cell gNB-CU”. Likewise for Cell A also.

- What are the various possibilities of SIB1 transmission in the NES cell?

The NES cell is a cell that may transmit On Demand SIB1 (OD-SIB1) transmission in response to UL WUS from a UE. It need not transmit periodic SIB1 (default transmission) all the time. Based on this understanding, a NES cell can have one of the following three “SIB1 modes”:

- Default mode: NES cell is currently broadcasting periodic SIB1

- OD-SIB1 mode: NES cell is currently broadcasting only OD-SIB1

- NES mode: NES cell is currently not broadcasting either periodic SIB1 or OD-SIB1

NES cell gNB-DU to store “SIB1 mode” which reflects whether the NES cell is transmitting periodic SIB1 (default), OD-SIB1, or not transmitting any SIB1 (NES mode).

We propose an introduction of ES Assistance Information which is transmitted from NES cell gNB-DU to NES cell gNB-CU over the F1 interface which carries the relevant information like the cell id with corresponding proposed/current SIB1 mode, UL WUS Configuration etc.

Additional fields may be added to the ES Assistance Information as further progress is made in the topics of NES.

- Which entity in the NES cell makes the Energy Saving decision

There are several reasons which make the NES cell gNB-DU the preferred entity to decide whether the NES cell should transition to an energy saving mode.

Firstly, for any cell, the gNB-DU is aware of the current traffic, including the RRM resources in use, number of UEs connected or camped to the cell and the type of bearers established for the UEs.

Secondly, the gNB-DU receives the UL WUS configuration from the UEs when the UE intends to trigger OD-SIB1 from the NES cell.

And lastly, in any cell, the gNB-DU is the entity which broadcasts SIB1.

When the NES cell is in the NES node, on receiving the UL WUS configuration from a UE, the NES cell gNB-DU can begin OD-SIB1 transmission without any signaling overhead with any other entity e.g. NES cell gNB-CU. Moreover, on observing no/low traffic in the cell, the NES cell gNB-DU may decide to switch from default mode to NES mode, where it stops ongoing SIB1 transmissions.

Hence, we propose that the decision of which SIB1 mode be used should lie with the NES cell gNB-DU. This may be done in agreement with NES Cell gNB-CU.

NES cell gNB DU is the preferred network entity which is best equipped to decide SIB1 mode to be used in the NES cell. This may be done in agreement with NES Cell gNB-CU and a “confirmation” from Cell A.

- Note: The above decision is possible only when Cell A “confirms” the transmission of UL WUS configuration of the NES cell. This is discussed in detail in the next section.

What are the triggers for NES cell to transmit OD-SIB1 or periodic SIB1?

As proposed in the previous paragraph, the decision of which SIB1 mode to adopt lies with the NES cell gNB-DU. So irrespective of the trigger, the request for OD-SIB1 must come to the NES cell gNB-DU. Following are some of the triggers which can result in NES gNB-DU deciding on SIB1 transmission:

- If the gNB-DU in NES mode receives UL WUS request from a UE, then it changes the SIB1 mode to OD-SIB1 mode, and starts broadcasting OD-SIB1.

- If the gNB-DU in NES mode or OD-SIB1 mode receives a large number of UL WUS requests from multiple UEs, it may decide to change the mode to default SIB1 mode and begins periodic SIB1 transmission to deactivate energy saving.

- If the gNB-CU receives a large number of incoming HO, or UE attach requests, then it may request the gNB-DU to deactivate the energy saving mode and begin periodic SIB1 transmission (default SIB1 mode).

- Under some conditions in Cell A like an idle mode overload condition, Cell A may want to activate load balancing and offload some of its IDLE/INACTIVE UEs to NES cell(s) if possible. In which case, Cell A may trigger the NES cell to broadcast OD-SIB1 or periodic SIB1 depending on particular use case. This request will be sent from Cell A gNB-CU to NES cell gNB-CU over Xn and then to NES cell gNB-DU over F1 interface.

We have proposed with examples various triggers from NES cell gNB-DU, NES cell gNB-CU and Cell A to activate the transmission of OD-SIB1 or periodic SIB1 by NES Cell

What are the triggers for NES cell to stop transmission of OD-SIB1 or periodic SIB1?

Typically, when NES cell gNB-DU observes there are no UEs in RRC Connected, RRC Inactive or Idle mode, then it may decide to activate the energy saving mode. This may be done in agreement with NES Cell gNB-CU and confirmation from Cell A. There may be other conditions configured by the network operator to activate energy saving.

When the conditions are met, the NES cell gNB-DU stops broadcasting all SIB1 transmissions and sends SIB1 mode change update to NES cell gNB-CU over the F1 interface.

# NES Cell Information at Cell A

As proposed in the section 2, the NES gNB CU is aware of the proposed and applied “SIB1 mode” of an NES cell. This mode, and any change to this mode is notified by the NES gNB DU to the NES gNB CU over the F1 interface. The current SIB1 transmission mode of NES cell(s) is required to be notified to Cell A to support Case 2 (which was approved as WI in RAN#105).

The NES cell gNB-CU may send the proposed or applied SIB1 mode and any modification to the SIB1 mode to Cell A gNB over Xn. In case the NES cell is under the coverage of multiple Cell As, then this information is transmitted to all the relevant Cell A gNB-CUs. Cell A gNB uses the NES cells’ UL WUS Configuration in its SIB broadcast to the camped UEs.

If there is any change in the UL WUS configuration at the NES Cell, then NES Cell gNB-DU informs the NES Cell gNB-CU of the update over F1 interface. The updated UL WUS configuration is then sent to Cell A gNB-CU over the Xn interface.

We propose an introduction of ES Assistance Information which is transmitted from NES cell gNB-CU to Cell A gNB-CU over the Xn interface which carries the relevant information like the cell id with corresponding proposed/current SIB1 mode, UL WUS Configuration etc

If there is any change in the UL WUS configuration at the NES Cell, then NES Cell gNB-DU informs the NES Cell gNB-CU of the update over F1 interface. The updated UL WUS configuration is then sent to Cell A gNB-CU over the Xn interface.

- What information is required from the NES Cell at Cell A?

Cell A gNB gets information from NES cell gNB which includes the following:

- PCI of the NES cell

- UL WUS configuration of the NES cell

- Proposed “SIB1 mode” of the NES cell

The information from NES cell to Cell A includes PCI, UL WUS configuration and the proposed/current SIB1 mode of the NES cell.

What happen if Cell A “rejects” the transmission of UL WUS configuration of the NES Cell?

In RAN3#125, the following was agreed:

Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

Based on the above agreement, once NES cell proposes to go into an energy saving mode, it sends the relevant NES information to Cell A. If Cell A “confirms” to transmit UL WUS configuration to the UEs camped in Cell A, then NES cell can proceed to go into the energy saving state. However, if Cell A “rejects” to transmit UL WUS configuration to its IDLE/INACTIVE UEs, then NES cell has two options: If there is another Cell A which has “confirmed” to transmit its UL WUS configuration, then NES cell may still proceed to activate its energy saving state. However, if there is no Cell A which has “confirmed”, then NES cell will have to cancel its proposal of activating the energy saving state and continue with the current SIB1 transmission mode. At this point, the NES cell may choose to employ the Rel18 ES feature of changing the periodicity of common channel transmission. This will enable to NES cell to save some energy if not the maximum possible.

If the NES cell proposing to transition to an energy saving mode receives at least one confirmation from a Cell A to transmit its UL WUS configuration to the UE camped on it, then it may proceed to transition to the proposed energy saving mode.

How and when does Cell A transmit the UL WUS of the NES cell to the UEs camped on Cell A?

It has been agreed in RAN2 that Cell A will broadcast the UL WUS Configuration of the NES cell in one of the existing SIBs or a new SIB will be defined for it. We propose that once Cell A receives the NES cell information, it immediately prepares the content of the selected SIB to reflect this information. If the selected SIB is prepared by the DU, the Cell A gNB-CU transmits this information to Cell A gNB-DU. If the content of the selected SIB is prepared by the CU, then CU encodes the SIB with the received NES Cell information.

The NES cell ID list and their corresponding UL-WUS configurations are sent from Cell A gNB-CU to Cell A gNB-DU if the selected SIB for this transmission is prepared by the DU.

# Call flow to support the various SIB1 modes of NES Cell

- NES Cell transitioning to Energy Saving mode

Let’s assume the baseline scenario where NES cell is broadcasting periodic SIB1. If the gNB-DU observes there are no UEs in RRC Connected, RRC Inactive or Idle mode, then it may decide to activate the energy saving mode. This may be done in agreement with NES Cell gNB-CU. There may be other conditions configured by the network operator to activate energy saving. The NES cell gNB-DU then stops all SIB1 transmission, changes the SIB1 mode to reflect this and sends the SIB1 mode and the UL WUS Configuration to the NES gNB-CU over the F1 interface. The NES cell gNB-CU communicates this with Cell A gNB-CU over Xn, which in turn transmits it to Cell A gNB-DU over F1 for broadcasting in Cell A SIB1.

- NES Cell transitioning to OD-SIB1 mode

When conditions for a UE camped on Cell A are favorable, it sends a UL WUS request to the NES cell. The NES cell gNB-DU, on receiving such a request, changes the SIB1 mode to OD-SIB1, and begins to broadcast OD-SIB1. It also informs the NES cell gNB-CU with the SIB1 mode update and the UL WUS configuration over the F1 interface. The NES cell gNB-CU transmits this to Cell A gNB-CU over Xn, which in turn transmits it to Cell A gNB-DU over F1 for broadcasting in Cell A SIB1. The UE, on reading NES Cell SIB1, can perform cell reselection and camp on NES cell.

- NES Cell deactivating Energy Saving mode

When conditions are favorable at the NES cell to deactivate energy saving mode, the NES Cell gNB-DU changes the SIB1 mode to periodic SIB1. The change in SIB1 mode is conveyed from NES cell gNB-DU to NES cell gNB-CU over F1. NES cell transmits SIB1 mode along with the PCI of the NES Cell to Cell A gNB-CU over Xn, which communicates this information to Cell A gNB-DU over F1. The Cell A SIB1 can then be updated to not broadcast any NES cell information. If a UE camped on Cell A is in favorable condition for cell reselection, it reads the NES cell SIB1 and performs cell reselection to camp on the NES cell.

# Conclusion

We have the following observations:

Observation 1	gNB-DU is the network entity responsible for broadcasting SIB1.

We have the following proposals:

Proposal 1	NES cell gNB-DU to store “SIB1 mode” which reflects whether the NES cell is transmitting periodic SIB1 (default), OD-SIB1, or not transmitting any SIB1 (NES mode).

Proposal 2	We propose an introduction of ES Assistance Information which is transmitted from NES cell gNB-DU to NES cell gNB-CU over the F1 interface which carries the relevant information like the cell id with corresponding proposed/current SIB1 mode, UL WUS Configuration etc.

Proposal 3	NES cell gNB DU is the preferred network entity which is best equipped to decide SIB1 mode to be used in the NES cell. This may be done in agreement with NES Cell gNB-CU and a “confirmation” from Cell A.

Proposal 4	We have proposed with examples various triggers from NES cell gNB-DU, NES cell gNB-CU and Cell A to activate the transmission of OD-SIB1 or periodic SIB1 by NES Cell

Proposal 5	We propose an introduction of ES Assistance Information which is transmitted from NES cell gNB-CU to Cell A gNB-CU over the Xn interface which carries the relevant information like the cell id with corresponding proposed/current SIB1 mode, UL WUS Configuration etc

Proposal 6	If there is any change in the UL WUS configuration at the NES Cell, then NES Cell gNB-DU informs the NES Cell gNB-CU of the update over F1 interface. The updated UL WUS configuration is then sent to Cell A gNB-CU over the Xn interface.

Proposal 7	The information from NES cell to Cell A includes PCI, UL WUS configuration and the proposed/current SIB1 mode of the NES cell.

Proposal 8	If the NES cell proposing to transition to an energy saving mode receives at least one confirmation from a Cell A to transmit its UL WUS configuration to the UE camped on it, then it may proceed to transition to the proposed energy saving mode.

Proposal 9	The NES cell ID list and their corresponding UL-WUS configurations are sent from Cell A gNB-CU to Cell A gNB-DU if the selected SIB for this transmission is prepared by the DU.

# Reference

3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.

Chairman’s Notes RAN3 #123-bis

Chairman’s Notes RAN3#124

R3-243868 Summary of offline disc

Chairman’s Notes RAN3#125