3GPP TSG-RAN WG3 Meeting #125bis	R3-245649

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discusson on support of L3 measurement based LTM support over F1

Document for:	Discussions & Approval

- Introduction

At last RAN2 #127 meeting, RAN2 sent a LS R3-245008/ R2-2407606 on L3 measurement based LTM support over F1, as below:

1. Overall Description: In the late phase of Rel-18 LTM definition, it was decided, that LTM can be triggered based on Layer 3 measurements by the UE. Consequently, Layer 3 measurements are delivered via RRC to the CU and the CU needs to request the DU to send MAC CE to trigger LTM execution. During RAN2 discussion, it was recognised, that there is no message over F1 which would be sent from CU to DU requesting to trigger MAC CE for the case that the LTM mobility is based on L3 measurements. It was also recognised that such a message would be needed from Release 18 onwards. 2. Actions: RAN2 kindly asks RAN3 to introduce F1 support for the case where the CU needs to request the DU to send MAC CE to trigger LTM execution based on UE Layer 3 measurements.

Regarding how to support LTM triggering based on L3 measurements, there are two options on the table:

- Option 1: The CU decides the target candidate cell based on L3 measurement results, and inform the selected target candidate cell to the DU via F1 interface;

- Option 2: The CU sends L3 measurement results to the DU via F1 interface, and the DU selects the target candidate cell based on the received L3 measurement results.

This paper addresses to select which option to support L3 measurement based LTM support over F1.

- Discussion

In option 1, the CU can directly decide the target candidate cell according to received L3 measurement results, similar to the legacy L3 handover. And then the CU informs the selected target candidate cell to the DU, to enable the DU generate and send the LTM cell switch MAC CE to the UE. In this way, only one new IE is required to be introduced in an F1AP message (e.g. UE context modification message, CU-DU cell switch notification message, or a new F1AP message), to indicate the target candidate cell ID (e.g. the candidate configuration index, or NR CGI).

In option 2, the L3 measurement results for all related candidate cells need to be transferred to the DU completely. Considering that there is no F1AP message to transfer such L3 measurements from the CU to the DU in the current spec, a new F1AP message shall be required to include the L3 measurement results of candidate cells. The message should include the candidate cell ID, and the corresponding SSB or CSI-RS based L3 measurement results (e.g.  RSRP, RSRQ and SINR measurements). Compared to option 1, larger signalling overhead via F1AP is required in option 2. Besides, the F1AP message carrying L3 measurement results may need to be sent frequently, e.g. every time when the CU receives the L3 measurement report from the UE, considering that L3 measurement results on candidate cells may be triggered at different time point and one report may also include results on partial candidate cells.

Some companies may argue that the DU can obtain more information (e.g. whether the candidate cell has acquired TA) to assit the target candidate cell selection. However, the DU usually decides whether to trigger the early sync procedure and which candidate cell is selected to perform the early sync procedure, based on the received L1 measurements on candidate cells. If there is no L1 measurements reported to the DU, the DU may be unable to select the suitable candidate cells to trigger the early sync procedure. Even if the early TA acquisition has been initiated towards some candidate cells, the CU can also know such information in inter-DU cases, since the acquired TA of candidate cells shall be firstly sent from the target DU to the CU, and then the CU transfers it to the source DU. In this case, the CU can also consider the early TA information of candidate cells when selecting the suitable target candidate cell, in addition to L3 measurement results.

Proposal 1: The CU decides the target candidate cell based on L3 measurement results, and inform the selected target candidate cell to the DU via F1 interface.

Proposal 2: If the CU decides the target candidate cell based on L3 measurement results, it sends LTM Cell Switch Information on L3 included in CU-DU CELL SWITCH NOTIFICATION message.

- Conclusion

After the above analysis, we provide the following proposals.

Proposal 1: The CU decides the target candidate cell based on L3 measurement results, and inform the selected target candidate cell to the DU via F1 interface.

Proposal 2: If the CU decides the target candidate cell based on L3 measurement results, it sends LTM Cell Switch Information on L3 IE included in CU-DU CELL SWITCH NOTIFICATION message.

We also provide the corresponding CR for TS38.473 and TS38.401.