3GPP TSG-RAN WG3 Meeting #125bis	R3-245659

Hefei, China, 14th - 18th Oct 2024

Agenda Item:	9.1

Source:	ZTE Corporation

Title:	Further Discussion on UE Location Information for NB-IoT NTN

Document for:	Discussion

1  Introduction

In previous meetings, RAN3 has discussed the issue of coarse UE location information reporting for IoT NTN, and there has been no consensus so far. However, in last meeting, some companies provided some clarification on the requirement from the operator’s prospective, which should be taken into account for the further discussion[1].

In this contribution, we provide our understanding on this issue based on the operator’s requirement and prepare the corresponding CRs and the draft reply LS to SA2.

2  Discussion

As given in [2], the coarse UE location information from the MME can be used by the eNB to address the Intra-beam carrier allocation, Intra-beam load balancing and Inter-beam load balancing. While, some companies thought this solution may lead to heavy signalling overhead due to the high number of UEs, especially for the periodic reporting.

Since this issue only applies for the IoT UEs which should be assumed with low mobility, the corresponding coarse UE location should be relatively static. Therefore, there is no need to send the coarse UE location information frequently from MME to the eNB. And the coarse UE location information could be sent from MME to the eNB only during the Initial Attach procedure.

SA2 has provided the following coarse location information reporting procedure via NAS in [3], i.e., the UE shall report its Coarse Location Information in the Security Mode Complete message to the MME.

For satellite access over NB-IoT, it the UE indicated support for reporting its Coarse Location Information, the MME may request the UE to send its Coarse Location Information by setting the Coarse Location Information Request in the Security Mode Command message and the UE then reports its Coarse Location Information in the Security Mode Complete message to the MME. To perform UE location verification as described in clause 4.13.4, the MME provides the reported Coarse Location Information to the E-SMLC as described in clause 9.1.17 of TS 23.271 [57].

After receiving the coarse location information from the UE, the MME should send this info to the eNB via the subsequent S1AP INITIAL CONTEXT SETUP REQUEST message.

Proposal: To meet the requirement of load balancing, after receiving the coarse location information for the UE via NAS, the MME should send this info to the eNB via the INITIAL CONTEXT SETUP REQUEST message.

3  Conclusion

Proposal: To meet the requirement of load balancing, after receiving the coarse location information for the UE via NAS, the MME should send this info to the eNB via the INITIAL CONTEXT SETUP REQUEST message.

The corresponding CRs and draft LS to SA2 are given in the Annex.

References

- RAN3#125 Chairman Notes

- R3-244652, Summary of Offline Discussion – Coarse UE Location Information for IoT NTN, Ericsson

- S2-2403493, Mechanisms for UE location reporting for NB-IoT satellite access, Ercisson

Annex

Reply LS to SA2

3GPP TSG-RAN WG3 Meeting #123bis	R3-24xxxx

Changsha, China, 15th – 19th April 2024

Title:	[Draft] Reply LS on UE Location Information for NB-IoT NTN

Response to:	Reply LS on UE Location Information for NB-IoT NTN (S2-2403851)

Release:	Rel-18

Work Item:	IoT_NTN_enh

Source:	ZTE (to be RAN3)

To:	SA2

Cc:	RAN2, CT1, SA1, SA3-LI

Contact Person:

      - Name:	Jiren Han

      - E-mail Address:	han.jiren@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall Description:

RAN3 thanks SA2 for the LS on UE Location Information for NB-IoT NTN.

In the LS, SA2 has the following issue:

SA2 would therefore like to ask whether RAN2 and RAN3 think it may be useful for MME to further signal the coarse location information received from the UE in NAS back to eNB e.g. using one of the existing Location Reporting Procedure in order for the eNB to send updated ULI to CN. SA2 did not reflect this change in the attached SA2 approved TS 23.401 CR and would like to receive feedback from RAN2 and RAN3.

RAN3 would like to provide the following feedback:

To meet the requirement of Intra-beam carrier allocation, Intra-beam load balancing and Inter-beam load balancing, it could be useful for MME to further signal the coarse location received from the UE in NAS back to eNB. And RAN3 has agreed to support the transfer of the coarse UE location information from MME to eNB via S1AP INITIAL CONTEXT SETUP REQUEST message, as given in the attachments.

2. Actions:

To SA2 group.

ACTION: 	RAN3 kindly asks SA2 to take the above into consideration.

3. Date of Next RAN3 Meetings:

TSG RAN WG3 Meeting #126     18th – 22th, Nov. 2024 		 Orlando, US

TSG RAN WG3 Meeting #127     17th – 21th, Feb. 2025 		 Athens, Greece

CR to TS36.300

3GPP TSG-RAN WG3 Meeting #125bis	R3-24xxxx

# Maastricht, NL, 14th - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | N/A | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on mapping of tracking area and fixed geographical area for NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The coarse NB-IoT UE location information from the MME can be used by the eNB to address the Intra-beam carrier allocation, Intra-beam load balancing and Inter-beam load balancing. Since this issue only applies for the IoT UEs which should be assumed with low mobility, the corresponding coarse UE location should be relatively static. Therefore, there is no need to send the coarse UE location information frequently from MME to the eNB. And the coarse UE location information could be sent from MME to the eNB only during the Initial Attach procedure. - From RAN3’s prospective, the transfer of the NB-IoT UE coarse location information from the MME to eNB should be supported. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only the stage 2 description of Coarse UE location reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description to support the transfer of the NB-IoT UE coarse location information from the MME to eNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Intra-beam carrier allocation, Intra-beam load balancing and Inter-beam load balancing cannot be addressed by eNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.21.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

23.21.9	Coarse UE location reporting

Upon network request, after AS security is established in connected mode, BL UEs and UEs in enhanced coverage can report their coarse UE location information (most significant bits of the GNSS coordinates, ensuring an accuracy in the order of 2km) to the eNB if available.

For a RRC_CONNECTED NB-IoT UE, after receiving the coarse UE location via NAS, the MME shall send this coarse UE location information to the eNB, if available.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

CR to TS36.413

3GPP TSG-RAN WG3 Meeting #125bis	R3-24xxxx

# Hefei, China, 14th - 18th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | xxx | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE location information reporting for NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The coarse NB-IoT UE location information from the MME can be used by the eNB to address the Intra-beam carrier allocation, Intra-beam load balancing and Inter-beam load balancing. Since this issue only applies for the IoT UEs which should be assumed with low mobility, the corresponding coarse UE location should be relatively static. Therefore, there is no need to send the coarse UE location information frequently from MME to the eNB. And the coarse UE location information could be sent from MME to the eNB only during the Initial Attach procedure. - From RAN3’s prospective, the transfer of the NB-IoT UE coarse location information from the MME to eNB should be supported over S1 via INITIAL CONTEXT SETUP REQUEST message. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only the transfer of the NB-IoT UE coarse location information from the MME to eNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Coarse Location Information IE in the INITAL CONTEXT SETUP REQUEST message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reporting of coarse location information from MME to eNB is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 8.3.1.2; 9.1.4.1; 9.3.3; 9.3.4; 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[3]	3GPP TS 36.410: "S1 General Aspects and Principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

>>Unchanged part skipped<<

[51]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[52]	3GPP TS 38.401: "NG-RAN; Architecture description".

[x]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.1	Initial Context Setup

8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE Context including E-RAB context, the Security Key, Handover Restriction List, UE Radio capability and UE Security Capabilities etc. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial Context Setup procedure. Successful operation.

In case of the establishment of an E-RAB the EPC must be prepared to receive user data before the INITIAL CONTEXT SETUP RESPONSE message has been received by the MME. If no UE-associated logical S1-connection exists, the UE-associated logical S1-connection shall be established at reception of the INITIAL CONTEXT SETUP REQUEST message.

The INITIAL CONTEXT SETUP REQUEST message shall contain within the E-RAB to be Setup List IE the information required by the eNB to build the new E-RAB configuration consisting of at least one additional E-RAB.

The E-RAB to be Setup Item IE may contain:

-	the NAS-PDU IE,

-	the Correlation ID IE in case of LIPA operation,

-	the SIPTO Correlation ID IE in case of SIPTO@LN operation,

-	the Bearer Type IE.

-	the Security Indication IE

The INITIAL CONTEXT SETUP REQUEST message may contain

-	the Trace Activation IE.

-	the Handover Restriction List IE, which may contain roaming or access restrictions.

-	the UE Radio Capability IE.

-	the Subscriber Profile ID for RAT/Frequency priority IE.

-	the Additional RRM Policy Index IE.

-	the CS Fallback Indicator IE.

-	the SRVCC Operation Possible IE.

-	the CSG Membership Status IE.

-	the Registered LAI IE.

-	the GUMMEI IE, which indicates the MME serving the UE, and shall only be present according to subclauses 4.6.2 and 4.7.6.6 of TS 36.300 [14].

-	the MME UE S1AP ID 2 IE, which indicates the MME UE S1AP ID assigned by the MME, and shall only be present according to subclause 4.6.2 of TS 36.300 [14].

-	the Management Based MDT Allowed IE.

-	the Management Based MDT PLMN List IE.

-	the Additional CS Fallback Indicator IE.

-	the Masked IMEISV IE.

-	the Expected UE Behaviour IE.

-	the ProSe Authorized IE.

-	the UE User Plane CIoT Support Indicator IE.

-	the V2X Services Authorized IE.

-	the UE Sidelink Aggregate Maximum Bit Rate IE.

-	the NR UE Security Capabilities IE.

-	the Aerial UE subscription information IE.

-	the Pending Data Indication IE.

-	the IAB Authorized IE.

-	the NR V2X Services Authorized IE.

-	the NR UE Sidelink Aggregate Maximum Bit Rate IE.

-	the PC5 QoS Parameters IE.

-    the Coarse Location Informaton IE.

The INITIAL CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE, if available in the MME.

If the Correlation ID IE is included in the INITIAL CONTEXT SETUP REQUEST message towards the eNB with L-GW function for LIPA operation, then the eNB shall use this information for LIPA operation for the concerned E-RAB.

If the SIPTO Correlation ID IE is included in the INITIAL CONTEXT SETUP REQUEST message towards the eNB with L-GW function for SIPTO@LN operation, then the eNB shall use this information for SIPTO@LN operation for the concerned E-RAB.

If the Bearer Type IE is included in the INITIAL CONTEXT SETUP REQUEST message and is set to "non IP", then the eNB shall not perform IP header compression for the concerned E-RAB.

If the Ethernet Type IE is included in the INITIAL CONTEXT SETUP REQUEST message and is set to "True", then the eNB shall, if supported, take this into account to perform header compression appropriately for the concerned E-RAB.

If the Security Indication IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, act as defined in the E-RAB Setup procedure for the concerned E-RAB.

If the Masked IMEISV IE is contained in the INITIAL CONTEXT SETUP REQUEST the target eNB shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the Expected UE Behaviour IE is included in the INITIAL CONTEXT SETUP REQUEST message, the eNB shall, if supported, store this information and may use it to determine the RRC connection time.

Upon receipt of the INITIAL CONTEXT SETUP REQUEST message, the eNB shall

-	attempt to execute the requested E-RAB configuration.

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE.

-	pass the value contained in the E-RAB ID IE and the NAS-PDU IE received for the E-RAB for each established Data radio bearer to the radio interface protocol. The eNB shall not send the NAS PDUs associated to the failed Data radio bearers to the UE.

-	store the received Handover Restriction List in the UE context.

-	store the received UE Radio Capability in the UE context.

-	store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [14].

-	if supported, store the received Additional RRM Policy Index IE in the UE context and use it as defined in TS 36.300 [14].

-	store the received SRVCC Operation Possible in the UE context and use it as defined in TS 23.216 [9].

-	store the received UE Security Capabilities in the UE context.

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS 33.401 [15].

-	store the received CSG Membership Status, if supported, in the UE context.

-	store the received Management Based MDT Allowed information, if supported, in the UE context.

-	store the received Management Based MDT PLMN List information, if supported, in the UE context.

-	store the received ProSe Authorization information, if supported, in the UE context.

-	store the received V2X Services Authorization information, if supported, in the UE context.

-	store the received UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for V2X services.

-	store the received IAB Authorization Information, if supported, in the UE context, and use it accordingly for the IAB-MT as specified in TS 38.401 [52].

-	store the received NR V2X Services Authorization information, if supported, in the UE context.

-	store the received NR UE Sidelink Aggregate Maximum Bit Rate, if supported, in the UE context, and use it for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

-	store the received PC5 QoS Parameters, if supported, in the UE context, and use it for the concerned UE’s NR sidelink communication as specified in TS 23.285 [49].

-    store the received Coarse Location Information, if supported, in the UE context.

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.4.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the MME to request the setup of a UE context.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME UE S1AP ID | M |  | 9.2.3.3 |  | YES | reject |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | M |  | 9.2.1.20 |  | YES | reject |
| E-RAB to Be Setup List |  | 1 |  |  | YES | reject |
| >E-RAB to Be Setup Item IEs |  | 1 .. <maxnoofE-RABs> |  |  | EACH | reject |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>E-RAB Level QoS Parameters | M |  | 9.2.1.15 | Includes necessary QoS parameters. | - |  |
| >>Transport Layer Address | M |  | 9.2.2.1 |  | - |  |
| >>GTP-TEID | M |  | 9.2.2.2 |  | - |  |
| >>NAS-PDU | O |  | 9.2.3.5 |  | - |  |
| >>Correlation ID | O |  | 9.2.1.80 |  | YES | ignore |
| >>SIPTO Correlation ID | O |  | Correlation ID 9.2.1.80 |  | YES | ignore |
| >>Bearer Type | O |  | 9.2.1.116 |  | YES | reject |
| >>Ethernet Type | O |  | 9.2.1.147 |  | YES | ignore |
| >>Security Indication | O |  | 9.2.1.163 |  | YES | reject |
| UE Security Capabilities | M |  | 9.2.1.40 |  | YES | reject |
| Security Key | M |  | 9.2.1.41 | The KeNB is provided after the key-generation in the MME, see TS 33.401 [15]. | YES | reject |
| Trace Activation | O |  | 9.2.1.4 |  | YES | ignore |
| Handover Restriction List | O |  | 9.2.1.22 |  | YES | ignore |
| UE Radio Capability | O |  | 9.2.1.27 |  | YES | ignore |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | YES | ignore |
| CS Fallback Indicator | O |  | 9.2.3.21 |  | YES | reject |
| SRVCC Operation Possible | O |  | 9.2.1.58 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.1.73 |  | YES | ignore |
| Registered LAI | O |  | 9.2.3.1 |  | YES | ignore |
| GUMMEI | O |  | 9.2.3.9 | This IE indicates the MME serving the UE. | YES | ignore |
| MME UE S1AP ID 2 | O |  | 9.2.3.3 | This IE indicates the MME UE S1AP ID assigned by the MME. | YES | ignore |
| Management Based MDT Allowed | O |  | 9.2.1.83 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.1.89 |  | YES | ignore |
| Additional CS Fallback Indicator | C-ifCSFBhighpriority |  | 9.2.3.37 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.38 |  | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.1.96 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.1.99 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.2.1.113 |  | YES | ignore |
| V2X Services Authorized | O |  | 9.2.1.120 |  | YES | ignore |
| UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.122 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| NR UE Security Capabilities | O |  | 9.2.1.127 |  | YES | ignore |
| CE-mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Aerial UE subscription information | O |  | 9.2.1.136 |  | YES | ignore |
| Pending Data Indication | O |  | 9.2.3.55 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.1.140 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| IAB Authorized | O |  | 9.2.1.146 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.1.148 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.1.149 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.1.153 |  | YES | reject |
| Coarse Location Information | O |  | OCTECT STRING | Corresponds to the Ellipsoid-Point IE as defined in TS 37.355 [x]. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

CellAccessMode,

Cdma2000HORequiredIndication,

Cdma2000HOStatus,

Cdma2000OneXSRVCCInfo,

Cdma2000OneXRAND,

Cdma2000PDU,

Cdma2000RATType,

Cdma2000SectorID,

>>unchanged part skipped<<

PagingCause,

SecurityIndication,

SecurityResult,

LTE-NTN-TAI-Information,

CoarseLocationInformation

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

FROM S1AP-Containers

id-AssistanceDataForPaging,

id-AerialUEsubscriptionInformation,

id-uEaggregateMaximumBitrate,

id-BearerType,

id-Cause,

id-CellAccessMode,

id-CellIdentifierAndCELevelForCECapableUEs,

id-cdma2000HORequiredIndication,

id-cdma2000HOStatus,

id-cdma2000OneXSRVCCInfo,

id-cdma2000OneXRAND,

id-cdma2000PDU,

id-cdma2000RATType,

id-cdma2000SectorID,

>>unchanged part skipped<<

id-UERadioCapabilityID,

id-UERadioCapability-NR-Format,

id-NotifySourceeNB,

id-eNB-EarlyStatusTransfer-TransparentContainer,

id-WUS-Assistance-Information,

id-NB-IoT-PagingDRX,

id-UERadioCapabilityForPaging-NR-Format,

id-PagingCause,

id-SecurityIndication,

id-SecurityResult,

id-LTE-NTN-TAI-Information,

id-E-RABToBeUpdatedList,

id-E-RABToBeUpdatedItem,

id-CoarseLocationInformation

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID					CRITICALITY reject	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID					CRITICALITY reject	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-uEaggregateMaximumBitrate		CRITICALITY reject	TYPE UEAggregateMaximumBitrate			PRESENCE mandatory}|

{ ID id-E-RABToBeSetupListCtxtSUReq		CRITICALITY reject	TYPE E-RABToBeSetupListCtxtSUReq		PRESENCE mandatory}|

{ ID id-UESecurityCapabilities			CRITICALITY reject	TYPE UESecurityCapabilities				PRESENCE mandatory}|

{ ID id-SecurityKey						CRITICALITY reject	TYPE SecurityKey						PRESENCE mandatory}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE optional}|

{ ID id-HandoverRestrictionList			CRITICALITY ignore	TYPE HandoverRestrictionList			PRESENCE optional}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability					PRESENCE optional}|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	TYPE SubscriberProfileIDforRFP			PRESENCE optional}|

{ ID id-CSFallbackIndicator				CRITICALITY reject	TYPE CSFallbackIndicator				PRESENCE optional}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible				PRESENCE optional}|

{ ID id-CSGMembershipStatus				CRITICALITY ignore	TYPE CSGMembershipStatus				PRESENCE optional}|

{ ID id-RegisteredLAI					CRITICALITY ignore	TYPE LAI								PRESENCE optional}|

{ ID id-GUMMEI-ID						CRITICALITY ignore	TYPE GUMMEI								PRESENCE optional}|

{ ID id-MME-UE-S1AP-ID-2				CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE optional}|

{ ID id-ManagementBasedMDTAllowed		CRITICALITY ignore	TYPE ManagementBasedMDTAllowed			PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList		CRITICALITY ignore	TYPE MDTPLMNList						PRESENCE optional}|

{ ID id-AdditionalCSFallbackIndicator	CRITICALITY ignore	TYPE AdditionalCSFallbackIndicator	PRESENCE conditional}|

-- The above IE shall be present if the CS Fallback Indicator IE is set to the value “CS Fallback High Priority”

{ ID id-Masked-IMEISV					CRITICALITY ignore	TYPE Masked-IMEISV						PRESENCE optional}|

{ ID id-ExpectedUEBehaviour				CRITICALITY ignore	TYPE ExpectedUEBehaviour				PRESENCE optional}|

{ ID id-ProSeAuthorized					CRITICALITY ignore	TYPE ProSeAuthorized					PRESENCE optional}|

{ ID id-UEUserPlaneCIoTSupportIndicator	CRITICALITY ignore	TYPE UEUserPlaneCIoTSupportIndicator	PRESENCE optional}|

{ ID id-V2XServicesAuthorized			CRITICALITY ignore	TYPE V2XServicesAuthorized				PRESENCE optional}|

{ ID id-UESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE UESidelinkAggregateMaximumBitrate	PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-NRUESecurityCapabilities				CRITICALITY ignore	TYPE NRUESecurityCapabilities			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted			PRESENCE optional}|

{ ID id-AerialUEsubscriptionInformation	CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional}|

{ ID id-PendingDataIndication			CRITICALITY ignore	TYPE PendingDataIndication				PRESENCE optional}|

{ ID id-Subscription-Based-UE-DifferentiationInfo		CRITICALITY ignore	TYPE Subscription-Based-UE-DifferentiationInfo		PRESENCE optional}|

{ ID id-AdditionalRRMPriorityIndex		CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex			PRESENCE optional }|

{ ID id-IAB-Authorized					CRITICALITY ignore	TYPE IAB-Authorized						PRESENCE optional}|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate			CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional }|

{ ID id-UERadioCapabilityID							CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional}|

{ ID id-CoarseLocationInformation					CRITICALITY ignore	TYPE CoarseLocationInformation								PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

>>unchanged part skipped<<

CNType ::= ENUMERATED {

fiveGCForbidden,

...,

epc-Forbiddden

}

CoarseLocationInformation ::= OCTET STRING

ConcurrentWarningMessageIndicator ::= ENUMERATED {

true

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

>>unchanged part skipped<<

id-M4ReportAmount									ProtocolIE-ID ::= 346

id-M5ReportAmount									ProtocolIE-ID ::= 347

id-M6ReportAmount									ProtocolIE-ID ::= 348

id-M7ReportAmount									ProtocolIE-ID ::= 349

id-TimeBasedHandoverInformation						ProtocolIE-ID ::= 350

id-Bearers-SubjectToDLDiscarding-Item				ProtocolIE-ID ::= 351

id-Bearers-SubjectToDLDiscardingList				ProtocolIE-ID ::= 352

id-CoarseLocationInformation                        ProtocolIE-ID ::= xxx

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>