3GPP TSG-RAN WG3 Meeting #125-bis				                  R3-245665

Hefei, China, 14th - 18th Oct 2024

Agenda Item:	10.4

Source:	ZTE Corporation

Title: 		(TP to BL CR for TS38.413, TS38.300, TS38.423) Rel-18 leftovers

Document for:	Discussion and Approval

1	Introduction

This paper discusses the Rel-18 leftovers of SON/MDT.

- Discussion

2.1 RACH optimization for SDT

If the conditions for data volume and DL RSRP are satisfied for MO-SDT (the amount of SDT data awaiting transmission is less than or equal to a configured data volume threshold and the DL RSRP is above a configured RSRP threshold), UE could initiate SDT procedure and first check if there are valid CG resources available for SDT. If yes, but RA is initiated for SDT transmission if the conditions for CG-SDT are not satisfied, it is essential that CG-SDT related information is included in RA report to let gNB know why CG-SDT is not satisfied.

Proposal 1:   RAN3 to discuss the necessity of including CG-SDT related information in the RA report if the UE configured with CG-SDT fails to initiate the CG-SDT procedure.

2.2 MHI Enhancement for SCG Deactivation/Activation

RAN3 receives LS[1] from RAN2 in this meeting, see below.

RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state: - RAN2#127: It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state.. For how to collect the time spent information, there are following two options: Option 1: Network based solution: The network collects the information about time spent in the PSCell in activated or deactivated state. Option 2: UE based solution: The UE includes the information about time spent in the PSCell in activated vs deactivated state in MHI (Mobility History Information). RAN2 kindly asks RAN3 to analyse whether Network based solution is sufficient enough for network optimization, or whether a UE based solution is needed.

From RAN3 perspective, the network may use its recorded UE History Information (UHI) and the received Mobility History Information (MHI) from the UE for MRO purposes. However, MHI is transmitted from the UE to the network only when the UE switches from idle or inactive states to a connected state. Therefore,the network needs to record UHI itself and exchange it with the target node during UE mobility in a connected state for MRO. Thus, the network-based solution of enhancing UHI with SCG activation time information is needed. We suggest RAN3 to implement the network-based solution.

Additionally, the network-based solution might miss some of the UE’s history information. If the UE switches to an idle state, the network cannot link the stored UHI for the same UE when it reconnects to the network. Hence, the UE-based solution (MHI based) is also needed.

Proposal 2:  Both the network-based solution(UHI based) and the UE-based solution(MHI based) are needed.

SCG (de)activation is to tune data traffic transmission  to optimize DC configurations to balance the UE battery usage and data traffic efficiency. Although the percentage of SCG activation time can provide insights into UE traffic volume, there is no guarantee that this percentage is appropriately configured by the RAN. Therefore, recording only the percentage of SCG activation time is insufficient, it is also necessary to include UE traffic characteristics (e.g., average throughput ) over SCG together with activation time information. This allows the RAN to better tune UE traffic over SCG resources based on UE history information.

Proposal 3:  RAN3 agrees to implement the network-based solution by enhancing SCG UHI to include SCG activation time and the UE average throughput over SCG.

Based on above discussion,we provide the corresponding TP for network-based solution for TS38.413 and TS38.300 in annex:

Proposal 4: RAN3 agrees the  corresponding TP of network-based solution for TS38.413 and TS38.300 provided in annex.

2.3 MRO for MR-DC SCG failure

At the latest meeting, the X2AP TP in R3-244825 was agreed, and the eNB needs to decode SCGFailureInformationNR and transfer the necessary information to the en-gNB via the new SCG Failure Information Report message. Based on the progress with X2AP, a similar enhancement for XnAP to support (NG)EN-DC SCG failure needs to be introduced.

Proposal 5: Based on the progress with X2AP, a similar enhancement for XnAP needs to be introduced. The (NG)eNB  needs to decode SCGFailureInformationNR and transfer necessary information to the gNB via XnAP SCG Failure Information Report message.  (The  corresponding TP for TS38.423 is provided in annex).

- Reference

[1] R3-245009, LS on MHI enhancement solution for SCG deactivation/activation, CMCC

- Conclusion

Based on the above discussion, we provide the following proposals.

RACH optimization for SDT:

Proposal 1:   RAN3 to discuss the necessity of including CG-SDT related information in the RA report if the UE configured with CG-SDT fails to initiate the CG-SDT procedure.

MHI Enhancement for SCG Deactivation/Activation:

Proposal 2:  Both the network-based solution(UHI based) and the UE-based solution(MHI based) are needed.

Proposal 3:  RAN3 agrees to implement the network-based solution by enhancing SCG UHI to include SCG activation time and the UE average throughput over SCG.

Proposal 4: RAN3 agrees the  corresponding TP of network-based solution for TS38.413 and TS38.300 provided in annex.

Proposal 5: Based on the progress with X2AP, a similar enhancement for XnAP needs to be introduced. The (NG)eNB  needs to decode SCGFailureInformationNR and transfer necessary information to the gNB via XnAP SCG Failure Information Report message.  (The  corresponding TP for TS38.423 is provided in annex).

5 Annex: Text Proposal

- TP to BL CR for TS 38.413 on MHI enhancement on SCG (de)activaon

<<<<<<<<<<<<<<<<<<<< START CHANGE >>>>>>>>>>>>>>>>>>>>

9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |
| Percentage of SCG Activation Time | O |  | INTEGER (0..100) |  |
| Average UE Throughput DL | O |  | INTEGER (0..4,000,000,000,000,…) | This IE indicates the number of bits delivered by NG-RAN in UL over the corresponding PScell. The unit is: bit/s. |
| Average UE Throughput UL | O |  | INTEGER (0..4,000,000,000,000,…) | This IE indicates the number of bits delivered by NG-RAN in DL over the corresponding PScell. The unit is: bit/s. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>

- TP to BL CR for TS 38.300 on MHI enhancement on SCG (de)activaon

<<<<<<<<<<<<<<<<<<<< START CHANGE >>>>>>>>>>>>>>>>>>>>

### 15.5.4	UE History Information from the UE

The source NG-RAN node collects and stores the UE History Information for as long as the UE stays in one of its cells.

The UE may report the UE history information when connecting to a cell of the NG-RAN node, consisting of PCell and PSCell mobility history information, as specified in TS 38.331 [12].

When information needs to be discarded because the list is full, such information will be discarded in order of its position in the list, starting with the oldest cell record. If the list is full, and the UE history information from the UE is available, the UE history information from the UE should also be discarded.

The resulting information is then used in subsequent handover preparations by means of the Handover Preparation procedures over the NG and XN interfaces, which provide the target NG-RAN node with a list of previously visited cells and associated (per-cell) information elements. The Handover Preparation procedures also trigger the target NG-RAN node to start collection and storage of UE history Information and thus to propagate the collected information.

In case of CHO, the target NG-RAN node updates the time UE stayed in cell of the latest PCell entry (i.e. the source cell) when the UE successfully accesses a candidate cell of the target NG-RAN node. The updated value of the time UE stayed in the source cell is equal to the value received from the source NG-RAN node during the Handover Preparation plus the time from receiving the Handover Request message from the source NG-RAN node to receiving the RRC Reconfiguration Complete message from the UE. When the target NG-RAN node receives the SCG UHI from the source NG-RAN node via the Handover Request message, the target NG-RAN node also updates the time UE stayed in cell, percentage of cell activation time, and average UE throughput in cell of the latest PSCell entry (i.e. the source PSCell) as specified in TS 37.340 [21].

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>

- TP to BL CR for TS 38.413 on MR-DC SCG failure

<<<<<<<<<<<<<<<<<<<< START CHANGE >>>>>>>>>>>>>>>>>>>>

9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.103 |  | YES | ignore |
| Time SCG Failure | O |  | INTEGER (0..1023) |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>