3GPP TSG-RAN WG3 Meeting #125bis	R3-245677

Hefei, China, 14th – 18th Oct 2024

Title: 	Response to R3-245065, R3-245319 and R3-245591

Source: 	ZTE Corporation

Agenda item:	8.3

Document for:	Discussion and Decision

- Introduction

In the RAN2 #127, RAN2 agreed to send a reply LS R2-2407816/R3-245012 to RAN3, as below.

R2-2407816 Reply LS on SDT signalling optimization for partial context transfer 1. Overall Description: RAN2 would like to thank RAN3 for the reply LS on SDT signalling optimization for partial context transfer in R2-2404123 (R3-242198). With regards to the network-based solution discussed in RAN3, RAN2 would like to inform RAN3 that: - If RRCSetup is received during SDT, the UE will discard any pending UL data and set up a new RRC Connection. Thus, the pending UL data will be lost. - If RRCSetup is received during SDT, the UE will discard the current INACTIVE AS context and AS security context which requires RRC establishment related signalling to establish AS context and AS security context again In addition, RAN2 also discussed whether there is any issue if MAC is not reset upon receiving RRCSetup after data is exchanged during SDT. RAN2 discussed whether the network can handle this scenario without performing a MAC reset by network implementation or if a MAC reset is needed. Some companies in RAN2 are not sure whether network implementation can handle this without MAC reset. RAN2 would like to add that if a MAC reset is needed, this would be a non-backward compatible change to Rel-17 and is not desirable.

Meanwhile, for this topic, RAN2 also made the following agreement in their Chairman’s note.

Agreements on the LS in addition to what is already agreed in the LS from last meeting

1	Acknowledge that there is an issue for the cases that there is some data in the buffer which causes dy-synch issues.  RAN2 discussed how to solve the issue and identified two possible ways to handle this: potentially a MAC reset may be needed or NW implementation can handle it.   If a MAC reset is needed, this would be a non-backward compatible change (i.e. not desirable), and some companies are not sure whether NW implementation can handle this issue.

This paper further provides our views on these contributions, R3-245065, R3-245319 and R3-245591.

- Discussion

  - Overall procedure for network-based solution

The network-based solution is similar to the existing procedure in TS38.300, as below.

[TS 38.300] The following figure describes the UE triggered transition from RRC_INACTIVE to RRC_CONNECTED in case of UE context retrieval failure: Figure 9.2.2.4.1-2: UE triggered transition from RRC_INACTIVE to RRC_CONNECTED (UE context retrieval failure) 1. The UE resumes from RRC_INACTIVE, providing the I-RNTI, allocated by the last serving gNB. 2. The gNB, if able to resolve the gNB identity contained in the I-RNTI, requests the last serving gNB to provide UE Context data. 3. The last serving gNB cannot retrieve or verify the UE context data. 4. The last serving gNB indicates the failure to the gNB. 5. The gNB performs a fallback to establish a new RRC connection by sending RRCSetup. 6. A new connection is setup as described in clause 9.2.1.3. Figure 9.2.1.3-1: UE triggered transition from RRC_IDLE to RRC_CONNECTED

Based on the submitted contributions, we provide the overall procedure for 38.300 as below.

Figure 1. Overall procedure for Network-based solution

Compared with legacy procedure (i.e., let UE into RRC_inactive), the new procedure (i.e., let UE into RRC connected mode) seems more complicated.

Observation 1: Network-based solution introduces latency due to e.g., AMF involved and security involved.

After new RRC connection is established between UE and Network, the new NG connection between receiving gNB and CN is established. However, for this UE, at the same time, the old NG connection between last serving gNB and CN still exists. Since the old NG connection exists, the CN may continue the data transfer and may also initiate NAS signalling towards the UE via the old gNB. So, from CN’s point of view, for the same UE, we first need to understand whether it is feasible to have two NG connections with different gNbs, or, whether the CN has to release the old NG connection with the last serving gNB.

If it is not allowed to establish two NG connections with the same UE, the last serving gNB shall trigger to release NG connection before it send Xn: end SDT indicator to the receiving gNB. Then the buffered non-SDT type of DL data/signalling will be lost because new NG connection between CN and receiving gNB has not been established yet.

If it is allowed to establish two NG connections with the same UE, the receiving gNB shall send Xn: UE context release message to the last serving gNB then the last serving gNB will trigger NG: RAN initiated UE context release procedure to release the old NG connection.

Or, the CN can also initiate NG: CN initiated UE context release procedure to release the old NG connection, then a new cause value may be introduced due to KPI requirement.

Observation 2: For network-based solution, CN maybe establish two NG connections with two gNBs for the same UE but path switch procedure cannot be used.

Although two NG connection are established at the same time, the data forwarding cannot be supported because non-SDT type DRB/SRB is only established at the receiving gNB. When non-SDT DL data arrival, the DL data is buffered at the source gNB, but the DL data cannot be forwarded to the target gNB, so non-SDT DL data/signalling will be lost. The reason is that there is no data forwarding tunnel between source gNB and target gNB in this network-based solution. It should be noted that arrival of this non-SDT data is the reason for triggering the SDT session end in the first place and loss of this non-SDT data is hence counter-productive. More importantly, the non-SDT data may include NAS signalling which may have other implications from CN perspective. So, before agreeing this solution, we need to understand the impacts to NAS and hence an LS to SA2 should be sent.

Observation 3: Network-based solution will lose both UL/DL data/signalling because data forwarding tunnel cannot be established between last serving gNB and receiving gNB.

Proposal 1: RAN3 discusses whether to send LS to SA2, to ask whether CN can keep two NG connections between different gNBs for the same UE and inform them about potential loss of DL data/signalling in this scenario.

Proposal 2: RAN3 discusses whether loss of DL data can be prevented and if not, whether it is acceptable that the DL data that triggered the end of SDT session is lost in this solution (whilst other solutions do not have this disadvantage).

Proposal 3: RAN3 shall use figure 1 to discuss the feasibility and complexity of the network-based solution.

  - Overall procedure for network-based solution for split gNB

For split gNB, the network-based solution is similar to the existing procedure in TS38.401, as below.

[TS 38.401] Figure 8.6.2-1: RRC inactive to other RRC states transition procedure

Referred to the above existing procedure, for network-based solution, before the step 8, the step 5/6 shall use F1: UE context modification procedure instead of F1: UE context Setup procedure. Addition, the F1: UE context modification procedure shall be enhanced with a new indicator (e.g., Not reset MAC).

So that, in case of split gNB deployment, besides RRCSetup procedure, more E1AP/F1AP procedures are introduced.

Proposal 4: RAN3 shall use figure 2 to discuss the feasibility and complexity of the network-based solution for split gNB

Figure 2. Overall procedure for Network-based solution for split gNB

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: Network-based solution introduces latency due to e.g., AMF involved and security involved.

Observation 2: For network-based solution, CN maybe establish two NG connections with two gNBs for the same UE but path switch procedure cannot be used.

Observation 3: Network-based solution will lose both UL/DL data/signalling because data forwarding tunnel cannot be established between last serving gNB and receiving gNB.

Proposal 1: RAN3 discusses whether to send LS to SA2, to ask whether CN can keep two NG connections between different gNBs for the same UE and inform them about potential loss of DL data/signalling in this scenario.

Proposal 2 RAN3 discusses whether loss of DL data/signalling can be prevented and if not, whether it is acceptable that the DL data that triggered the end of SDT session is lost in this solution (whilst other solutions do not have this disadvantage).

Proposal 3: RAN3 shall use figure 1 to discuss the feasibility and complexity of the network-based solution.

Proposal 4: RAN3 shall use figure 2 to discuss the feasibility and complexity of the network-based solution for split gNB

More detail RAN3 spec impact can be seen in the related discussion paper R3-245046.

Figure 1. Overall procedure for Network-based solution

Figure 2. Overall procedure for Network-based solution for split gNB