3GPP TSG-RAN WG3 Meeting #125-bis	R3-245679

Hefei, P.R. China, October 14th – 18th 2024

Agenda item:	9.2

Source: 	Ericsson

Title: 	Response to R3-245123, R3-245124, R3-245160 and R3-245587

Document for:	Agreement

# Introduction

This paper is a response to R3-245123, R3-245124, R3-245160 and R3-245587. The issue at hand is RRC segmentation of QoE reports for UEs in NR-DC.

# Discussion

For several meetings RAN3 has been discussing how to support MN-SN coordination with respect to QoE report segmentation for a dual-connected UE engaged in QoE measurements. Two options have been considered:

- Option 1, in which the QoE-configuring node, upon requesting from the QoE-non-configuring node to take over the reception of QoE reports, indicates to the QoE-non-configuring node that QoE report segmentation is required.

- Option 2, in which the QoE-non-configuring node, responding to the request from the QoE-configuring node to take over the reception of QoE reports, indicates whether the QoE report segmentation is enabled. Note that this indication is sent in an unsolicited manner, i.e., without the QoE-configuring node explicitly requesting segmentation.

Papers R3-245123, R3-245124, R3-245160 and R3-245587 submitted to the present meeting propose Option 2. The assumption herein is that the QMC coordination request sent from the QoE-configuring node to the non-QoE configuring node, is an implicit request to the non-QoE configuring node to declare whether it enables RRC segmentation of QoE reports.

According to R3-245123, the assumed logic in Option 2 is that of the non-QoE configuring node should always try its best to enable RRC segmentation, independently of whether QoE reports are large or not.

As a summary, based on the above analysis, Option 2 can accommodate all possible cases no matter whether the previous knowledge can be obtained or not. And the logic behind Option 2 is simpler by acknowledging that the non QoE configuring node should try its best to enable the RRC segmentation, no matter whether the QoE reports are in fact segmented or not.

The argumentation in R3-245123 is built on the following two cases and one assumption:

- Case 1: If the QoE configuring node found that the QoE reports were not segmented based on previous knowledge (e.g. initially only a single connectivity is established between the gNB and the UE, and there was an S-based QoE configuration for which the QoE reports were not segmented), then during the RRC segmentation coordination in case of NR-DC, the QoE configuring node can explicitly request the non QoE configuring node NOT to enable the RRC segmentation over the non QoE configuring node. And upon the reception of such request, the non QoE configuring node may not enable the RRC segmentation accordingly.

- Case 2: If the QoE configuring node found that the QoE reports were segmented based on previous knowledge, then during the RRC segmentation coordination in case of NR-DC, the QoE configuring node can explicitly request the non QoE configuring node to enable the RRC segmentation over the non QoE configuring node. And upon the reception of such request, the non QoE configuring node should try its best to enable the RRC segmentation.

- Assuming the QoE configuring node is able to obtain the previous knowledge, for Case 2, if we can acknowledge that enabling the RRC segmentation is always preferred to avoid over-sized QoE reports to be discarded at the UE side, then both Option 1 and Option 2 could work, with no one prevails the other.

There are several issues with the above argumentation and Option 2.

First, it is incorrectly assumed that RRC segmentation is always possible. This assumption does not hold, because some nodes may not support QoE report segmentation, while some nodes may not want to enable it, even if requested by the QoE-configuring node. So, this is not only about whether the QoE-non-configuring node wants to enable segmentation, but also whether it supports it.

The second drawback of Option 2 comes from the fact that only the QoE-configuring node knows whether the QoE report segmentation is required or not. In a scenario where segmentation is needed, the reporting leg switch can be done only if the non-configuring node is willing/capable of configuring segmentation over its SRB. Otherwise, the QoE-configuring node should refrain from reporting leg switch because the reports will be lost. Since, in Option 2, only the QoE-configuring node knows whether the QoE report segmentation is required or not, while the QoE-non-configuring node does not, the following sequence of events can happen:

- The QoE-configuring node realizes that the QoE reports are large enough to require segmentation (note: this can happen after the node receives one or more reports and decides to request reporting leg switch, but it can also happen before reporting has started, e.g., if the node is unable the reception and forwarding of the reports to the MCE).

- The QoE-configuring node sends the QMC coordination request to the QoE-non-configuring node, asking for reporting leg switch. It does not indicate whether segmentation is needed (because Option 2 does not require the QoE-configuring node to explicitly indicate that it is needed).

- The QoE-non-configuring node determines that it has no possibility of, or is not willing to, enable segmentation, and it indicates it in the response. However, this is a blind, unsolicited response because, as per Option 2, the QoE-non-configuring node does not know whether segmentation is required at all.

- Given that the QoE-non-configuring node cannot/will noy provide, or does not support, report segmentation, the QoE-configuring node decides not to change the reporting leg (because, otherwise, the reports would be lost).

- Meanwhile, the QoE-non-configuring node is unaware about how to proceed, where the following ambiguity occurs:

  - The QoE-non-configuring node may assume that the segmentation is not needed, and that the reporting leg switch will take place and it unnecessarily allocates the resources for that, OR

  - The QoE-non-configuring node may assume that the reporting leg switch will not take place, since it indicated that it will not enable segmentation.

- Given the above ambiguity, the QoE-configuring node either:

  - Needs to execute another round of coordination and indicate to the QoE-non-configuring node that reporting leg will not be switched (e.g., the MN sets the QoE Reporting Path Request IE codepoint value to “srb4”).

    - Problems:

      - Reconfiguring a configuration that was never applied is logically wrong and is a violation of the use of the coordination procedure.

      - This reconfiguration requires an additional round of signalling.

  - Alternatively, the QoE-configuring node does nothing to resolve the confusion and ambiguity at the QoE-non-configuring node.

Based on he above we conclude that Option 2 may cause multiple ambiguities at the QoE-non-configuring node.

Observation 1: Option 2 introduces multiple functional ambiguities and unpredictable network behaviour.

Third, according to Option 2, the “segmentation enabled” IE is proposed to be optional, which causes the following ambiguities:

- Optional presence of the “segmentation enabled” IE means that the QoE-non-configuring node can choose by itself whether it should send it or not, without knowing for sure whether it needs to send it – in other words, the QoE-non-configuring node does not know whether the QoE-configuring node requires segmentation at all.

  - If the QoE-non-configuring node decides not to send the IE, because it may believe that segmentation is not needed, the QoE-configuring node does not know which of the following holds:

    - The QoE-non-configuring node supports segmentation but does not think that the QoE-configuring node requires it.

    - QoE-non-configuring node chooses not to enable segmentation.

    - QoE-non-configuring node does not support segmentation.

For this third issue, one workaround could be to make the presence of the “segmentation enabled” IE mandatory. However, this would mean that the IE would need to be included in every QMC coordination response sent by the QoE-non-configuring node, even when this is not needed, and even by the nodes that do not support segmentation. This does not make sense.

Another workaround could be to state in the specifications that the IE is optional, but also specify that a node that supports segmentation shall send it. This de facto means that the specifications would be mandating the behaviour of the sending node.

Observation 2: The ambiguity introduced by the optional presence of the “segmentation enabled” IE in Option 2 cannot be solved in a meaningful way.

From the above analysis we conclude that Option 2 has clear shortcomings and that it should not be supported. Hence, we propose to support Option 1, where the QoE-configuring node explicitly asks the QoE-non-configuring node whether the QoE-non-configuring node is willing to use segmentation for its SRB or not. Note that the issue is not about whether SN supports segmentation for SRB5 or not – in some scenarios, depending on the resource situation, a QoE-non-configuring node that supports segmentation for its SRB may not be willing to use it (e.g., to avoid receiving large QoE reports). As opposed to Option 2, Option 1 does not restrict the node behaviour, and, at the same time, it allows both peer nodes to receive all the necessary information.

Proposal: The QoE-configuring node explicitly inquires the QoE-non-configuring node in the QMC Coordination Request IE whether QoE-non-configuring node is willing to support QoE report segmentation, and the QoE-non-configuring node responds accordingly in the QMC Coordination Response IE.

In the Appendix we provide a CR where the MN (or the SN) explicitly requests the SN (or the MN) whether the SN (or the MN) is willing to support receiving QoE reports on SRB5 (or on SRB4). In the CR, two IEs are added - one in the QMC Coordination Request IE (to request whether RRC segmentation is required by the QoE configuring node), and one in the QMC Coordination Response IE (to indicate whether RRC segmentation is enabled by the QoE-non-configuring node).

# Conclusion

In this paper, the following is observed and proposed:

Observation 1: Option 2 introduces multiple functional ambiguities and unpredictable network behaviour.

Observation 2: The ambiguity introduced by the optional presence of the “segmentation enabled” IE in Option 2 cannot be solved in a meaningful way.

Proposal: The QoE-configuring node explicitly inquires the QoE-non-configuring node in the QMC Coordination Request IE whether QoE-non-configuring node is willing to support QoE report segmentation, and the QoE-non-configuring node responds accordingly in the QMC Coordination Response IE.

# Appendix CR to TS 38.423

3GPP TSG RAN3 125-bis	R3-24xxxx
Hefei, P.R. China, 14th – 18th October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | xxxx | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRC Segmentation allowed for QoE in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-17 specifications support maximum QoE report size of 144 kB. This means that reports larger than 8 kB need to be RRC-segmented before being sent by the UE. Meanwhile, according to current specification, for a UE in NR-DC configured to perform QoE measurements, if the QoE configuring node is the MN, and switch of reporting SRB from SRB4 to SRB5 is needed, the MN does not know whether the SN is willing to use RRC segmentation for SRB5. Similarly, if the QoE configuring node is the SN, it does not know whether the MN is willing to use RRC segmentation for SRB4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRC Segmentation Required IE in QMC Coordination Request IE. Add RRC Segmentation Allowed IE in QMC Coordination Response IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) on the QMC coordination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When QoE measurements are performed by a UE in NR-DC, and the QoE reports are of a size that requires RRC segmentation, either the change of reporting leg is not possible, or the QoE reports cannot be delivered to the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.297, 9.3.3.298, 9.3.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

#### 9.2.3.197	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  | - | - |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  | - | - |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | - | - |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. This IE is allocated by the M-NG-RAN node. | - | - |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. | - | - |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the QoE reports. | - | - |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. | - | - |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node to indicate whether the S-NG RAN node is interested in receiving further RVQoE reports. | - | - |
| >>Further RAN Visible QoE Reporting Path Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node the preferred SRB for receiving further RAN Visible QoE reports. | - | - |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. | - | - |
| >>Available RAN Visible QoE Metrics | O |  | 9.2.3.158 |  | - | - |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, …) | This IE indicates that the configuration has been released by the M-NG-RAN node. | - | - |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  | - | - |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  | - | - |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | - | - |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. | - | - |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the QoE reports. | - | - |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. | - | - |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used by the S-NG-RAN node when the S-NG-RAN node is the RAN Visible QoE configuring node and the M-NG-RAN node provides the bearers for the application session, to request from the M-NG-RAN node to indicate whether the M-NG-RAN node is interested in receiving further RVQoE reports. | - | - |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (true, ...) | This IE is used by the M-NG-RAN node when the M-NG-RAN node is the RAN Visible QoE configuring node and the S-NG-RAN node provides the bearers for the application session, to request from the S-NG-RAN node to indicate whether the S-NG RAN node is interested in receiving further RVQoE reports. | - | - |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration. | - | - |
| >>Available RAN Visible QoE Metrics |  |  | 9.2.3.158 |  | - | - |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, ...) | This IE indicates that the configuration has been released by the S-NG-RAN node. | - | - |
| RRC Segmentation Required | O |  | ENUMERATED (true, …) | This IE is used to request whether the node providing the QMC Coordination Response allows segmentation of MeasurementReportAppLayer RRC messages defined in TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

-------------------------------------------Next change-------------------------------------------

#### 9.2.3.198	QMC Coordination Response

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the M-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  | - | - |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  | - | - |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | - | - |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) | This IE indicates the identity of the application layer measurement configuration, and corresponds to information provided in the MeasConfigAppLayerId IE as defined in TS 38.331 [10]. | - | - |
| >>QoE Configuration Sending Path | O |  | ENUMERATED (mn, sn, …) |  | - | - |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  | - | - |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  | - | - |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the M-NG-RAN node is interested in receiving further RAN Visible QoE reports. | - | - |
| >>Further RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred path for further RAN Visible QoE reporting. | - | - |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. | - | - |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  | - | - |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  | - | - |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are received with the QMC activation request from the management system to identify one PLMN hosting the management system, and QMC ID is a 3-byte Octet String. | - | - |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  | - | - |
| >>RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  | - | - |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE indicates whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. | - | - |
| >>Further RAN Visible QoE Reporting Path Response | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred path for further RAN Visible QoE reporting. | - | - |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. | - | - |
| RRC Segmentation Allowed | O |  | ENUMERATED (allowed, …) | This IE is used to indicate that the node providing the QMC Coordination Response allows segmentation of MeasurementReportAppLayer RRC messages defined in TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

-------------------------------------------Next change-------------------------------------------

### 9.3.5	Information Element definitions

(skip unchanged)

id-MaximumDataBurstVolume,

id-CPAC-Preparation-Type,

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-RRCSegmentationRequired,

id-RRCSegmentationAllowed,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

,

-------------------------------------------Next change-------------------------------------------

QMCCoordinationRequest ::= SEQUENCE {

mN-to-SN-QMCCoordRequestList	MN-to-SN-QMCCoordRequestList	OPTIONAL,

sN-to-MN-QMCCoordRequestList	SN-to-MN-QMCCoordRequestList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationRequest-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-RRCSegmentationRequired	CRITICALITY ignore	EXTENSION RRCSegmentationRequired		PRESENCE optional },

...

}

RRCSegmentationRequired ::= ENUMERATED {true,...}

-------------------------------------------Next change-------------------------------------------

QMCCoordinationResponse ::= SEQUENCE {

mN-to-SN-QMCCoordResponseList	MN-to-SN-QMCCoordResponseList	OPTIONAL,

sN-to-MN-QMCCoordResponseList	SN-to-MN-QMCCoordResponseList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationResponse-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-RRCSegmentationAllowed	CRITICALITY ignore	EXTENSION RRCSegmentationAllowed		PRESENCE optional },

...

}

RRCSegmentationAllowed ::= ENUMERATED {allowed,...}

-------------------------------------------Next change-------------------------------------------

id-MaximumDataBurstVolume																			ProtocolIE-ID ::= 467

id-CPAC-Preparation-Type																			ProtocolIE-ID ::= 468

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-RRCSegmentationRequired																		ProtocolIE-ID ::= xx1

id-RRCSegmentationAllowed																		ProtocolIE-ID ::= xx2

END

-- ASN1STOP

-------------------------------------------End of changes-------------------------------------------