3GPP TSG-RAN WG3#125bis	R3-245680

Hefei, China, 14-18 October 2024

Agenda item:	8.3

Source:	ZTE Corporation

Title:	Response to R3-245511

Document for:	Discussion and Decision

- Introduction

SA5 has replied to the LS [1] from RAN3 regarding the improved KPIs related to end-to-end data volume transfer analytics.

In this paper, we provide responses to the issues raised and propose a path forward for convergence.

- Discussion

In LS [1], SA5 informs RAN3 that the misalignment between TS 28.558 and TS 28.552, as indicated by RAN3, has been addressed in the updated version, and the missing M4 measurement has been added.

SA5 would like to response to the questions with the following:

RAN3 evaluated carefully the two specifications and made the following observations with respect to TS 28.552 and TS 28.558:

- TS 28.558 does not define M4 measurement.

- There seems to be a misalignment between the two specifications with respect to which entity performs the measurements. In particular, RAN3 identified the following cases:

- The “Average delay DL air-interface” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “Average delay DL in gNB-DU” is measured by the gNB-DU in TS 28.552 and by NRCellCU (for non-split and 2-split scenario) and GNBCUUPFunction (for 3-split scenario) in TS 28.558.

- The “UL PDCP SDU Loss Rate” is measured by the GNBCUUPFunction and NRCellCU in TS 28.552 and by GNBCUUPFunction in TS 28.558.

From RAN3 perspective, the above misalignment should be corrected in TS 28.558 based on TS 28.552.

From SA5 perspective, the misalignment between TS28.558 and TS 28.552 has been corrected in TS28.558v18.1.0. The missing M4 measurement has be added into TS28.558 as attached.

Additionally, RAN3 provided observations on the S-TMSI identifier, while SA5 clarified in their TS 28.558 revision that the “Measured UE Identifier” parameter is not applicable to NG-RAN UE-level (MDT) measurements and is only relevant for 5GC UE-level measurements. Therefore, RAN3 needs to consider an approach to address the continuous MDT collection in the “Rel-19 AI/ML for NG-RAN Enhancement” without relying on the S-TMSI.

Observation 1: RAN3 may consider an approach to address the continuous MDT collection in the “Rel-19 AI/ML for NG-RAN Enhancement” without relying on the S-TMSI.

It is worth mentioning that SA5 has explicitly stated that having a (temporary) identifier in the management-based MDT measurement report, indicating that the measurement is from the same source, is beneficial. Moreover, the normative work for continuous MDT collection is already captured in the WI objectives and will be discussed in the proceeding RAN3 meeting.

Observation 2: Continuous MDT collection is already captured in the R19 WI objectives and will be discussed in the proceeding RAN3 meeting, involving how to ensure the received MDT reports are from the same UE.

In R3-245511[2], proponent provides the understanding on the issue and would like to enhance the MDT mechanisms via minimal specification impacts. Then, the question is asked to SA5 whether a “correlation” identifier could be derived from one or more of the following parameters:

-	Trace Record

-	Trace Recording Session Reference

-	Time Stamp

-	CGI of the cell where the measurements were collected

-	RAN UE ID

However, it can bee seen that in the Agenda Item 11.4, a few papers about their own approach to support MDT mechanism are provided. There are various solutions, including network-based solutions and UE-based solutions. During the SI phase, RAN3 did not have enough time to discuss each solution in detail. Therefore, it is recommended to discuss continuous MDT collection in Agenda Item 11.4 first. Once preliminary consensus is reached, an LS can be sent to SA5 for their input, especially since SA5 also recognizes the potential benefits of this approach.

Proposal 1: It is suggested that once RAN3 reaches a preliminary consensus or conclusion on continuous MDT collection, an LS can be sent to SA5 to provide clarification or seek their inputs.

3	Conclusion

In this paper, we provided responses to R3-245511. Following is our proposals and observations:

Observation 1: RAN3 may consider an approach to address the continuous MDT collection in the “Rel-19 AI/ML for NG-RAN Enhancement” without relying on the S-TMSI.

Observation 2: Continuous MDT collection is already captured in the R19 WI objectives and will be discussed in the proceeding RAN3 meeting, involving how to ensure the received MDT reports are from the same UE.

Proposal 1: It is suggested that once RAN3 reaches a preliminary consensus or conclusion on continuous MDT collection, an LS can be sent to SA5 to provide clarification or seek their inputs.

References

- R3-245021, Reply LS on improved KPIs involving end-to-end data volume transfer time analytics, SA5

- R3-245511, Respond to SA5 Reply LS on improved KPIs, Ericsson