3GPP TSG-RAN WG3 Meeting #125-bis	R3-245681

# Hefei, China, 14th – 18th October 2024

Agenda Item:	31.2

Source:	Ericsson

Title:	Response to R3-245375

Document for:	Discussion and Decision

# 1	Introduction

In R3-245375, the proposal is to allow the CU-CP to deactivate activity monitoring over E1, for e.g. FWA use-cases. This contribution discusses the use-case itself, and the possible solutions if RAN3 agrees to support it as new feature for Rel-19.

# 2	Discussion

According to stage-2, and as described in the CR cover page, inactivity monitoring shall be performed by the CU-UP:

“The node hosting user plane part of NR PDCP (e.g. gNB-CU, gNB-CU-UP, and for EN-DC, MeNB or SgNB depending on the bearer split) shall perform user inactivity monitoring and further informs its inactivity or (re)activation to the node having C-plane connection towards the core network (e.g. over E1, X2).”.

- To justify the use-case (i.e. deactivating activity monitoring over E1), the cover page states that the CU-CP will change the UE RRC state (e.g. send it to RRC_INACTIVE) when receiving the BEARER CONTEXT INACTIVITY NOTIFICATION message:

- Hence, at maximum 7200 seconds of inactivity the CU-UP will report inactivity to the CU-CP, and subsequently an RRC state change is triggered.

- But this is not correct. The CU-CP is free to ignore the BEARER CONTEXT INACTIVITY NOTIFICATION message and keep the UE in RRC_CONNECTED. The behavior of the CU-CP when receiving the inactivity notification from CU-UP is up to implementation.

- Observation 1: Behavior of the CU-CP when receiving the inactivity notification from CU-UP is up to implementation.

- Therefore, one implementation could set all inactivity timers for FWA UEs to 7200s and ignore the inactivity notification that may be sent by the CU-UP. Which means that the remaining point to discuss is related to the signaling load over E1: Does signaling load over E1 need to be optimized in case inactivity monitoring is not needed for some UEs?

- And maybe one last thing to consider before RAN3 takes a decision: the possibility to deactivate inactivity monitoring in CU-UP was already proposed and discussed during RAN3#104 (see R3-194494) and the following meetings.

- Proposal 1: RAN3 to discuss if signaling load over E1 needs to be optimized in case inactivity monitoring is not needed for some UEs.

If RAN3 decides to allow the CU-CP to deactivate inactivity monitoring over E1, there are several technical solutions that may be discussed. The first one is proposed in R3-245375 and is to extend the range of the inactivity timer by including the value “0”. However, this is not backward compatible.

- Observation 2: Solution proposed in R3-245375 is non backward compatible.

- One way to make this solution backward compatible would be to add a new integer value, as described below:

Inactivity-Timer	::=	INTEGER (1..7200, ..., 0)

- But using the value “0” may be misunderstood in that case. In some other features enabled by a timer, “0” is used by the requesting node to ask the sending node to send a report right away (i.e. after 0s).

- Another solution, without ASN.1 impact, would be to only remove the semantics descriptions of the different Inactivity Timer IEs. In that case, if none of the inactivity timers are sent by the CU-CP, which is already possible as they are all optional IEs, the inactivity monitoring could be interpreted by the CU-UP as deactivated. But this solution may not be functionally backward compatible.

- Another solution would be to add a new codepoint to the Activity Notification Level IE, as described below:

#### 9.3.1.67	Activity Notification Level

This IE contains information on which level activity notification shall be performed..

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activity Notification Level | M |  | ENUMERATED (DRB, PDU Session, UE, …, Deactivated) |  |

- But this may also not be functionally backward compatible, because a node not implementing the new codepoint may not know what activity notification level to apply.

- Therefore, the “most backward compatible solution” would be to add a new optional IE to BEARER CONTEXT SETUP REQUEST message, which will indicate to the CU-UP to disable inactivity notification for that Bearer Context and to ignore the Activity Notification Level IE. If the receiver does not implement the new IE, it will just perform inactivity monitoring as received in the Activity Notification Level IE.

Proposal 2: If the use-case described in R3-245375 is agreed by RAN3, add a new optional IE to BEARER CONTEXT SETUP REQUEST message to deactivate inactivity monitoring in CU-UP.

In addition, RAN3 needs to discuss if inactivity monitoring can be deactivated by using the Bearer Context Modification procedure.

Proposal 3: If the use-case described in R3-245375 is agreed by RAN3, RAN3 to discuss if inactivity monitoring can be deactivated by using the Bearer Context Modification procedure.

Also, a stage-2 CR (for TS 38.401) will also be needed to clarify that the inactivity notification may be disabled in the CU-UP.

Observation 3: If the use-case described in R3-245375 is agreed by RAN3, a stage-2 CR to TS 38.401 is also needed.

# 3	Conclusion

The use-case and solution proposed in R3-245375 were discussed and the following observations and proposals were made:

- Observation 1: Behavior of the CU-CP when receiving the inactivity notification from CU-UP is up to implementation.

- Proposal 1: RAN3 to discuss if signaling load over E1 needs to be optimized in case inactivity monitoring is not needed for some UEs.

- Observation 2: Solution proposed in R3-245375 is non backward compatible.

Proposal 2: If the use-case described in R3-245375 is agreed by RAN3, add a new optional IE to BEARER CONTEXT SETUP REQUEST message to deactivate inactivity monitoring in CU-UP.

Proposal 3: If the use-case described in R3-245375 is agreed by RAN3, RAN3 to discuss if inactivity monitoring can be deactivated by using the Bearer Context Modification procedure.

Observation 3: If the use-case described in R3-245375 is agreed by RAN3, a stage-2 CR to TS 38.401 is also needed.