3GPP TSG-RAN WG3#125bis	R3-245686

Hefei, China, 14-18 October 2024

Agenda Item:	9.1

Source:	Qualcomm

Title:	Summary of Discussion on NBIoT UE location

Document for:	Discussions & Approval

# Introduction

MME may send the coarse location back to the eNB per-UE and only once after UE attach by Initial UE Context Setup Request message?

FFS on other critical issues if any

CB: # 4_NBIoTNTN

- Discuss the open issues above

- Other critical corrections if agreeable

(moderator - QC)

Summary of offline disc R3-245686

# For Chairman’s notes

<TBD>

Summary for Chair Notes:

Proposal 1: Based on eNB request, MME will provide Coarse UE Location Information to the eNB for a given UE. Agree stage-2, stage-3 CRs and reply LS to SA2.

R3-245797

R3-245774

R3-245793

# 3	Discussion

## 3.1 UE Location Information from MME to eNB

Based on online and further offline discussions with multiple companies, moderator proposes following solution as compromise:

Step 1 : eNB will send 2 bit indication to MME.

- Bit 1 : eNB  requests MME (if available at MME) to provide UE coarse location one time upon receiving request from eNB.

- Bit 2 : eNB  requests MME to provide any subsequent updated UE Coarse Location Information to eNB when available at MME.

Step 2: when MME has UE coarse location information available, based on eNB requested information in step, MME will send UE coarse location information to eNB.

Question 1: Do companies agree with above compromised solution? Please provide any additional comments if any additional enhancements needed.

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes | As compromise, we are OK with above approach to make progress using 2 bit approach. |
| Ericsson | Yes but | So far, the use case requested by Inmarsat was for intra-beam carrier allocation, inter-beam load balancing and inter-beam load balancing (see R3-244652); at a minimum, this can be supported at UE attach (i.e.: request IE from eNB included in INITIAL UE MESSAGE message; coarse UE location IE included in INITIAL CONTEXT SETUP REQUEST, CONNECTION ESTABLISHMENT INDICATION and/or DOWNLINK NAS TRANSPORT messages). We should first capture the above, in an agreeable form, as a starting point. If the “when available at MME” use case is justified in the eNB, and provided that the eNB may revert its decision and signal it to MME at any time for the same UE, the above might be an acceptable compromise. |
| Nokia | See comments | First, there is No signaling load issue since the related S1AP message (e.g. INITIAL CONTEXT SETUP REQUEST message) is always sent before this topic. The only new requirement now is this message will include the UE coarse location. Adding a new IE to an existing S1AP message does Not cause signaling load issue. If companies insist adding a new IE to an existing message cause signaling load issue, we can accept a compromise including a package of Stage-2&Stage-3 TPs. - Stage-2 CR: based on R3-245429 (removing the OAM part due to Bit 1). The eNB can use the coarse location for ULI, and use it when a next existing S1AP message includes ULI. - Stage-3 CR: based on R3-245484 (updating the TP to “the eNB may use it for radio resource management for the UE”) Bit 2 may be not necessary. Bit 2 assumes the UE is RRC CONNECTED and may move a long distance (e.g. from cell edge to cell center or vice versa). This is very rare for IoT UE. The IoT UE is not supposed to be in RRC CONNECTED for long period. The IoT UE is usually connected to send/receive a few packets, then back to IDLE. In case the UE moves far away, the next UE coarse location can be reported when it is connected next time. When it is connected again, it usually performs TAU or SR procedure. That is why SA2 agreed the MME request coarse location only in Attach, TAU, or Service Request procedure. |
| Huawei | Yes | As compromise Bit 2: solution is acceptable. We could also go for Bit 1 solution with extendable code point. The problem is not only a matter of overload, if new procedure, this information is useless for many UE not subject to balancing. The question from SA2 was is it beneficial for the eNB to received this information, then it makes sense to have a request from the eNB to request this information when needed, and not be forced to received it and decode it … |
| NEC | No | When the MME transmits the Coarse Location Information to gNB based on the implementation, it is too complex to introduce Bit 1 and Bit 2. We do not see the benefit of the request bit(s), over-designed. |
| CATT | See comments | Bascially we think it maybe benefit with this requested based solution (only one indicator), this solution given more control to the eNB. FFS on signalling designation. |
| ZTE | Maybe not | We also have the concern whether it can be solved by MME implementation without the request bit? To avoid the over-design, shall we ask SA2 for clarifcation on whether it can be solved by MME implementation in the LS. Regarding the Stage 2, maybe a more simple way is just to remove “from the UE” in the original text. |
| Ericsson | Yes but (further context) | To ZTE, NEC: without the possibility to signal a request from eNB, hence leaving everything to MME configuration, there is no granularity. Either the MME will signal this for all UEs, or for none. This is not optimal and will lead to a waste of signaling (in Rel-19 even over the Earth-space link). |
| TCL | Yes | As compromise, we are OK with using 2 bit approach. |

Question 2: If answer to the Question 1 is “Yes”, can we agree to use the following messages for one time transfer of UE location?

- eNB requests for coarse UE location in S1AP Initial UE message (using bit 1 as proposed above)

- MME sends coarse UE location in S1AP INITIAL Context Setup Request message

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes |  |
| Ericsson | Yes but | To support CP CIoT, MME may need to send coarse UE location also in DOWNLINK NAS TRANSPORT and CONNECTION ESTABLISHMENT INDICATION messages. |
| Nokia | See comments | Refer to our comments on Q1. we can accept it as a compromise including Stage-2/3 TPs. |
| Huawei | Yes | Pigging backing the information on existing procedure is better than initiate dedicated message. |
| CATT |  | Compared with the size of signalling, we more concerned about the number of signalling interaction between eNB and MME, considering when eNB request the loactaion information, the MME maynot have such informations, then how to deal with the previous request should be further discussed, one thing we think should be avoided is MME send a DLmesaage to the eNB only for sending the corase location information. Thus, for us, initial context setup and CONNECTION ESTABLISHMENT INDICATION are ok, DOWNLINK NAS TRANSPORT should be treat carefull. |

Question 3: If answer to Question 1 is “Yes”, do companies agree to use the following S1AP messages for subsequent transfer of updated UE location based on eNB request? If any other message please mention it in the comments section.

- eNB requests for coarse UE location in S1AP Initial UE message (using bit 2 as proposed above)

- MME sends coarse UE location in S1AUE Context Modification Request message

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes |  |
| Ericsson | Yes but | We may also need to add a 3rd codepoint (or “bit”) to the request IE to signal “no more reporting” for this UE. |
| Nokia | See comments | Refer to our comments on Q1, it is not essential. |
| Huawei | Yes | Same view, as Ericsson, the MME should be aware of a STOP, in case this information is stored, the gNB may back to not loaded mode and stop to received decode this information. |
| CATT | See comment | FFS, may lead to unnessary signalling load, hard to control compapred with initial UE context setup |
| TCL | Yes |  |

Question 4: If any other alternative S1AP messages are needed, please provide details in comments .

| Company | Comments |
| --- | --- |

Based on further offline discussion with all companies: Majority companies are OK for eNB to request Coause UE Location Information from MME per S1-AP active session. i.e., MME will provide Coarse UE Location information to eNB once per S1-AP session per UE.

But 2 companies prefer MME to send Coarse UE Location information eNB without eNB request.

Proposal 1: Based on eNB request, MME will provide Coarse UE Location Information to the eNB for a given UE. Agree stage-2, stage-3 CRs and reply LS to SA2.

## 3.2 Other Corrections

Question 5: As per R3-245339 and R3-245485, Nokia, E/// and HW proposes to modify text in TS 36.300 on Mapped Cell ID as below –

### 23.21.6 Signalling The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported, except the following cases: - For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The mapping between Mapped Cell ID(s) and geographical area(s) is configured in the RAN and Core Network. NOTE 1: A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions).

Are companies ok with the changes proposed in the draftCR in R3-245339 and R3-245485? Please comment.

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | See Comments | Prefer to align 36.300 with similar text in 38.300. The text rewording can be first discussed in 38.300 NR NTN. |
| Ericsson | Yes | To QC: the corresponding CRs to TS 38.300 (R3-245357/8) have been submitted to this meeting in AI 9.2. Indeed, they should be discussed together here as a “package”. The rationale behind the proposed changes is that so far all use cases discussed justify using the mapped cell ID. Therefore, following existing RAN3 best practice, we propose to simply remove the list and simply mention any possible exceptions, as needed. This avoids the need to maintain or grow a list of use cases and results in a simpler specification text. |
| Nokia | Yes |  |
| Huawei | Yes | It is indeed a package with NR, see E/// comments. |
| ZTE | See comments | If we want to apply this new structure, it is better to change it in R19 NR NTN WI, if companies are all fine with it in R19, then we can correct it in R17 and R18 accordingly. |
| Ericsson | Yes (further context) | To ZTE: if agreed, it will also propagate to the Rel-19 BL CR. No problem here. |

This topic will be discussed in other CB.

Question 6: As per R3-245482, E///, Thales and HW proposes to add the below text in TS 36.305 as below –

### 5.3.3 Evolved Serving Mobile Location Centre (E-SMLC) The E-SMLC manages the support of different location services for target UEs, including positioning of UEs and delivery of assistance data to UEs. The E-SMLC may interact with the serving eNode B for a target UE in order to obtain position measurements for the UE, including uplink measurements made by the eNode B and downlink measurements made by the UE that were provided to the eNode B as part of other functions such as for support of handover. The E-SMLC may also interact with the serving eNode B to indicate to the serving eNode B the need to direct the UE to transmit SRS (see 5.2.2) signals to enable the uplink positioning method and to acquire the target UE configuration data needed by the LMUs to calculate the timing of these signals. The E-SMLC will select a set of LMUs to be used for the UTDOA positioning. The E-SMLC interacts with the selected LMUs to request timing measurements. The E-SMLC may interact with a target UE in order to deliver assistance data if requested for a particular location service, or to obtain a location estimate if that was requested. The E-SMLC may interact with multiple eNode B's to provide location assistance data information for broadcasting. The assistance data information for broadcast may optionally be segmented and/or ciphered by the E-SMLC. The E-SMLC may also interact with MMEs to provide ciphering key data information to the MME as described in greater detail in TS 23.271 [2]. For positioning of a target UE, the E-SMLC decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, and eNode B positioning capabilities. The E-SMLC then invokes these positioning methods in the UE and/or serving eNode B. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The E-SMLC may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined. For NTN, the E-SMLC is configured by the OAM with satellite related information (described in TS 36.300 [x]).

Do companies think draftCR R3-245482 is needed? If yes please provide reasoning.

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC |  | It is unclear which feature or correction this CR is addressing. Needs further discussion. |
| Ericsson | Yes | Given that the support for location verification using the EPC-NI-LR LCS procedure related to coarse location is not precluded, the E-SMLC needs to be aware of satellite-related information. Otherwise, it’s unclear how the E-SMLC gets this information. This aligns with the corresponding change we made for NR NTN in TS 38.305. |
| Nokia |  | Why? Based on WIDs, location verification is only performed for NR NTN, but not for IoT NTN. |
| Huawei | Yes | It is an alignment … |
| ZTE |  | We has the same concern with Nokia. |
| Ericsson | Yes (further context) | To Nokia, ZTE: if LCS is triggered toward the E-SMLC, the E-SMLC needs to be aware that this is a satellite deployment. The easiest thing to do is to state that the same OAM info is also applied to the E-SMLC (like for NR NTN). |
| TCL |  | Same conern with ZTE and Nokia |

Summary:

4 companies have concerns about this CR, moderator suggest proponents continue discussing with other companies and not to agree in this meeting.