3GPP TSG-RAN WG3 Meeting #125bis	R3-245690

# Hefei, China, October 14 – 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1467 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TRP Geographical coordinates [PosLocalCoords] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Inc., CATT, Xiaomi, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 agreed in RAN2#126 a CR R2-2405861 correcting Local Cartesian Coordinates in LPP. One change that can affect RAN3 is that RAN2 have specified a new element in the Relative Cartesian coordinates for local origin representation that indicates the horizontal axes orientation information for the reference point: Extract from R2-2405861 starts here: ======================================= NOTE 5: The locations may be provided in either geodetic coordinates (RelativeLocation) or local Cartesian coordinates (RelativeCartesianLocation), but not both. Relative Cartesian coordinates are provided with respect to the horizAxesOrientation of the local origin defined by the referencePoint field. ======================================= End of extract. Currently, in F1AP, the Relative Cartesian coordinates are defined in section 9.3.1.184 as the TRP Position Relative Cartesian IE, that refers to Relative Cartesian Location IE on 9.3.1.187 and is associated to a Reference Point IE defined in 9.3.1.188. To align with LPP and CT4 specification, it is suggested to introduce the horizontal axes orientation information and Reference Point High Accuracy Access Position to the Reference Point IE, when the ReferencePoint in the Reference Point IE CHOICE is set to Coordinate ID . However, due to NBC issue, it is not possible to add the IEs directly in 9.3.1.188 in the SEQUENCE (it is not extensible in ASN.1). Instead a BC solution that does not impact legacy functionality is to introduce an additional LocalOrigin IE as new CHOICE in the Reference point IE (9.3.1.188) that includes the Coordinate ID, Reference Point High Accuracy Access Position and horizontal axes orientation information. The horizontal axes orientation information is added as mandatory information in the Local Origin IE to avoid IOT issues (i.e., sending the coordinate ID in different ways). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new LocalOrigin IE CHOICE in the Reference Point IE (9.3.1.188), that includes Coordinate IE, Reference Point High Accuracy Access Position and Horizontal Axes Orientation encoded as in LPP, i.e., INTEGER(0..3599) Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR does not have impact from protocol point of view. This CR may have an impact from functional point of view because it impacts only the Geographical coordinates IE . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with LPP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.188, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.455 CR 0164 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 : resubmission to RAN3#125bis and addition of co-signers Rev2: putting mandatory IEs lines above optional ones in tabulars and ASN.1 |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 9.3.1.188	Reference Point

This information element provides a reference point location information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE ReferencePoint | M |  |  | Reference point to which relative location information is related to | - |  |
| >Coordinate ID |  |  |  |  |  |  |
| >>Coordinate ID | M |  | INTEGER(0.. 29-1,..) | Referential ID mapped via OAM | - |  |
| >Reference Point Coordinates |  |  |  |  |  |  |
| >>Reference Point Position | M |  | NG-RAN Access Point Position 9.3.1.174 |  | - |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |  |  |
| >>Reference Point High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.3.1.190 |  | - |  |
| >LocalOrigin |  |  |  |  | YES | ignore |
| >>Coordinate ID | M |  | INTEGER(0.. 29-1,..) | Referential ID mapped via OAM | - |  |
| >>Horizontal Axes Orientation | M |  | INTEGER (0..3599) | Indicates the local coordinate system horizontal orientation angle clockwise from northing. Scale factor 0.1 degrees. | - |  |
| >>Reference Point High Accuracy Access Position | O |  | NG-RAN High Accuracy Access Point Position 9.3.1.190 |  | - |  |

Next change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Skipped text unchanged

id-XR-Bcast-Information,

id-MaxDataBurstVolume,

id-BarringExemptionforEmerCallInfo,

id-SIB17bis-message,

id-ReportingIntervalIMs,

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

Next change

ReferencePoint ::= CHOICE {

coordinateID					CoordinateID,

referencePointCoordinate		AccessPointPosition,

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition,

choice-Extension				ProtocolIE-SingleContainer { { ReferencePoint-ExtIEs} }

}

ReferencePoint-ExtIEs F1AP-PROTOCOL-IES ::= {

{ID id-LocalOrigin	CRITICALITY ignore TYPE LocalOrigin	PRESENCE mandatory},

...

}

LocalOrigin	::= SEQUENCE {

relativeCoordinateID			CoordinateID,

horizontalAxesOrientation		INTEGER (0..3599),

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { LocalOrigin-ExtIEs} } OPTIONAL,

...

}

id-LocalOrigin-Id-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Next change

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Skipped text unchanged

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= xxx

END

-- ASN1STOP