3GPP TSG-RAN WG3 Meeting #125-bis	R3-245508

Hefei, P.R.C, 14th Oct – 18th Oct, 2024

Agenda Item:	11.4

Source:	Ericsson (Moderator)

Title:	SoD on CB: # AIRAN5_ContinousMDT

Document for:	Approval

# 1	Introduction

This SoD is to discuss the following CB:

CB: # AIRAN5_ContinousMDT

- Check the principle above

- LS to SA5?

(moderator - E///)

Summary of offline disc R3-245719

# Summary for chairman notes

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Further discussions are needed on how to define a temporary identifier for Continuous MDT

Further discussions are needed on the solution for measurement correlation in Continuous MDT

To be discussed online:

Solutions for Continuous MDT shall not incur in UE impacts

# 2	Discussion

## 2.1 	Continuous MDT Initiation

The following sentence was captured for discussion during online discussions:

RAN3 assumes that the OAM triggers the Management Based MDT session for Continuous MDT towards RAN.

As per legacy Management Based MDT functionalities, the OAM signals an MDT configuration towards the NG-RAN. The NG-RAN receives such Management Based MDT configuration and, based on such configuration as well as on Management Based MDT PLMN List, it selects UEs for management based MDT measurement collection.

Given that Continuous MDT is based on Management Based MDT, a similar process is foreseen, where the OAM signals to the RAN a Continuous MDT Configuration and where the RAN selects UEs for Continuous MDT measurement collection.

It is proposed to turn the following sentence into an agreement:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

## 2.2 	Correlation of Continuous MDT measurements

After initiation of the Continuous MDT session, the RAN will receive measurements from the UE and it will generate Trace Records towards the TCE. The question is how to enable the TCE to identify Trace Records generated by the same UE.

Following feedback form SA5 in their reply LS in R3-243941, the identifier that should be used to correlate Continuous MDT reports should be 
“a (temporary) identifier in the management-based MDT measurement report. This identity shall be unique within the lifetime of the measurement session.”

Three approaches have been identified:

- The OAM provides to the RAN a specific Trace Reference ID that the RAN can use to generate the Trace Reference to include in the Trace Record Header. The RAN selects a TRSR based on the received TR. By reading the parameters in the Trace Record Header, the TCE can deduce that Trace Records are associated to the same UE.

  - Example: The RAN includes in the Trace Record Header the Trace Reference ID received from the OAM, which indicates that the Trace Record belongs to a continuous MDT process. The RAN generates a Trace Recording Session Reference that includes parts (e.g. a prefix) of the Trace Reference ID. This is also included in the Trace Record Header

    - When the UE moves to Idle, the UE is configured with Logged MDT and the UE is provided with the Trace Reference ID and the Trace Recording Session Reference ID.

    - When the UE moves to RRC_Connected, the UE reports MDT Logs, including the TR and TRSR. The new serving RAN reads the TR and TRSR, understands that the UE is a “continuous MDT UE” (from the reported TR) and it configures the UE with continuous MDT again.

    - The new serving RAN will select for the UE a “Continuous MDT Trace Reference ID” and it will assign a TRSR with the same “prefix” as the previous TRSR. New Trace Records generated for this UE and reported to the TCE will contain the TRSR with the “prefix” selected by the previous gNB. This allows the TCE to collate MDT Trace Records from the same UE.

    - If the UE moves to a different gNB in RRC_Connected, the TRSR prefix is signalled to the target gNB. This indicates that the UE is a continuous MDT UE, i.e. it should be selected for continuous MDT, and it will indicate to the target gNB which TRSR prefix to select.

- A new job type is introduced for Continuous MDT, including both Immediate and Logged MDT processes. The OAM provides to the RAN a Trace Reference ID for the new Continuous MDT job. The RAN includes the TR and a TRSR for all Trace Records generated for the new Continuous MDT session. By reading the parameters in the Trace Record Header, the TCE can deduce that Trace Records are associated to the same UE.

  - The RAN signals the TR and TRSR to the AMF

  - If the UE moves to RRC_Idle and back to RRC_Connected, the AMF signals to the new serving gNB the parameters stored for the UE. Such parameters are included in the Trace Record Header. This allows the TCE to identify Trace Records from the same UE.

- The OAM initiates Continuous MDT towards the RAN. The AMF creates an identifier, e.g. generated when the UE registers to the network and UE context is created in the Core Network and maintained in the Core Network for the duration of the UE context.

  - If the UE moves to RRC Idle and back to RRC_Connected, this identifier is signalled from the AMF to the RAN.

  - The identifier is included in the Trace Records signalled to the TCE (confirm that this is correct)

It is proposed to discussed solutions 1, 2 and 3, and to converge on the following:

- To select one or more of such approaches

- To formulate an LS towards SA5 to ask them whether the selected approaches are feasible and how can SA5 support them.

QC: Solution 2 and 3 colve both problems of correlation and continuity of measurements

HW: we are not trying to identify the UE but only the reports coming from the same UE

Samsung: agree that the aim is to identify MDT reports from the same UE and not the UE itself

Further discussions are needed on how to define a temporary identifier for Continuous MDT

Further discussions are needed on the solution for measurement correlation in Continuous MDT

An LS to SA5 to be sent once more agreements on measurement correlation are taken.

Temporary identifier definition:

a temporary identifier that can be included in the management-based MDT measurement report. This identity shall be unique within the lifetime of the continuous MDT measurement session. This identity shall not reveal the UE identity