3GPP TSG-RAN WG3 Meeting #125bis	R3-245725

Hefei, China, 14th – 18th Oct 2024

Agenda Item:	16.4

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # AIoT3_Location

Document for:	Approval

# Introduction

- CB: # AIoT3_Location

- - Discuss the open issues above

- - Cleanup the FFS for AIoT location if agreeable

- - Capture agreements if any

- (moderator - Nok)

Summary of offline disc R3-245725

# For the Chairman’s Notes

General

For AIoT device locating, agree that information enabling the AIoT CN to determine the location of the reader to which the AIoT device responded to an Inventory Request is reported in the Inventory Report to the AIoT CN.

FFS whether report to the AIoT CN in further other messages.

Topology 1

Agree that reporting the AIoT RAN node ID to AIoCN is not good enough to know the AIoT device location at reader ID granularity.

Agree that the AIoT RAN node reports an identifier of the reader in the Inventory Report message to the AIoT CN and the AIoT CN has determined the location of the identified reader.

Select between one of these two solutions on how the AIoT CN has determined the location of a reader supported by an AIoT RAN node:

Solution 1: the AIoT CN has been configured by O&M about the readers supported by an AIoT RAN node and their location.

Solution 2: the AIoT CN has received the list of readers supported by an AIoT RAN node and their location in an XXAP Setup/Configuration Update message receivd beforehand.

FFS whether to include an optional additional location of the AIoT device determined by the AIoT RAN node in the Inventory Report message.

Topology 2

Select between one or more of these three solutions for enabling the AIoT CN to determine the location of the AIoT_enabled UE Reader to which the AIoT device responded to an Inventory Request:

Solution 1: the AIoT_enabled UE Reader knows its location (e.g. UE has GNSS info) and this is reported to the AIoT CN in the Inventory Report message.

Solution 2: an identifier of the AIoT_enabled UE Reader is reported to the AIoT CN in the Inventory Report and the AIoT CN knows the location of the identified UE using legacy LCS procedure(s).

Solution 3: an identifier of the AIoT_enabled UE Reader is reported to the AIoT CN in the Inventory Report and the AIoT CN knows the location of the identified UE via User Location Information reported by the gNB (in almost all NGAP messages).

TP

Agree the following TP in R3-245827:

[TP for TR 38.769] TP for Locating Ambient IoT devices

# Discussion-First round

## Background

The SID has the following objective:

- Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network.

The TR has the current agreements and editor’s notes.

## 6.8	Locating ambient IoT devices

Editor’s note 1: Proximity determination may be in a 6.8.x sub-clause, or another arrangement, depending on how the study proceeds.

Editor’s note 2: Corresponds to the third RAN3 objective in the SID, to identify potential solutions for locating an Ambient IoT device with no specification impact, e.g., reusing existing user location report, or minimal specification impact to convey location information to core network.

### 6.8.1	General

AIoT device location information may be used for the following purposes:

(a) improving the AIoT operation itself, e.g. by AIoT CN sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly.

(b) providing location information to the consumer of the AIoT service.

Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes.

For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device.

Editor’s Note 1: How to know the “reader” location is FFS. Whether to use more than one “readers” within one AIoT RAN for location purposes is FFS.

Editor’s Note 2: Signalling details on how to provide the location information of the AIoT device to the AIoT CN needs further study.

Editor’s Note 3: Introducing finer granularity for locating an Ambient device besides the “reader ID granularity” needs further study.

Editor’s Note 4: Analysising the gap between the positioning requirements in the SID and the feasibility for AIoT devices.

The following sections aim at rsolution of these editor’s notes.

## How to know the reader location in Topology 1

We have an agreement that AIoT CN should infer the AIoT device location at “reader ID granularity”.

For topology 1 most companies agree that the AIoT RAN node can support multiple readers (e.g. TRPs) which may not be at the exact location of the AIoT RAN node. Therefore, the AIoT RAN node ID cannot serve as the location of the AIoT device.

Proposal 1: agree that the AIoT RAN node ID is not good enough to know the AIoT device location at reader ID granularity.

Question 1: is proposal 1 agreeable?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | One AIoT RAN can have multiple TRPs(readers), we support to have TRP level information. |
| ZTE | Yes | TPRs, and maybe gNBs are also needed. |
| Nokia | Yes |  |
| Samsung | Yes |  |
| Qualcomm | Yes, but.. | We also need discussion on Reader awareness at AIoT CN in T1. And this will decide whether we need just the AIoT RAN node ID or also the Reader ID. We also have to define what is “Reader” in the TP. Can we add a definition in TP e.g., “Reader refers to to a logical entity that hosts the common reader function”. |
| LGE | Yes |  |
| Xiaomi | Yes | TRP ID can be regared as Reader ID |
| CATT | Yes | One AIoT RAN node may hold more than one readers, reader ID granularity could be supported. TRP could be regard as the reader, thus, we can consider reuse NG-RAN ode ID + TRP ID to respresent for the coarse location of the device. |
| NEC | Yes |  |
| Ericsson | Depends on whether this is about purpose a) or b) | For purpose a), knowing the node-level location of the device is sufficient in order to assist subsequent AIoT transactions. But it is for sure better to have a last-Reader-ID-location available. For purpose b) advantageous to have better location information. |

If proposal 1 is agreed, the exact location of the reader ID can be used as the device location at reader ID granularity.

Proposal 2: agree to report information enabling AIoT CN to determine the location of the reader to which the device responded during the location request.

Question 2: is proposal 2 agreeable?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | In T1 as the base station (TRPs) will not move, CN can get TRP location information via OAM configuration or reported from RAN via XX/NG setup procedure, we prefer OAM configuration. |
| ZTE | Yes but | In T1, reporting TRPs to AIoT CN is optional. If absent, AIoT CN thinks the location of the reader is AIoT RAN node, it means that AIoT CN sends one inventory/command request to the AIoT RAN, then AIoT RAN can further select one or more TPRs controlled by itself. |
| Nokia | Yes |  |
| Samsung | Depends | The current P2 introduces extra functionality to AIoT CN on the determination of the reader location, we believe further coordination with SA2 is needed if we intend to do so. |
| Qualcomm | Rephrase | Rephrase something like this: In T1, AIoT CN can determine the reader location via OAM configuration or via coordination between AIoT RAN node and AIoT CN node. |
| LGE | Yes |  |
| Xiaomi | Agree with QC’s rewording | We’re ok to have both options in study phase, i.e., OAM-based and signalling-based |
| CATT | Similar view with QC | Better to make the proposal clear. As TRP information, especially the TRP location info is stable, OAM based solution should be sufficient. |
| NEC | Yes | We do not think we agreed the “location request” from AIoT CN to AIoT RAN reader. So, suggest to reword to “For topology 1, RAN3 agrees to report information enabling AIoT CN to determine the location of the reader. to which the device responded during the location request” |
| Ericsson | Depends on whether this is about purpose a) or b) | Is this about to report the exact location as said above? What does exact mean for the moderator? Is this Q for purpose a) or b)? In case of bi-static, do we know whether both reader IDs are to be reported, or rather the one providing R2D or the D2R reader? |

If proposal 2 is agreeable, there are two types of information for making AIoT CN determine the location of the reader to which the device responded during the location request:

Option 1: the AioT RAN node knows the reader location and reports the Reader’s location together with the AioT device ID

Option 2: the AioT RAN node reports the Reader ID together with the AioT device ID and the AioT CN knows the location of all the reader IDs related to an AioT RAN node.

Option 3: the AIoT RAN node determines the location of the AIoT device by means of either, in case of topology 1, AIoT RAN node internal implementation, e.g. triangulation by means of measuring the backscattered signals, or equivalent of topology 2, by means of a specified approach with necessary changes on e.g. NR Uu, etc.

Question 3: which option do you prefer?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Option 2 | In T1 as the base station (TRPs) will not move, CN can get TRP location information via OAM configuration or reported from RAN via XX setup procedure, we prefer OAM configuration. Therefore, for each device, RAN only need to report reader ID. |
| ZTE | Option 2 | Agree with HW. |
| Nokia | Option 2 |  |
| Samsung | Option 2 |  |
| Qualomm | Option 2 |  |
| LGE | Option 2 |  |
| Xiaomi | Option 2 |  |
| CATT | Option 2 |  |
| NEC | Option 2 |  |
| Ericsson | Depends on whether this is about purpose a) or b) | We assume that is all for purpose b). There is no function in AIOT CN associated with the location information, the information would be for the application. And the queston is whether we should go via the LCS architecture. This is not about “preference”, the Q is whether 1 and 2 can serve the purpose of providing a device location with an assumed accuracy in the range of the projected communication range of a reader is sufficient, in dense device situations and 3D setup of e.g. warehouses. Such approximation of the device location however is not suitable for avoiding (human) labour intensive and time-consuming operations and not suitable for performing subsequent automated operations, e.g. retrieval of goods for further transportation by means of roboters. Options 1 and 2 are not suitable for such operations, option 3 or any equivalent approach is. |

For option 2, there are two solutions for AIoT CN to know the location of each reader ID of an AioT RAN node:

Solution 1: AioT CN is configured by O&M about the readers supported by an AioT RAN node and their location

Solution 2: AioT CN receives the list of readers supported by an AioT RAN node and their location in NG setup/update like procedure.

Question 4: which solution do you prefer?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | 1 | Slightly prefer solution 1. |
| ZTE | 1 | OAM, no need to by signalling |
| Nokia | Solution 2 | No need of configuration effort, like Tas are configured in RAN and automatically reported to CN. |
| Samsung |  | Our understanding is that an AioT RAN node sends the reader list is enough for T1 for basic operations. |
| Qualcomm | Capture both options in TR | This needs more discussion. |
| LGE | Solution 2 | Agree with Nokia |
| Xiaomi | 1 and 2 | We think both solutions can be captured in the study. |
| CATT | Both, prefer 1 | Both of them are feasible, prefer 1 a little bit. |
| NEC | 1 |  |
| Ericsson | Too detailled for too many open Qs. | That is going to far, too many open questions. |

## How to know the reader location in Topology 2

We have an agreement to have “reader ID granularity”.

For topology 2 the reader is the UE reader. Therefore, the proposal becomes:

Proposal 1: agree to report information enabling AIoT CN to determine the location of the reader to which the device responded during the location request.

Question 5: is proposal 1 agreeable?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | ? | Not understand the question…. CN should know from which UE the device access from, therefore for RRC based solution, RAN need to report UE ID, for NAS/UP based solutions, CN knows by default. |
| ZTE | Yes |  |
| Nokia | Yes | Same as topology 1 question 2. |
| Samsung |  | Same the answer above. In addition, we can use legacy positioning methods to obtain the location of the UE reader in an AioT enabled CN. |
| Qualcomm | Repharse | In T2, AioT CN can determine the reader location (i.e., the location of A-IoT enabled UE). Whether this can be done via existing positioning techniques or via existing ULI mechanism or via new dedicated procedures is to be discussed. |
| LGE | Yes |  |
| Xiaomi | Agree with QC’s rewording |  |
| CATT | Yes | AIoT CN could deduce the location of UE reader via ULI, or any LCS method. No extra spec change is expected. |
| NEC | Yes |  |
| Ericsson | For now we can take both Topologys together. |  |

For AIoT CN to know the location of the reader, if the reader is stationary then same as topology 1 can be applied.

Considering moving UE readers, O&M solution is not doable, there are similarly two options:

Option 1: UE knows its location and reports it to AIoT CN via RRC/NG (RRC stack), via NAS (NAS stack), via UP (UP stack)

Option 2: UE reports its UE ID to AIoT CN and AIoT CN knows the location of UE ID.  For option 2, the SID does not foresee standards impact therefore option 2 assumes that AIoT CN has an AMF and UE ID is received by the AMF which can infer the location of the UE using legacy LCS procedure.

Question 6: which option should be supported?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Neither | CN knows UE location via User location information reported by RAN (in almost all UL NGAP messages), if CN would like to have more detailed location of the UE, CN triggers legacy postioning which does not have specification impact. |
| ZTE | Option 2 |  |
| Nokia | Option 2 |  |
| Samsung |  | Share view with Huawei. |
| Qualcomm | Capture all options open. | We can discuss in normative phase whether ULI is reused, or we reuse existing positioning techniques or both. In any case, not much spec impact |
| LGE | Option 2 |  |
| Xiaomi | Depend on the types of the location | If it means the geographical location info of the UE, option 1 can not be applied to RRC-based solution for the privacy issue. For option 1, for NAS/UP-based sonlution, UE can report its location with sercurity guarantee. If UE can have GNSS info, it does not need to trigger LCS service, it can indicate the network directly. For option 2 is also possible by exsiting meachnism |
| CATT | Option 2 | No matter UE reader is movement or not, AIoT CN could get the location of UE reader without any extra UU/NG/XX impact. |
| NEC | Option 2 |  |
| Ericsson | See Q3 and Q9, and the other open questions. no need to go further at the moment. |  |

## Signalling on how to report the location of the AIoT device to AIoT CN

Several companies proposed to report the location of the AIoT device using the Inventory Report message. One company in R3-245333 proposed instead to introduce a new location reporting configuration procedure:

Option A: report the device ID- Location of device ID in the Inventory Report message

Option B: introduce a new Location Reporting procedure as follows:

- The AIoT location report config can include when/how to report location information e.g., location to be reported just “one-time” or periodically or upon some other event being met. FFS on the type of events.

Question 7: which option do you prefer?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Option A | At least A should be supported. |
| ZTE | No | Do not need to report of the device ID-Location. Reader can report the association between device list and itself to the AIoT CN. Then AIoT CN can use reader ID-Location as device ID-Location. |
| Nokia | Option A |  |
| Samsung |  | If the location of device ID is the associated reader id, then Option A is ok. |
| Qualcomm | Both |  |
| LGE | Option A |  |
| Xiaomi | At least option A | Whether B is needed pending to SA2’s requirement |
| CATT | No | Maybe no need to report anything about device location, CN could know the mapping betwee devices and UE readers via the UE reader specific signalling over XXAP/NGAP. CN could know the UE reader granularity device location. |
| NEC | Option A |  |
| Ericsson | Depends on the purpose | See answers to other Qs. |

## Add Finer location granularity

Do you support add finer granlurity than reader granularity:

Option 1: add finer granularity

Option 2: not add finer granularity

Question 8: which option do you support?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | No need, based on SID. |
| ZTE | No |  |
| Nokia | No |  |
| Samsung | No |  |
| LGE | No |  |
| Xiaomi |  | Finer granularity can be further discussed if time allowed |
| CATT | No |  |
| NEC | No | Not for this release, may be studied in further release. |
| Ericsson | See Q9 |  |

## Gap with positioning requirements of the SID

Do you think that reader granularity has a gap with SID requirements?

Option 1: yes, there is a gap, explain why

Option 2: no gap, explain why

Question 9: which option you think is right?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Huawei | No | - SID: Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. Do not see the gap. |
| ZTE | No |  |
| Nokia | No | Same view as Huawei. |
| Samsung | No |  |
| CATT | No | In this release, no clear objective to support the use cases with finer positioning accuracy . |
| Ericsson |  | See answer to question 3. There is no need to identify a detailled solution that goes beyond what is stated in the objective. But it is very valid to study, whether, from requirement point of view, purposes a) and b) can be supported for realistic and likely use-cases. In our opinion, purpose b) needs finer granularity. If this can be stated as the outcome of the study, that would be honest and could provide a useful answer to necessary work/study in the future. |

# Conclusion, Recommendations

# References