3GPP TSG-RAN WG3 #125-bis                                             	                          R3-245745

Hefei, China, October 14th –18th, 2024                                       Revision of R3-245314

Agenda item:	   13.2

Source: 		Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India), Reliance JIO

Title: 	Enabling subsequent inter-gNB LTM by decoupling of cell and anchor changes

Document for:	Discussion

# 1	Introduction

In the last RAN3 meeting (RAN3 #125 [1]), further progress was made in developing the procedures of inter-CU LTM considering the case when the gNB-CU is switched at LTM cell switch.

RAN3 #125 agreements:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

This paper discusses inter-gNB LTM with decoupled cell/CU anchor changes.

# 2	Main idea: decoupling of cell switch and CU anchor switch in inter-gNB LTM

## 2.1	Part 1: support subsequent cell switches with no anchor change

Figure 1a – Subsequent inter-gNB LTM cell switches with no CU anchor change

Figure 1a illustrates the inter-gNB LTM scenario, where the UE undergoes multiple inter-gNB cell switches while remaining connected to the same RRC/PDCP anchor.

## 2.2	Part 2 [optional]: support CU anchor change with no cell switch

Figure 1b –CU anchor change with no cell switch

Figure 1b illustrates the scenario where the UE connects to a gNB-CU through a cell managed by another gNB-CU. In this scenario, the UE’s RRC/PDCP anchor is switched between the two gNB-CUs without changing the UE’s serving cell.

## 2.3	Motivation/benefits

Decoupling the inter-gNB cell switch from the CU anchor switch allows for:

- Reusing legacy UE procedures to support subsequent mobility

- Reusing legacy UE procedures to support CU anchor changes

- Skipping security updates and PDCP reestablishment during cell switches

- Simplifying network procedures to support subsequent inter-gNB LTM mobility, such as eliminating the need for NG-U path switches, Xn preparation per cell switch, and data forwarding issues

Observation 1: Inter-gNB LTM with decoupled cell and CU anchor switches allows for reusing legacy UE procedures to support subsequent mobility, improving UE performance, and simplifying network handling of LTM procedures.

Proposal 1a: RAN3 to pursue inter-gNB LTM with decoupled cell and CU anchor changes, including:

- Supporting subsequent inter-gNB LTM cell switches without CU anchor change

- Supporting CU anchor changes without cell switch

Proposal 1b: RAN3 to reuse legacy procedures and signaling flows for inter-gNB LTM with decoupled cell and CU anchor changes, where possible.

# 3	Signaling Procedures

## 3.1	Procedure for preparation of candidate configuration

| Figure – NR-DC addition of SCG (legacy) | Figure – LTM preparation |
| --- | --- |

The inter-gNB LTM preparation procedure can reuse procedures from the NR-DC framework for the addition of SCG.

Proposal 2: The Inter-gNB LTM preparation procedure to use same the signaling flow and information exchange as the NR-DC SCG addition, where the serving gNB provides the candidate gNB:

- DRB info,

- Serving gNB-side TNL info

- SRB IDs for admission

and the candidate gNB returns:

- Candidate cell group config

- Admitted DRB/SRB IDs

- Candidate-gNB TNL info

## 3.2	Procedure for cell switch without anchor change:

| Figure – Intra-gNB LTM execution (legacy) | Figure – Inter-gNB LTM execution |
| --- | --- |

The Rel-19 LTM inter-gNB cell switch procedure can reuse the Rel-18 LTM intra-gNB cell switch procedure.

Proposal 3a: The Rel-19 LTM inter-gNB cell switch procedure to reuse the Rel-18 LTM intra-gNB cell switch procedure.

## 3.3	Procedure for subsequent cell switch without anchor change:

| Figure – Subsequent intra-gNB LTM execution (legacy) | Figure – Subsequent inter-gNB LTM execution |
| --- | --- |

The Rel-19 LTM subsequent inter-gNB cell switch procedure can reuse the Rel-18 LTM subsequent intra-gNB cell switch procedure, which is similar to the initial cell switch procedure.

Proposal 3b: The Rel-19 LTM subsequent inter-gNB cell switch procedure to reuse the Rel-18 LTM subsequent intra-gNB cell switch procedure.

## 3.4	Procedure for gNB1-initiated modification of RB config which may lead to gNB2-modification of CellGroupConfig (if supported)

| Figure – MN-initiated modification of MN-terminated SCG bearers (legacy) | Figure – gNB1-initiated modification of RB config |
| --- | --- |

This is similar to LTM preparation procedure.

Proposal 4: The anchor-gNB-initiated modification of the RB config to reuse the signaling flow and information exchange from the NR-DC framework for MN-initiated modification of the MN-terminated SCG bearer config, where:

- The anchor gNB-CU sends updated DRB info, anchor-gNB-side TNL info, and SRB IDs for admission to the candidate gNB, and

- The candidate gNB returns updated candidate cell group config, admitted DRB/SRB IDs and candidate-gNB TNL info to the anchor gNB-CU

## 3.5	Procedure for gNB2-initiated modification of CellGroupConfig (if supported)

| Figure – SN-initiated modification of CPA-CellGroupConfig (legacy) | Figure – gNB2-initiated modification of LTM candidate CellGroupConfig |
| --- | --- |

The candidate-gNB-initiated modification of the LTM candidate CellGroupConfig can reuse procedures from the CPA framework for SN-initiated modification of the CPA candidate CellGroupConfig.

Proposal 5: The candidate-gNB-initiated modification of the LTM candidate CG Config to reuse the signaling flow and information exchange from the CPA framework for SN-initiated modification of the CPA candidate CGConfig, where:

- The candidate gNB-DU sends the updated candidate CellGroupConfig to its controlling gNB-CU

- The controlling gNB-CU sends the updated candidate CellGroupConfig to the anchor gNB-CU

- The anchor-gNB-CU reconfigures the UE with the updated candidate CellGroupConfig and confirms the change to the controlling gNB-CU.

## 3.6	Procedure for ULI:

| Figure – NR-DC ULI (legacy) | Figure – LTM ULI |
| --- | --- |

The location reporting procedure during inter-gNB LTM can reuse signaling flow and information exchange from the location reporting procedure of NR-DC.

Proposal 6: The location reporting procedure during inter-gNB LTM to reuse signaling flow and information exchange from the location reporting procedure of NR-DC.

## 3.7	Procedure for transport of RRC messages:

| Figure – NR-DC RRC transfer (legacy) | Figure – LTM RRC transfer |
| --- | --- |

The transfer of RRC messages between the UE and the anchor-CU during inter-gNB LTM can reuse the RRC transfer procedure of NR-DC with stage-3 enhancements.

Proposal 7a: Xn/F1 RRC transfer procedures to reuse the legacy transport of RRC messages between the UE and the anchor gNB-CU.

## 3.8	Procedure for transport of UP traffic:

| Figure – NR-DC UP traffic transport (legacy) | Figure – LTM UP traffic transport |
| --- | --- |

The transfer of UP traffic between the UE and the anchor-CU during inter-gNB LTM can reuse the UP traffic transport mechanism of MN-terminated SCG bearers in NR-DC.

Proposal 7b: The transfer of UP traffic between the UE and the anchor-CU during inter-gNB LTM to reuse the UP traffic transport mechanism of MN-terminated SCG bearers in NR-DC.

## 3.9	Procedure for anchor switch with no cell change:

| Figure – Security update with no cell change (legacy) | Figure – LTM CU anchor change with no cell change |
| --- | --- |

The procedure for CU-anchor switch without cell switch can reuse the Rel-15 RRC Reconfiguration procedure for security change without cell change. Additionally, the procedure for CU-anchor switch requires a UE context transfer procedure between the source anchor-CU and the target anchor-CU.

Proposal 8a: Reuse the Rel-15 RRC Reconfiguration procedure for security update without cell change to support switch of anchor-CUs without serving cell change.

Proposal 8b: Support UE context transfer procedure between the source anchor CU and target anchor CU to support CU anchor switch with no cell switch.

## 3.10	Summary

Based on the above, the spec effort to support inter-gNB LTM with decoupling of cell/CU anchor change is minimal.

Observation 2: The necessary specification effort to support inter-gNB LTM with decoupling of cell/CU anchor change is minimal.

# Conclusion

This contribution discussed inter-gNB LTM with decoupled cell/CU anchor changes. The following observations and proposals have been made:

Motivation/benefits:

Observation 1: Inter-gNB LTM with decoupled cell and CU anchor switches allows for reusing legacy UE procedures to support subsequent mobility, improving UE performance, and simplifying network handling of LTM procedures.

Proposal 1a: RAN3 to pursue inter-gNB LTM with decoupled cell and CU anchor changes, including:

- Supporting subsequent inter-gNB LTM cell switches without CU anchor change

- Supporting CU anchor changes without cell switch

Proposal 1b: RAN3 to reuse legacy procedures and signaling flows for inter-gNB LTM with decoupled cell and CU anchor changes, where possible.

Procedure for preparation of candidate configuration:

Proposal 2: The Inter-gNB LTM preparation procedure to use same the signaling flow and information exchange as the NR-DC SCG addition, where the serving gNB provides the candidate gNB:

- DRB info,

- Serving gNB-side TNL info

- SRB IDs for admission

and the candidate gNB returns:

- Candidate cell group config

- Admitted DRB/SRB IDs

- Candidate-gNB TNL info

Procedure for [subsequent] cell switch without anchor change:

Proposal 3a: The Rel-19 LTM inter-gNB cell switch procedure to reuse the Rel-18 LTM intra-gNB cell switch procedure.

Proposal 3b: The Rel-19 LTM subsequent inter-gNB cell switch procedure to reuse the Rel-18 LTM subsequent intra-gNB cell switch procedure.

Procedure for gNB1-initiated modification of RB config (if supported):

Proposal 4: The anchor-gNB-initiated modification of the RB config to reuse the signaling flow and information exchange from the NR-DC framework for MN-initiated modification of the MN-terminated SCG bearer config, where:

- The anchor gNB-CU sends updated DRB info, anchor-gNB-side TNL info, and SRB IDs for admission to the candidate gNB, and

- The candidate gNB returns updated candidate cell group config, admitted DRB/SRB IDs and candidate-gNB TNL info to the anchor gNB-CU

Procedure for gNB2-initiated modification of CellGroupConfig (if supported):

Proposal 5: The candidate-gNB-initiated modification of the LTM candidate CG Config to reuse the signaling flow and information exchange from the CPA framework for SN-initiated modification of the CPA candidate CGConfig, where:

- The candidate gNB-DU sends the updated candidate CellGroupConfig to its controlling gNB-CU

- The controlling gNB-CU sends the updated candidate CellGroupConfig to the anchor gNB-CU

- The anchor-gNB-CU reconfigures the UE with the updated candidate CellGroupConfig and confirms the change to the controlling gNB-CU.

Procedure for ULI:

Proposal 6: The location reporting procedure during inter-gNB LTM to reuse signaling flow and information exchange from the location reporting procedure of NR-DC.

Procedure for transport of RRC messages/UP traffic:

Proposal 7a: Xn/F1 RRC transfer procedures to reuse the legacy transport of RRC messages between the UE and the anchor gNB-CU.

Proposal 7b: The transfer of UP traffic between the UE and the anchor-CU during inter-gNB LTM to reuse the UP traffic transport mechanism of MN-terminated SCG bearers in NR-DC.

Procedure for anchor switch with no cell change:

Proposal 8a: Reuse the Rel-15 RRC Reconfiguration procedure for security update without cell change to support switch of anchor-CUs without serving cell change.

Proposal 8b: Support UE context transfer procedure between the source anchor CU and target anchor CU to support CU anchor switch with no cell switch.

Summary:

Observation 2: The necessary specification effort to support inter-gNB LTM with decoupling of cell/CU anchor change is minimal.

# References

- Chair notes, RAN3 #125 meeting.

- R3-245313, RAN3 #125-bis contribution, Signalling enhancements for inter-CU LTM HO, Qualcomm Incorporated, Vodafone, Bharti Airtel (India).