3GPP TSG RAN WG3#125bis	R3-245747

14th October – 18th October - 2024

Hefei, China

Agenda Item:	13.2

Source:	China Telecom, Lenovo

Title:	Summary of offline on LTM procedures

Document for:	Approval

Introduction

CB: # R19MobilityEnh

- Capture the agreements of stage-2 and stage-3 for LTM procedures for inter-CU scenarios

- working on TPs based on the above agreements.

- compare two solutions for Inter-CU LTM (5201, 5676, 5315), and identify the critical issue of each solution if any, capture the views in R3-245748.

(moderator - CT and Lenovo)

Summary of the offline discussion in R3-245747

For the Chairman’s Notes

Propose the following:

R3-245203 rev [in R3-245798] – to be agreed

R3-245317 rev [in R3-245833] – to be agreed

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

Discussion

During the online section, we reached following progress on support of inter-CU LTM.

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

The LTM Configuration IDs are allocated by the source CU.

In this offline, we plan to continue to discuss the remaining issues on support of inter-CU LTM.

Support of LTM in Standalone scenario

Initial LTM configuration request

CSI related configuration

During the online session, we reached the following working assumption:

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Moderator wants to check whether we can turn this Working Assumption into agreement?

Moderator summary:

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Reference configuration

RAN2 agreed to support one single reference configuration in Rel-19 inter-CU LTM, as shown below:

RAN2 #127 1. Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

Basically, it means that the source gNB-CU can generate a reference configuration and sends the reference configuration to all candidate gNBs in Handover Request message.

Proposal: The source gNB-CU sends the reference configuration to all candidate gNBs in Handover Request message.

Question: Check whether the above proposal is agreeable?

Moderator summary:

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

LTM Configuration Update

In endorsed BLCR for TS38.300, there are remining issues in step6 and 7:

6. The source gNB requests LTM configuration update to the candidate gNB(s). Editor’s Note: Details are FFS on step 6. 7. The candidate gNB(s) sends the response (LTM CONFIGURATION UPDATE ACKNOWLEDGE) to the source gNB. Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional and FFS on the message name.

In R3-245568[LGE], it has following proposals:

Proposal 3: Confirm the name of the second round-trip procedure as “LTM Configuration Update”, as suggested in our current 38.300 BL CR.

Proposal 4: The second round-trip (“LTM Configuration Update”) is marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM.

Besides, if we reached consensus on initial LTM configurations, we could further discuss what LTM information should be included in the new introduced LTM Configuration Update message (e.g. in Step6 and 7).

The potential proposal can be as follow:

- CSI related configuration (CSI resource and report configuration) can be updated via LTM Configuration Update procedure.

- Early sync configuration (TCI state and RACH configuration) can be updated via LTM Configuration Update procedure.

- LTM reference configuration can be updated via LTM Configuration Update procedure.

Moderator wants to check companies’ views towards this issue and whether we can reach some agreements.

Moderator summary:

Confirm the name of the new procedure as “LTM Configuration Update”.

TA Information Transfer

During the online session, we have a brief discussion on the TA Information Transfer message based on Nokia’s proposal.

Online notes are cited as below: I Introduce a new Class-2 non-UE associated XnAP TA INFORMATION TRANSFER procedure from Target gNB to Source gNB to signal Early TA related information for LTM. E///: Prefer for the UE associated solution. Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario. RAN3 to study which information is to be sent from target gNB to source gNB along with TA values within XnAP TA INFORMATION TRANSFER and CU-DU/DU-CU TA INFORMATION TRANSFER messages.

Here we can discuss the message type and what information should be included in the TA Information Transfer message.

Moderator summary:

Candidate-initiated LTM Cancellation

In the last RAN3 meeting, we have adopted Class-2 procedure for candidate gNB-initiated LTM cancellation, but we left an open issue to down select from Option1 and Option3 in this meeting:

- Option 1: Reuse CHO Cancel [ZTE, Lenovo, NEC, CMCC]

- Option 2: Rename CHO Cancel

- Option 3: Introduce new procedure [Samsung, Huawei, CATT, Ericsson, CT]

Many companies show their preference on the above options in their contribution, moderators want to further check companies’ views and decide which option to choose.

Question: Which option to choose? Option1 or Option3?

Moderator summary:

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

LTM Release

In RAN3#124 meeting, we left an open issue on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Currently there is an FFS in the current BLCR for TS38.300 (the editor notes under Step20).

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue. Editor’s Note: Details are FFS on step 20.

During the online section, we reached following working assumption:

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB

Moderator wants to check whether we can change the Working Assumption into agreement and remove the FFS in current BLCR.

Moderator summary:

UE association setup

For subsequent LTM, after each cell switch, the UE association need to be established between the new serving gNB and candidate gNB(s). Based on companies’ contributions, there are following proposals:

In [5040, Nokia]:

Proposal 9: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM MODIFICATION REQUEST message to update the information related to a given UE context.

In [5050, ZTE]:

Proposal 12: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used.

In [5202, HW]:

One possible solution is that each candidate gNB allocates UE XnAP ID and send to other candidate gNBs in the preparation step.

Another solution is that after each cell switch, the new serving gNB allocates the source node UE XnAP ID and send it to other candidate gNB-CUs, and then the candidate gNB-CUs reply with the target node UE XnAP ID. This UE XnAP ID negotiation can be done for example, during the handover success notification procedure.

Proposal 14: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case.

In [5316, Ericsson]:

Proposal 6: To support subsequent LTM in the RAN, the new XnAP procedure (e.g., LTM Configuration Update) is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs.

In [5568, LGE]:

Proposal 7: Discuss and consider allowing the UE associated XnAP signalling connections directly between C-CUs, for the unified/simplified specification handling and reduced signaling overhead.

Moderator summarized above proposals in to following options to be further discussed:

- Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and send to other candidate gNBs. [HW, LGE]

- Option 2: After each cell switch, the new serving gNB signals XnAP ID to other candidate gNBs, and then the candidate gNB reply with the Target node UE XnAP ID. [Nokia, HW, Ericsson]

- Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB. [ZTE]

Question: Which option is agreeable? And which message to use?

Moderator summary:

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

Collected LTM information transfer

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, etc) may need to sync up among candidate gNBs.

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

(the step 7 is CU->DU: UE CONTEXT MODICATION REQUEST; the step 19 is DU->CU: DU-CU CELL SWITCH NOTIFICATION; the step 22 is DU->CU: ACCESS SUCCESS)

RAN3 to discuss how the LTM configurations are synced up among candidate gNBs in case of subsequent LTM.  Similar as F1AP, there are possible three options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) after the initial handover preparation.

- Option 2: Send the collected LTM information to the target gNB when the source gNB send the LTM Cell Switch Command to the UE (e.g., via Cell Switch Notification message).

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

In R3-245280, it has following proposal:

Proposal: To align with Rel-18 intra-CU LTM, to take all Option 1, 2 and 3 can be done by implementation.

Moderator summary:

Not treated.

Data forwarding

In current BLCR for TS38.300, there is an FFS under the note about the late data forwarding:

NOTE : Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. Editor’s Note: Details are FFS on above note.

It seems a common understating in RAN3 that the late data forwarding can be initiated as soon as the source gNB receives the HANDOVER SUCCESS message, even if the early data forwarding is supported for inter-CU LTM.

Moderator wants to have a quick check on whether we can remove the editor note in current BLCR.

Proposal: Agree the late data forwarding can be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

Moderator summary:

Not treated.

Based on companies’ contribution, another data forwarding issue need to be solved is the data forwarding address transfer among candidate gNBs for subsequent LTM:

In [5163, SS], it has following proposals:

Proposal 4: Candidate gNB provides the data forwarding proposal and TNL addresses information for admitted PDU sessions or DRBs in HO REQ ACK in advance during LTM preparation phase.

Proposal 5: Source gNB provides the collected all the data forwarding addresses information (including source and candidate gNB) to other each candidate gNB if this candidate gNB proposes data forwarding proposal during LTM preparation phase.

In [5040, Nokia], it has following proposals:

Proposal 8: The Source gNB includes the PDU Session Resource Information List information of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

In [5050, ZTE], it has following proposals:

Proposal 10: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to perform data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Proposal 11: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure.

Moderator summarized above proposals in to following options to be further discussed:

- Option 1: The data forwarding address is send to the candidate node during LTM preparation phase.

- Option 2: The data forwarding addresses is send to the candidate node in the XnAP CELL SWITCH NOTIFICATION message

- Option 3: The new serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Question: Which option is agreeable? And which message to use?

Moderator summary:

Not treated.

Support of inter-CU LTM in NR-DC scenario.

If time allows, we can open the discussion on support of inter-CU LTM in DC scenario in the offline.

Applicable scenario

Based on [5041, Nokia], the following are NR-DC scenarios applicable for Rel-19 LTM DC work scope:

- Inter-CU SCG LTM with unchanged MCG

- Inter-CU MCG LTM with SCG retained or released

Given that intra-CU LTM scenarios for both MCG and SCG LTM were in scope of Rel-18, in Rel-19 there is need to further support coexistence of these scenarios with Inter-CU scenarios as follows:

- Simultaneous configuration and operation of Intra-CU MCG LTM and Inter-CU SCG LTM

- Simultaneous configuration and operation of Intra-CU SCG LTM and Inter-CU MCG LTM

Proposal 1: RAN3 to agree to support the coexistence of the (a) Intra-CU MCG LTM and Inter-CU SCG LTM, and (b) Intra-CU SCG LTM and Inter-CU MCG LTM.

Question: Check whether the above proposal is agreeable?

Moderator summary:

Not treated.

Avoid simultaneous MCG and SCG LTM

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Based on companies’ contributions, there are following proposals:

In [5041, Nokia]:

Proposal 2: In case of NR-DC, MN indicates to SN about inter-CU MCG LTM configuration/de-configuration.

Proposal 3: In case of NR-DC, SN indicates to MN about inter-CU SCG LTM configuration/de-configuration.

In [5202, HW]:

Proposal 15: When the MN decides to initiate inter-CU MCG LTM for a UE, it sends a notification to the SN.

Proposal 16: When the SN decides to initiate inter-CU SCG LTM for a UE, it sends a request to the MN, which may be accepted or rejected.

In [5460, Lenovo]:

Proposal 5: MN indicates SN about MCG LTM configuration, to avoid simultaneous MCG and SCG LTM configurations.

Proposal 6: SN indicates MN about SCG LTM configuration, to avoid simultaneous MCG and SCG LTM configurations.

Question: Check which proposal is agreeable?

Moderator summary:

Not treated.

Stage 2/Stage 3 TPs

Based on the chairman notes’ s guidance, in this offline discussion, we should work on stage2/3 TPs based on the agreements. Since we have already endorsed the BLCR for TS38.300, we can update the current stage2 BLCR first and then try to work on Stage3 XnAP. If time allows, we can further check whether to adopt other stage2 BLCR/TPs (i.e. TS38.401, TS 38.420).

TPs for TS38.300

The TP for TS38.300 is proposed in [5318, E///], [5203, HW], [5144, CT], [5246, CATT], [5522, CMCC], [5653, ZTE].

TPs for TS38.423

The TP for TS38.423 is proposed in [5317, E///], [5652, ZTE], [5281, NEC], [5459, Lenovo], [5570, LGE].

TPs for TS38.401

The TP for TS38.401 is proposed in [5045, Nokia] and [5164, Samsung].

Other TPs

The TP for TS 38.473 is proposed in [5204, HW], and the baseline CR for E1AP is proposed in [5569, LGE].

Conclusion, Recommendations [if needed]

If needed

References

| R3-245032 | (BL CR to 38.300) Inter-CU LTM basic procedure (Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung) | draftCR |
| --- | --- | --- |
| R3-245007 | LS on security key update of inter-CU SCG LTM (RAN2(CATT)) | LS in |
| R3-245040 | Discussion on Rel-19 Inter-CU LTM (Nokia) | discussion |
| R3-245202 | Discussion on Inter-CU LTM procedure (Huawei) | discussion |
| R3-245280 | inter-CU LTM issues (NEC) | discussion |
| R3-245050 | Discussion on inter-CU LTM (ZTE Corporation) | discussion |
| R3-245142 | Discussion on signaling enhancement for inter-CU LTM (China Telecom) | discussion |
| R3-245162 | Discussion on inter-gNB-CU LTM (Samsung) | discussion |
| R3-245568 | Continuing discussions on Inter-CU LTM signaling design (LG Electronics Inc.) | discussion |
| R3-245585 | Discussion on inter-CU LTM non-DC case (NTT DOCOMO INC..) | discussion |
| R3-245458 | [TP to BLCR for TS 38.300] Inter-CU LTM (Lenovo) | other |
| R3-245316 | Further discussion on XnAP signaling for inter-CU LTM (Ericsson) | discussion |
| R3-245645 | Initial considerations on Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-245264 | Discussion on Early Sync and CFRA resource configurations in inter-CU LTM (Google) | discussion |
| R3-245163 | Additional discussion on Inter-gNB-CU LTM (Samsung) | discussion |
| R3-245643 | Data forwarding and transmission in Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-245245 | Discussion for Inter-CU LTM in DC (CATT) | discussion |
| R3-245460 | General aspects of inter-CU SCG LTM (Lenovo) | discussion |
| R3-245041 | Discussion on Inter-CU LTM Dual Connectivity (Nokia) | discussion |
| R3-245143 | Discussion on support of inter-CU LTM in NR-DC scenario (China Telecom) | discussion |
| R3-245244 | Discussion for Inter-CU LTM (CATT) | discussion |
| R3-245521 | Discussion on inter-CU LTM (CMCC) | discussion |
| R3-245318 | (TP for TS 38.300) Stage-2 signalings for inter-CU LTM (Ericsson) | other |
| R3-245203 | (TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure (Huawei) | other |
| R3-245144 | (TP for TS38.300) On support of inter-CU LTM (China Telecom) | other |
| R3-245246 | TP for LTM BLCR for TS38.300 (CATT) | other |
| R3-245522 | (TP to BL CR for TS 38.300) Inter-CU LTM procedure (CMCC) | other |
| R3-245653 | TP to 38.300 for inter-CU LTM (ZTE Corporation) | other |
| R3-245045 | TP (BL CR TS 38.401) L1/2 Triggered Mobility (LTM) Procedures (Nokia) | other |
| R3-245164 | (TP to BLCR for TS 38.401) Procedure on Inter-gNB-CU LTM (Samsung) | other |
| R3-245317 | (TP for TS 38.423) Xn support for inter-CU LTM (Ericsson) | other |
| R3-245652 | TP to 38.423 for inter-CU LTM (ZTE Corporation) | other |
| R3-245281 | (TP to 38.423 on inter-CU LTM) inter-CU LTM related information (NEC) | other |
| R3-245459 | [TP for TS 38.423] Inter-CU LTM (Lenovo) | other |
| R3-245570 | (TP for NR_Mob_Ph4 TS 38.423) (LG Electronics Inc.) | other |
| R3-245204 | (TP for LTM BLCR for TS 38.473): F1 impact for Inter-CU LTM (Huawei) | other |
| R3-245569 | Baseline CR for introducing Rel-19 Mobility enhancement in E1AP (LG Electronics Inc.) | CR0130r3, TS 37.483 v18.2.0, Rel-19, Cat. B |
| R3-245313 | Signalling enhancements for inter-CU LTM HO (Qualcomm Incorporated, Vodafone, Bharti Airtel (India)) | discussion |
| R3-245201 | Discussion on the PDCP anchor based solution (Huawei, Ericsson, CATT, China Telecom, CMCC,Samsung) | Discussion Resp in R3-245676 |
| R3-245314 | Enabling subsequent inter-gNB LTM by decoupling of cell and anchor changes (Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India)) | discussion |
| R3-245315 | Impact analysis of coupling LTM cell switch and CU anchor changes on subsequent inter-gNB LTM (Qualcomm Incorporated, Vodafone, NTT DOCOMO, Sony, Bharti Airtel (India)) | discussion |
| R3-245676 | Response to R3-245201 | discussion |

Appendix

Previous meeting agreements:

RAN3#123bis:

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

RAN3#124:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

RAN3#125:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure