3GPP TSG-RAN WG3 Meeting #125bis	R3-245764

Hefei, China, 14 -18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1501 | rev | 1 | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on BAP mapping configuration for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, NEC, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R16 IAB topic, the following two IEs are included in the BAP MAPPING CONFIGURATION message: - BH Routing Information Added List IE. It contains only one child IE which is BH Routing Information Added List Item IE - BH Routing Information Removed List IE. It contains the BH Routing Information Removed List Item IE as the only child IE In Rel-17 IAB enhancement, two new IEs are added in the BAP MAPPING CONFIGURATION message: - BAP Header Rewriting Added List IE. It contains only one child IE which is BAP Header Rewriting Added List Item IE - BAP Header Rewriting Removed List IE. It contains the BAP Header Rewriting Removed List Item IE as the only child IE And it is noticed that in ASN.1 part, ProtocolIE-SingleContainer is used to define the BH-Routing-Information-Added-List-ItemIEs, BH-Routing-Information-Removed-List-ItemIEs, BAP-Header-Rewriting-Added-List-ItemIEs, BAP-Header-Rewriting-Removed-List-ItemIE. In such case, the id-BH-Routing-Information-Added-List-Item, id-BH-Routing-Information-Removed-List-Item, id-BAP-Header-Rewriting-Added-List-Item, and id-BAP-Header-Rewriting-Removed-List-Item should be included as Mandatory IE, but in current specification, they are marked as optional. BH-Routing-Information-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries)) OF ProtocolIE-SingleContainer { { BH-Routing-Information-Added-List-ItemIEs } } BH-Routing-Information-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries)) OF ProtocolIE-SingleContainer { { BH-Routing-Information-Removed-List-ItemIEs } } BH-Routing-Information-Added-List-ItemIEs F1AP-PROTOCOL-IES ::= { { ID id-BH-Routing-Information-Added-List-Item CRITICALITY ignore TYPE BH-Routing-Information-Added-List-Item PRESENCE optional }, ... } BH-Routing-Information-Removed-List-ItemIEs F1AP-PROTOCOL-IES ::= { { ID id-BH-Routing-Information-Removed-List-Item CRITICALITY ignore TYPE BH-Routing-Information-Removed-List-Item PRESENCE optional }, BAP-Header-Rewriting-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries)) OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Added-List-ItemIEs } } BAP-Header-Rewriting-Added-List-ItemIEs F1AP-PROTOCOL-IES ::= { { ID id-BAP-Header-Rewriting-Added-List-Item CRITICALITY ignore TYPE BAP-Header-Rewriting-Added-List-Item PRESENCE optional}, ... } BAP-Header-Rewriting-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries)) OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Removed-List-ItemIEs } } BAP-Header-Rewriting-Removed-List-ItemIEs F1AP-PROTOCOL-IES ::= { { ID id-BAP-Header-Rewriting-Removed-List-Item CRITICALITY ignore TYPE BAP-Header-Rewriting-Removed-List-Item PRESENCE optional}, ... } |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the ASN.1 part to change the following IE from optional to be mandatory: id-BH-Routing-Information-Added-List-Item in the BH-Routing-Information-Added-List-ItemIEs, id-BH-Routing-Information-Removed-List-Item in the BH-Routing-Information-Removed-List-ItemIEs, id-BAP-Header-Rewriting-Added-List-Item in the BAP-Header-Rewriting-Added-List-ItemIEs, id-BAP-Header-Rewriting-Removed-List-Item in the BAP-Header-Rewriting-Removed-List-ItemIE Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because the criticality of the IE is “ignore”, i.e. any missing presence of the mandatory IE will be ignored. The functional impact is that the unpredictable behaviour of the receiver of the IE is avoided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Design of ASN.1 has error, since the ProtocolIE-SingleContainer cannot include optional IE. Will cause unpredictable behaviour for receiver of the IE if the container is empty. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Update the cover page to indicate the functional impact and update the consequences if not approved. |  |  |  |  |  |  |  |  |

CHANGES START

9.4.4	PDU Definitions

>>>>>>>>>>>>>>>unchanged parts are skipped<<<<<<<<<<<<<<<<

-- **************************************************************

--

#### -- IAB PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- BAP Mapping Configuration ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION

-- **************************************************************

BAPMappingConfiguration ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ {BAPMappingConfiguration-IEs} },

...

}

BAPMappingConfiguration-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE	TransactionID	PRESENCE mandatory}|

{ ID id-BH-Routing-Information-Added-List		CRITICALITY ignore	TYPE	BH-Routing-Information-Added-List	PRESENCE optional}|

{ ID id-BH-Routing-Information-Removed-List		CRITICALITY ignore	TYPE	BH-Routing-Information-Removed-List	PRESENCE optional}|

{ ID id-TrafficMappingInformation				CRITICALITY ignore	TYPE	TrafficMappingInfo						PRESENCE optional},

...

}

BH-Routing-Information-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BH-Routing-Information-Added-List-ItemIEs } }

BH-Routing-Information-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BH-Routing-Information-Removed-List-ItemIEs } }

BH-Routing-Information-Added-List-ItemIEs	F1AP-PROTOCOL-IES ::= {

{ ID id-BH-Routing-Information-Added-List-Item				CRITICALITY ignore	TYPE BH-Routing-Information-Added-List-Item						PRESENCE mandatory},

...

}

BH-Routing-Information-Removed-List-ItemIEs	F1AP-PROTOCOL-IES ::= {

{ ID id-BH-Routing-Information-Removed-List-Item				CRITICALITY ignore	TYPE BH-Routing-Information-Removed-List-Item						PRESENCE mandatory},

...

}

BAP-Header-Rewriting-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Added-List-ItemIEs } }

BAP-Header-Rewriting-Added-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BAP-Header-Rewriting-Added-List-Item	CRITICALITY ignore	TYPE BAP-Header-Rewriting-Added-List-Item PRESENCE mandatory},

...

}

BAP-Header-Rewriting-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Removed-List-ItemIEs } }

BAP-Header-Rewriting-Removed-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BAP-Header-Rewriting-Removed-List-Item	CRITICALITY ignore	TYPE BAP-Header-Rewriting-Removed-List-Item PRESENCE mandatory},

...

}

CHANGES END