3GPP TSG-RAN3 Meeting #125bis	R3-245809

# Hefei, China, 14-18 Oct, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IAB F1-C transfer in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson, Lenovo, Huawei, Qualcomm, Nokia, Nokia Shanghai Bell, CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-10-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In R17, F1-C transfer in NR-DC is supported in IAB including two scenarios as captured in TS 37.340. And the terms “F1-terminating IAB-donor” and “non F1-terminating IAB-donor” are used for the scenarios of the F1-transfer in NR-DC. However, the terms “F1-terminating IAB-donor” and “non F1-terminating IAB-donor” refer to the two IAB-donors for the boundary IAB-node for partial migration, inter-donor redundancy and inter-donor RLF recovery scenarios. The definitions for F1-terminating IAB-donor and boundary IAB-donor in TS 38.401 are copied in the below for information: TS 38.401 F1-terminating IAB-donor of boundary IAB-node: Refers to the IAB-donor that terminates F1 for the boundary IAB-node. Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery. As we can see, the terms “F1-terminating donor” and “non-F1 terminating donor” are not applicable to the scenario of F1-C transfer in NR-DC. Specifically, there is only one IAB donor in the F1-C transfer scenario while the other gNB serving the IAB-MT doesn’t act as an IAB-donor. - In Figure 7.12-1, “M-NG-RAN node” and “S-NG-RAN node” are used, which is not aligned with the terminology used in the TS 37.340. - The caption of Figure 10.10.2-5 “Scenario 2: F1-C Traffic Transfer procedure between IAB-MT and MN (F1-terminating node) in NR-DC” is misleading since the F1-C traffic is between IAB-DU and CU, rather than between IAB-MT and MN/SN. - The caption of Figure 10.15-2 “Scenario 1: F1-C transfer between IAB-MT and SN (F1-terminating node) in NR-DC” is misleading since the F1-C traffic is between IAB-DU and CU, rather than between IAB-MT and MN/SN. - The text “the F1-AP message encapsulated in SCTP/IP or F1-C related (SCTP/)IP packet” is misleading since it can be interpreted as “F1-AP message encapsulated in SCTP/IP or F1-AP message encapsulated in F1-C related (SCTP/)IP packet” which is incorrect. - In RRC Transfer procedure for supporting F1-C traffic transfer in NR-DC in section 10.10.2, step 2 and 3 only mentions F1-AP message, however the F1-C traffic includes F1-C related (SCTP/)IP packet as well. - In the procedure for F1-C traffic transfer, it is described that “IAB-MT sends an F1-AP message...” which is misleading since the F1-C traffic is generated by IAB-DU and forwarded by IAB-MT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace the terms “F1-terminating IAB-donor” and “non F1-terminating IAB-donor” with “IAB-donor” and “gNB” respectively in the text for the F1-transfer in NR-DC scenario. - Fix Figure 7.12-1 using “MN” and “SN” instead of “M-NG-RAN node” and “S-NG-RAN node”. - Update the caption of Figure 10.10.2-5 and Figure 10.15-2 without mentioning the IAB-MT. - Update the text “the F1-AP message encapsulated in SCTP/IP or F1-C related (SCTP/)IP packet” to clarify that it refers to “an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet”. - Update step 2 and 3 in section 10.10.2 to include also F1-C related (SCTP/)IP packet in addition to F1-AP message. - Update procedure text for F1-C traffic transfer to clarify that F1-C traffic is generated by IAB-DU and forwarded by IAB-MT. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has no impact from the functional point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of F1-transfer in NR-DC is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12, 10.10.2, 10.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

## 7.12	F1-C Traffic Transfer in NR-DC

In NR-DC, an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet can be transferred via BAP sublayer or via SRB between the IAB-node and the IAB-donor (as specified in TS 38.401 [7]), as specified in TS 38.331 [4]. When both MCG and SCG are configured to transfer the F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet, it is up to the IAB-node implementation for path selection. Two scenarios are supported, as shown in Figure 7.12-1.

Figure 7.12-1: F1-C transfer in NR-DC; a) Scenario 1; b) Scenario 2

Scenario 1: IAB-node exchanges an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet with the SN (IAB-donor) using NR access link via MN, and exchanges F1-U traffic using backhaul link(s) with SN. SRB2 is used for transporting the F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet between IAB-MT and MN (see TS 38.331 [4]), and the F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet is transferred in a container via XnAP between MN and SN, see TS 38.423 [5].

Scenario 2: IAB-node exchanges an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet with the MN (IAB-donor) using NR access link via SN, and exchanges F1-U traffic using backhaul link(s) with MN. Split SRB2 is used for transporting the F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet between IAB-MT and SN (see TS 38.331 [4]), and the F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet is transferred in a container via XnAP between SN and MN, see TS 38.423 [5].

-------------------------------------------Next change-------------------------------------------

10.10.2	MR-DC with 5GC

>>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<<

F1-C traffic transfer:

Figure 10.10.2-5: RRC Transfer procedure for supporting scenario 2 of F1-C traffic transfer in NR-DC

1.	The IAB-MT receives from its co-located IAB-DU an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet and forwards it to the SN via split SRB2 in a container within ULInformationTransfer message encapsulated in a PDCP PDU as specified in TS 38.331 [4].

2.	The SN initiates the RRC Transfer procedure, in which it transfers the received PDCP PDU (ULInformationTransfer message) including the F1-AP message or the F1-C related (SCTP/)IP packet.

3.	When the MN (IAB-donor) sends a F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet, it starts the procedure by initiating the RRC Transfer procedure, if split SRB2 is determined to be used and usage of SCG path is determined. The MN sends the F1-AP message or the F1-C related (SCTP/)IP packet to the SN in a container within DLInformationTransfer message encapsulated in a PDCP PDU specified in TS 38.331 [4].

4.	The SN forwards the encapsulated DLInformationTransfer message in a PDCP PDU as specified in TS 38.331 [4] to IAB-MT.

-------------------------------------------Next change-------------------------------------------

10.15	F1-C Traffic Transfer

In EN-DC/NR-DC, the F1-C Traffic Transfer message is sent by the MN to the SN or by the SN to MN to transfer the F1-C traffic to and from an IAB-node.

Figure 10.15-1: F1-C Traffic Transfer procedure in EN-DC

1.	When the IAB-MT receives from its co-located IAB-DU an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet, it forwards it to the MN in a container within ULInformationTransfer message as specified in TS 36.331 [10].

2.	The MN initiates the F1-C Traffic Transfer procedure, in which it transfers the received F1-AP message encapsulated in (SCTP/)IP or the F1-C related (SCTP/)IP packet as an octet string.

3.	When the SN sends an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet, it sends it to the MN as an octet string by initiating the F1-C Traffic Transfer procedure.

4.	The MN sends the received F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet to the IAB-MT in a container within DLInformationTransfer message as specified in TS 36.331 [10].

Figure 10.15-2: F1-C Traffic Transfer procedure for scenario 1 of F1-C traffic transfer in NR-DC

1.	The IAB-MT receives from its co-located IAB-DU an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet and forwards it to the MN via SRB2 in a container within ULInformationTransfer message as specified in TS 38.331 [4].

2.	The MN initiates the F1-C Traffic Transfer procedure, in which it transfers the received F1-AP message encapsulated in (SCTP/)IP or the F1-C related (SCTP/)IP packet as an octet string.

3.	The SN (IAB-donor) sends an F1-AP message encapsulated in SCTP/IP or an F1-C related (SCTP/)IP packet to the MN as an octet string by initiating the F1-C Traffic Transfer procedure.

4.	The MN sends the received F1-AP message encapsulated in SCTP/IP or the F1-C related (SCTP/)IP packet to the IAB-MT via SRB2 in a container within DLInformationTransfer message as specified in TS 38.331 [4].

-------------------------------------------End of changes-------------------------------------------