3GPP TSG-RAN WG3 Meeting #125-bis	R3-245829

Hefei , CN, 14 - 18 Oct, 2024

Title:	(TP to TR 38.769) Protocol Stacks of Topology 2 solutions

Agenda Item:	16.2

Source:	Huawei, CMCC, ZTE, CATT, NEC, Xiaomi, Lenovo

Document for:	other

- Introduction

In last meeting, the following were listed as solutions for topology 2:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

The detailed protocol stack was not captured due to limit of discussion/time, considering all the previous discussions, in this paper, the details of these solutions are provided.

- Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.4.2.1	Solutions for Topology 2

6.4.2.1.0	General

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

NOTE:	The protocol stack for each solution option does not illustrate AIoT CN internal architecture and how A-IoT upper layer information is transported, if any. Details are subject to SA2 agreements.

6.4.2.1.1	Solution1: RRC based solution

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1-1: RRC based solution of Topology 2

6.4.2.1.2	Solution 2: NAS based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s DL/UL NAS packets, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport procedures over NGAP.

Editor’s Note: How the A-IoT CN and the A-IoT-enabled UE are communicated via A-IoT related messages is upto SA2 decision and should be depicted by SA2.

Figure 6.4.2.1-2: NAS based solution of Topology 2

Editor’s Note: Control and coordination of AIoT radio resources should be enabled in NAS based solution, there are different options to achieve this purpose, as described in 6.5.1.3, validity of these options is FFS.

6.4.2.1.3	Solution 3: UP based solution

The A-IoT related messages between the A-IoT CN and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s PDU Session, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s user plane data as legacy, i.e., over NG-U GTP-U tunnels.

Editor’s Note: How the A-IoT CN and the A-IoT-enabled UE are communicated via A-IoT related messages is upto SA2 decision and should be depicted by SA2.

Figure 6.4.2.1-3: UP based solution of Topology 2

Editor’s Note: Control and coordination of AIoT radio resources should be enabled in NAS based solution, there are different options to achieve this purpose, as described in 6.5.1.3, validity of these options is FFS.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>