3GPP TSG-RAN WG3 Meeting #125bis	R3-245848

Hefei, P.R. China, 14th – 18th October 2024

Agenda Item:	16.3

Source:	Ericsson

Title:	[TP for TR 38.769] CB:#AIoT2_RANCNinterface

Document for:	Discussions & Approval

# 1	Introduction

This document is the summar of offline discussion CB: # AIoT2_RANCNinterface

- Check the details in R3-245307, taking R3-245581 into account

- Continue the discussion on reader selection for Topology1 and Topology2 based on progress in RAN2

(moderator - E///)

This document contains a TP for 38.769.

# 2	Text Proposal for TR 38.769v1.0.0

#### 6.5.1.3	AIoT radio resource allocation in case of NAS/UP based solutions

In NAS/UP based solutions, AIoT radio resources can be requested in advance to the NAS/UP based communication with the AIoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger AIoT radio resource allocation, e.g. upon CN request or or upon UE request, etc. Some aspects of AIoT radio allocation may be preconfigured by OAM.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.5.2	Signaling and Procedures for Topology 1

#### 6.5.2.1	Candidate procedures for A-IoT Inventory for Topology 1

NOTE 1: XX communication depicted in the following chapters uses protocol elements (messages and information elements) detailed in section 6.5.1 and are not repeated unless additional description is necessary.

Editor’s Note 2: Reader selection is FFS.

#### 6.5.2.1	Candidate procedures for A-IoT Inventory for Topology 1

Figure 6.5.2.1-1: Message flow for AIoT Inventory in Topology 1

1a.	The AIoT CN sends an Inventory request message to the AIoT RAN node, taking into account, among others, the AIoT transaction scope.

1b	The AIoT RAN node allocates and coordinates the usage of AIoT radio resources.

2.	The AIoT RAN node sends an Inventory response message to the AIoT CN.

NOTE 1: In step 2, the AIoT RAN node may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3.	The AIoT RAN node performs the inventory procedure towards the AIoT device(s) over the AIoT radio interface.

4a/4b.	After receiving the inventory result from the AIoT device(s), the AIoT RAN node may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different AIoT devices.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 6.5.3	Signaling and Procedures for Topology 2

Editor’s Note: Reader selection is FFS.

#### 6.5.3.1	Candidate procedures for A-IoT Inventory for Topology 2

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flows, working on the content of the messages including ownership, associated functions, scope, etc.

##### 6.5.3.1.2	RRC solution

Figure 6.5.3.1.3-1: Message flow for A-IoT Inventory in Topology 2 – RRC solution

1a.	The AIoT CN sends an Inventory request message to the AIoT enabled gNB

1b	The AIoT enabled gNB allocates and coordinates usage of AIoT radio resources.

1c/2a	RRC communication with the AIoT enabled UE takes place.

Editor’s Note1:	RRC based communication is only depicted schematically, details in RAN2 FFS.

2.	The AIoT-enabled gNB sends an Inventory response message to the AIoT CN.

NOTE 1: In step 2, the AIoT-enabled gNB may instead send an Inventory failure message to the AIoT CN indicating that the inventory procedure could not be initiated towards the AIoT device(s).

3.	The AIoT-enabled gNB requests the AIoT-enabled UE(s) to trigger inventory procedure towards the AIoT device(s).

4a/4b.	After receiving inventory result reported from the AIoT enabled UEs, the AIoT-enabled gNB may send one or multiple Inventory reports towards the AIoT CN including the received inventory result.

NOTE 2: Steps 4a/4b may happen in parallel with Step 3 for different AIoT devices.

Editor’s Note 2:	Step 4a/4b between AIoT-enable UE and AIoT-enabled gNB can be refined by RAN2.

##### 6.5.3.1.1	NAS/UP solution

Editor’s note 1: Future discussions on A-IoT Inventory will take place based on the following message flow, working on the content of the messages including ownership, associated functions, scope, etc.

Figure 6.5.3.1.1-1: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

Editor’s note 2: how and where to depict signalling suitable for triggering A-IoT RAN node functions for A-IoT radio resource management needs further discussions for direct communication between A-IoT CN and A-IoT-enabled UE.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>