3GPP TSG-RAN WG3 Meeting #125	R3-244225

Maastricht, Netherlands, 19 - 23 August, 2024

Agenda item:	 10.4

Source:		Samsung

Title:	Discussion on the support of the leftovers in Rel-18 SON/MDT

Document for:		Discussion and Decision

1	Introduction

The following leftovers in Rel-18 SON/MDT have been agreed in the scope of Rel-19 SON/MDT WI.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure

For MHI Enhancement for SCG Deactivation/Activation, there is no agreement achieved in RAN2 yet. Therefore we could wait for RAN2 progress firstly.

The contribution discussed how to support the MRO for MR-DC SCG failure and RACH optimization for SDT.

2	Discussion

2.1 RACH optimization for SDT

RA-SDT has been designed to send the small data over RACH resources, which enables UE to transmit/send data in RRC inactive state. It is an efficient solution to avoid UE coming back to RRC connected state for the small data. gNB sets the following configuration parameters for RA-SDT and sends to UE via SIB1:

- RSRP threshold for UE to determine whether to perform SDT procedure

- Data volume threshold used to determine whether SDT can be initiated

- The value of logicalChannelSR-DelayTimer applied during SDT for logical channels configured with SDT

- A timer for SDT (T319a)

The SDT performance may be not good due to poor configuration. For example, with the low RSRP threshold, the data transmission may be interrupted due to poor channel condition. In addition, if data volume threshold is high but the timer is small, the data can not be sent within the timer limited period, so that the UE needs to re-initialize a SDT procedure or go to RRC connected state to continue the transmission. Thus, to do self-optimization, the information of RA-SDT is needed to transfer to the node who sets the configuration. The configuration information is broadcasted in system information block. For split architecture, the configuration parameters are set by DU, so F1AP impact should be considered to transfer the related information from CU to DU for DU self-optimization.

With above analysis, the following information should be provided by the UE to give the reference for node to do self-optimization.

- RSRP when SDT fails: this information gives reference information to the node to decide whether the RSRP threshold is set too low. For example, the RSRP threshold is set as a low value, while the channel condition as such threshold is not good to support the SDT transmission. If SDT fails, the nodes need to set the RSRP threshold as a higher value.

- Remaining data volume after SDT transmission: this information tells the node whether the data volume threshold is set properly or not. When the remaining data volume is small, it means that UE needs to re-initialize a SDT procedure or go to RRC connected state to continue the transmission for such a small data volume. The more desired way is to set a higher data volume threshold for the SDT triggering or set a larger timer for transmission.

Proposal 1: The following SDT related information should be provided by the UE to give the reference for node to do self-optimization:

- RSRP when SDT fails

- Remaining data volume after SDT transmission

For the need of collecting the configuration related information for the RA-based SDT, the reason is that the maximum storage time of the RA report for the UE is 48 hours. It can not assume that the RA-SDT configuration is not changed during such a long time. So it is better to provide the configuration under which the RA-SDT is done to the node. And it can avoid the node to do the adjustment for the proper configuration instead of the one which leads to the bad performance. To reduce the signaling burden, it is better for UE to report the time from the start of SDT to the reporting of RA report to the node. The node can derive the related configuration based on the time.

Proposal 2: UE reports the time from the start of SDT to the reporting of RA report to the node. Based on it, the node can derive the SDT related configuration to do self-optimization.

2.2 MR-DC SCG failure scenario

At RAN3#123bis meeting, the following initial agreements were achieved:

MRO for MR-DC SCG failure:

Keep the following R18 agreement in R19:

Support MRO for SCG failure in EN-DC, NGEN-DC and NE-DC scenarios.

LS to RAN2 on the feasibility to support MRO for SCG failure in R19 in R3-242195

RAN2 has sent the reply LS in R3-244011.

RAN2 thanks RAN3 for the LS, and RAN2 agrees to only support EN-DC case on MRO for MR-DC SCG failure. RAN2 understands that whether also supporting NGEN-DC has no additional RAN2 impact hence RAN3 can decide. If time allows in Rel-19, RAN2 can consider NE-DC scenario.

It could be observed that RAN2 will support EN-DC case at least.

For NG-EN DC, RAN2 agreed to leave the decision to RAN3. RAN3 already made agreement that NGEN-DC will be supported in R19 in RAN3#123bis meeting. So this agreement can be sticked.

Observation: RAN3 could start to discuss the solution to support MRO for SCG failure in EN-DC, NGEN-DC firstly.

The main open issue in RAN3 is that the exchange of information contained in SCG failure information between MN and SN of different RAT.

For EN-DC and NGEN-DC, the SCGFailureInformationNR is in LTE RRC format. The MN (eNB or ng-eNB) can decode the information in normal way. For MRO purpose, the MN needs to forward the SCGFailureInformationNR to a SN (gNB) if the failure is brought by the SN. The SN may not understand the SCGFailureInformationNR in LTE RRC format.

There are two options:

- The MN decodes SCGFailureInformationNR and put the necessary information to X2/Xn message for EN-DC/NGEN-DC.

- The MN includes the contents in SCGFailureInformationNR to inter-node RRC message and send the inter-node RRC message to the SN.

Both of the two options work.

The following information are included in SCGFailureInformation for MRO root cause analysis:

•	the CGI of the Source PSCell

•	CGI of the Failed PSCell

•	timeSCGFailure

•	connectionFailureType

The CGI of the Source PSCell and the CGI of the Failed PSCell have been included in the SCG FAILURE INFORMATION REPORT message from the MN to the SN. The additional information needed is timeSCGFailure and connectionFailureType. Either include them directly in the SCG FAILURE INFORMATION REPORT message or in an inter-node RRC message.

In the Annex A and B, the TPs for both options are provided for information.

From the TPs, it could be observed that the main difference is where to define a group of IEs i.e. RAN2 spec or RAN3 spec.

There are the following benefits for option (1):

- RAN3 can fully decide what information needs to be transmitted to the SN without bothering RAN2. Otherwise, each time RAN3 finds an additional parameter is necessary, RAN3 needs to send LS to RAN2.

- Option (1) will also simplify the SN implementation i.e. avoid SN to decode the inter-node RRC message.

- For Option (2), RAN2 needs to check what information has been included in X2AP/XnAP messages e.g. the CGI of the Source PSCell and the CGI of the Failed PSCell and leave them out. RAN3. Option (1) doesn’t need such cross check.

Proposal 3: It is proposed for RAN3 to select one option between option (1) and option (2). Option (1) is preferred.

Proposal 4: The MN includes the necessary information in SCGFailureInformationNR to X2/Xn message for EN-DC and NGEN-DC.

Proposal 5: Agree the TP in Annex A or Annex B to TS38.423.

During Rel-18 discussion, the following agreements were achieved at RAN3#117-e and RAN3-117bis-e meeting for EN-DC and NGEN-DC:

MRO for MR-DC SCG failure:

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for NGEN-DC SCG failure, and necessary updates can be added on top of it if needed.

Take Stage 2 descriptions of PSCell change failure in TS37.340 as baseline for EN-DC SCG failure, and necessary updates can be added on top of it if needed.

Based on these agreements, a stage 2 TP for TS37.340 is provided in Annex C.

Proposal 6: Agree the TP in Annex C for TS37.340.

For supporting MRO for EN-DC SCG failure, SCG Failure Information Report and SCG Failure Transfer procedures should be defined in TS36.423. A TP for TS36.423 is provided in Annex D.

Proposal 7: Agree the TP in Annex D for TS36.423.

3	Conclusion

This contribution discussed MRO for MR-DC SCG failure and RACH optimization for SDT. We have the following proposals.

RACH optimization for SDT

Proposal 1: The following SDT related information should be provided by the UE to give the reference for node to do self-optimization:

- RSRP when SDT fails

- Remaining data volume after SDT transmission

Proposal 2: UE reports the time from the start of SDT to the reporting of RA report to the node. Based on it, the node can derive the SDT related configuration to do self-optimization.

MRO for MR-DC SCG failure:

Observation: RAN3 could start to discuss the solution to support MRO for SCG failure in EN-DC, NGEN-DC firstly.

Proposal 3: It is proposed for RAN3 to select one option between option (1) and option (2). Option (1) is preferred.

Proposal 4: The MN includes the necessary information in SCGFailureInformationNR to X2/Xn message for EN-DC and NGEN-DC.

Proposal 5: Agree the TP in Annex A or Annex B to TS38.423.

Proposal 6: Agree the TP in Annex C for TS37.340.

Proposal 7: Agree the TP in Annex D for TS36.423.

Reference

[1] TS38.331

[2] TS36.331

[2] TS38.423

Annex A: TP for BLCR TS38.423 – Option (1)

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.103 |  | YES | ignore |
| MR-DC SCG Failure Information | O |  | 9.2.2.xxx |  | YES | ignore |

#### 9.2.2.xxx	MR-DC SCG Failure Information

This IE contains information about the MR-DC SCG failure.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Time SCG Failure | M |  | INTEGER (0..1023) |  | – |  |
| Connection Failure Type | M |  | ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16, t312-Expiry-r16, bh-RLF-r16, beamFailure-r17, spare3, spare2, spare1} |  | – |  |

Annex B: TP for BLCR TS38.423 – Option (2)

#### 9.1.2.29	SCG FAILURE INFORMATION REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a PSCell change failure event.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node. | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node. | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformation message or the SCGFailureInformationEUTRA message as defined in TS 38.331 [10] or the SCGFailureInformation message or the SCGFailureInformationNR message as defined in TS 36.331 [14] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| CPAC Configuration | O |  | 9.2.2.103 |  | YES | ignore |
| MR-DC SCG Failure Information | O |  | OCTET STRING | Contains the xxx as defined in TS 38.331 [10] or the yyy as defined in TS 36.331 [14] | YES | ignore |

Annex C: TP for BLCR TS37.340

### 10.18.2	PSCell change failure

One of the functions of self-optimization for PSCell change is to detect PSCell change failures that occur due to Too late PSCell change or Too early PSCell change, or Triggering PSCell change to wrong PSCell. These problems are defined as follows:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

In the definition above, the "successful PSCell change" refers to the UE state, namely the successful completion of the RA procedure.

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The MN puts the necessary information in SCGFailureInformationNR to SCG Failure Information Report message for EN-DC and NGEN-DC. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

Annex D: TP for BLCR TS36.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 7	Functions of X2AP

*****************skip unchanged part ******************

The mapping between the above functions and X2 EPs is shown in the table below.

Table 7-1: Mapping between X2AP functions and X2AP EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| Mobility Management | a) Handover Preparation b) SN Status Transfer c) UE Context Release d) Handover Cancel e) Handover Success f) Conditional Handover Cancel |
| Dual Connectivity | a) SeNB Addition Preparation b) SeNB Reconfiguration Completion c) MeNB initiated SeNB Modification Preparation d) SeNB initiated SeNB Modification e) MeNB initiated SeNB Release f) SeNB initiated SeNB Release g) SeNB Counter Check |
| E-UTRA-NR Dual Connectivity | a) SgNB Addition Preparation b) SgNB Reconfiguration Completion c) MeNB initiated SgNB Modification Preparation d) SgNB initiated SgNB Modification e) SgNB change f) MeNB initiated SgNB Release g) SgNB initiated SgNB Release h) SgNB Counter Check i) RRC transfer j) EN-DC X2 Setup k) EN-DC Configuration Update l) EN-DC Cell Activation m) SgNB Activity Notification n) EN-DC X2 Removal o) gNB Status Indication p) EN-DC Resource Status Reporting Initiation q) EN-DC Resource Status Reporting r) F1-C Traffic Transfer |
| Load Management | a) Load Indication b) Resource Status Reporting Initiation c) Resource Status Reporting |
| Reporting of General Error Situations | Error Indication |
| Resetting the X2 | Reset |
| Setting up the X2 | X2 Setup |
| eNB Configuration Update | a) eNB Configuration Update b) Cell Activation |
| Mobility Parameters Management | Mobility Settings Change |
| Mobility Robustness Optimisation | a) Radio Link Failure Indication b) Handover Report c) SCG Failure Information Report d) SCG Failure Transfer |
| Energy Saving | a) eNB Configuration Update b) Cell Activation |
| X2 Release | X2 Release |
| Message Transfer Registration | X2AP Message Transfer |
| Removing the X2 | X2 Removal |
| Inter-eNB UE Context Retrieval | a) Retrieve UE Context b) Data Forwarding Address Indication |
| Secondary RAT Data Usage Report | Secondary RAT Data Usage Report |
| E-UTRA - NR Spectrum Sharing | E-UTRA - NR Cell Resource Coordination |
| EN-DC Configuration Transfer | EN-DC Configuration Transfer |
| UE Radio Capability ID Mapping | UE Radio Capability ID Mapping |

# 8	X2AP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

*****************skip unchanged part ******************

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |
| SgNB Reconfiguration Completion | SGNB RECONFIGURATION COMPLETE |
| SgNB Counter Check | SGNB COUNTER CHECK REQUEST |
| RRC Transfer | RRC TRANSFER |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| SgNB Activity Notification | SGNB ACTIVITY NOTIFICATION |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| gNB Status Indication | GNB STATUS INDICATION |
| EN-DC Configuration Transfer | EN-DC CONFIGURATION TRANSFER |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| EN-DC Resource Status Reporting | EN-DC RESOURCE STATUS UPDATE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |
| Access and Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Conditional PSCell Change Cancel | CONDITIONAL PSCELL CHANGE CANCEL |
| RACH Indication | RACH INDICATION |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |
| SCG Failure Transfer | SCG FAILURE TRANSFER |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.7.x	SCG Failure Information Report

#### 8.7.x.1	General

The purpose of the SCG Failure Information Report procedure is to provide SCG mobility related information to an en-gNB.

The procedure uses UE-associated signalling.

#### 8.3.x.2	Successful Operation

Figure 8.7.x.2-1: SCG Failure Information Report, successful operation

The MeNB initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to the en-gNB. Upon receiving the message, the en-gNB shall assume that a PSCell change failure event was detected.

The SCG FAILURE INFORMATION REPORT message may include:

-	the SN Mobility Information IE, if the SN Mobility Information IE was sent for the PSCell change procedure from the en-gNB;

-	the Source PSCell CGI IE, if the Source PSCell CGI IE was sent for the PSCell change procedure from the en-gNB.

If the SCG FAILURE INFORMATION REPORT message includes the Source PSCell CGI IE, the en-gNB shall, if supported, store the information.

If the SCG FAILURE INFORMATION REPORT message includes the Failed PSCell CGI IE, the en-gNB shall, if supported, store the information and act as specified in TS 36.300 [15].

If received, the en-gNB uses the above information for SCG failure reason detection and optimisation.

#### 8.3.x.3	Unsuccessful Operation

Not applicable.

#### 8.3.x.4	Abnormal Conditions

Void.

### 8.7.y	SCG Failure Transfer

#### 8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Information Transfer, successful operation

en-gNB initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If received, MeNB uses the information according to TS 36.300 [15].

#### 8.3.18.3	Unsuccessful Operation

Not applicable.

#### 8.3.18.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.x	SCG FAILURE INFORMATION REPORT

This message is sent by the MeNB to the en-gNB to report a PSCell change failure event.

Direction: MeNB  the en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Source PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed PSCell CGI | O |  | NR CGI 9.2.111 | NR CGI of PSCell where SCG failure occurs for PSCell change procedure | YES | ignore |
| SCG Failure Report Container | M |  | OCTET STRING | Contains the SCGFailureInformationNR message as defined in TS 36.331 [9] | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by en-gNB in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |

#### 9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>