3GPP TSG-RAN WG3 Meeting #125	R3-244297

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	(TP to TR 38.743) Split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

The agreements in last RAN3 meeting [1] which could have impacts to the support of split architecture as part of the Rel-18 leftovers objective of the RAN AI/ML enhancements SI are listed below:

Energy Saving Enhancements:

Measured Energy Cost can be transferred from DU to CU.

Only discuss on finer granularity of energy cost in Aug meeting and make the conclusion.

Split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

This contribution addresses the Rel-18 leftover related to the “Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI” and provides proposals for discussion, based on the agreements above. The FFS concerning the granularity of the Energy Cost, instead, is addressed in another contribution [2].

# 2. Discussion

## 2.1 Background

In the context of the Rel-18 WI “AI/ML for NG-RAN” [3] RAN3 specified data collection enhancements and signalling support within existing NG-RAN interfaces and architecture for three selected use cases, namely, (network) Energy Saving (ES), Mobility Enhancements (ME) and Load Balancing (LB). To this end, RAN3 introduces two new procedures, i.e., Data Collection Reporting Initiation (class 1 procedure) and Data Collection Reporting (class 2), which allow to signal over Xn both predictions and measurements to properly enable the considered inference-based use cases. However, due to time constraints, RAN3 agreed in RAN3#121bis that enhancements to support AI/ML for split RAN architecture are not pursued in Rel-18 (refer to the highlighted part in green in the agreement below), meaning that support for AI/ML in NG-RAN is only specified for the non-split architecture in Rel-18 [4]:

WA: Defining new procedures to transfer the measured EC over F1 follows the design over Xn.

The following overwrites the previous WA: enhancements to support AI/ML for split RAN architecture are not pursued in R18.

As a consequence, the support of the split architecture for the Rel-18 use cases was eventually included in the list of leftovers for further discussion in Rel-19. In the Rel-19 SID [5] it is clearly indicated that, for this specific leftover, the discussion should be based on the conclusions from Rel-18 WI. Therefore, in the following, a list of the relevant agreements reached in RAN3 during Rel-18 are reported.

From RAN3#117bis-e meeting, some initial agreements assessing the impact of RAN AI/ML over E1 and F1 interfaces were achieved as reported [6]:

UE traffic metric takes the data volume for a UE as the starting point.

The cell-level UE trajectory prediction function is located in gNB CU-CP.

The location for resource status prediction in split architecture:

- For current resource status input data from gNB DU, the resource status prediction function is located in gNB CU-CP.

- For current resource status input data from gNB CU-UP, the resource status prediction function is located in gNB CU-CP.

Also, a bunch of open issues that needed further discussed were listed [6]:

Discussion to be continued on the following:

- FFS on detailed E1/F1 impact for the exchange of current/predicted UE traffic after the sufficient work for non-split architecture.

- FFS on detailed E1/F1 impact for the exchange of current/predicted UE trajectory after the sufficient work for non-split architecture.

- FFS on detailed energy efficiency metric and corresponding E1/F1 impact after the sufficient work for non-split architecture.

The above FFSs led RAN3 to stop discussing the split architecture support of AI/ML for NG-RAN until RAN3#121 meeting, during which the discussion mainly focused on the ES use case, in particular on how to transfer the measured Energy Cost (EC) metric from DU to CU over the F1 interface, and the following agreement was reached [7]:

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

Then, in RAN3#121bis meeting, RAN3 discussed whether reusing the current F1AP procedures or defining a new procedure to enable the DU to report its measured EC to the CU (see highlighted text in cyan above). Note that only the F1 interface was considered at that time, since companies had concerns on transferring the measured EC of the CU-UP to the CU-CP over the E1 interface due to the fact that typically the CU-UP is virtualized within cloud platforms, hence making the computation of the concerned CU-UP’s energy consumption difficult. However, there was no consensus in RAN3 to turn the Working Assumption (WA) from RAN3#121bis on introducing a new F1AP procedure into an agreement due to the fact that introducing a new procedure to only transfer a single metric (the measured EC) over an interface was not justifiable. This led to the previously reported agreement to not introduce support of the split architecture for Rel-18 AI/ML in NG-RAN [4]:

WA: Defining new procedures to transfer the measured EC over F1 follows the design over Xn.

The following overwrites the previous WA: enhancements to support AI/ML for split RAN architecture are not pursued in R18.

In the next sections we provide our view on the AI/ML-related information that could be transferred over E1 and F1 interfaces, as well as an analysis on the specification impact required to enable such transfer.

## 2.2 On gNB-DU Energy Cost reporting over F1

In the last meeting RAN3 agreed that the measured Energy Cost (EC) can be transmitted over F1 from DU to CU(-CP) [1], basically re-confirming the agreement taken during Rel-18 WI discussions on split architecture:

Energy Saving Enhancements:

Measured Energy Cost can be transferred from DU to CU.

Hence, we think RAN3 need to only decide how to enable such reporting, i.e., discuss whether to re-use existing F1AP procedures or introduce brand new procedures similar to the XnAP’s Data Collection Reporting (Initiation) procedures.

Observation 1: RAN3 is requested to only discuss and decide on how to enable the reporting over F1 of the DU’s measured EC to the CU(-CP).

However, such discussion relates to Stage3 details that, in our view, should be discussed during the corresponding RAN AI/ML Rel-19 normative phase and not in the context of a feasibility study. Therefore, we propose the following:

Proposal 1: RAN3 to discuss and decide on how to enable the reporting of the measured DU’s EC to CU(-CP) during the Rel-19 normative phase for RAN AI/ML, i.e., whether new F1AP procedures should be specified or legacy procedures can be enhanced instead.

## 2.3 On reporting measured UE performance

In addition to the reporting of the measured EC from DU to CU, RAN3 also agreed in the last meeting that the UE performance after an AI/ML-based mobility decision should be reported, within the split target gNB, from CU-UP to CU-CP [1]:

Split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

RAN3 introduced in Rel-18 the Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss as the metrics representing the UE performance to be reported from target gNB to source gNB over Xn after an AI/ML-based mobility action (i.e., handover). In case the target gNB is a split node, we think it is important that the target CU/CU-CP is able to collect UE performance information, so that the target CU/CU-CP can report the aggregated UE performance to the source gNB for, e.g., AI/ML model monitoring in the source node.

Observation 2: After an AI/ML-based mobility action, and if the target node is a split gNB, the target CU/CU-CP needs to collect UE performance information so that the target CU/CU-CP can report the aggregated UE performance to the source gNB.

As per TS 28.558, the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss are measured in the gNB-DU.

While, for the Average Packet Delay, TS 28.558 specified the delay components for both DL and UL – refer to Figure 1 and Figure 2, respectively – some of which are measured by the DU and others by the CU-UP.

Figure 1 - RAN part of DL packet delay measurement.

Figure 2 – RAN part (including UE) of UL packet delay measurement.

Table 1 – UE performance metrics, where they are measured.

|  | gNB-DU | gNB-CU-UP |
| --- | --- | --- |
| Average UE Throughput DL | X |  |
| Average UE Throughput UL | X |  |
| Average Packet Loss | X |  |
| DL packet delay component | D1, D2 | D3, D4 |
| UL packet delay component | D2.1, D2.2 | D1, D2.3, D2.4 |

The above Table 1 shows where each of the UE performance metrics is measured, and it is clear that Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss should be reported to the CU(-CP) via F1 signalling. Therefore, we propose:

- Proposal 2: RAN3 to agree on exchanging over F1 (DU to CU/CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

For the Average Packet Delay, instead, since there are multiple components, some of them measured at DU level and others at UP level, we think that the CU(-CP) could be made aware of the overall Average Packet Delay via one of the following options:

- Option 1: CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to CU-CP;

- Option 2: DU and CU-UP independently send their own measurements of the Average Packet Delay components to CU-CP over F1 and E1, respectively; it’s then up to CU-CP to aggregate the received measurements to derive the overall Average Packet Delay measurement;

- Option 3: DU sends its own measurements of the Average Packet Delay components to CU-UP over F1 and CU-UP forwards such measurements to CU-CP transparently over E1; with respect to Option 1, in this way it could still be possible for the CU-CP to have visibility of the different delay components in DU and CU-UP (same as in Option 2).

In our view, considering that this is the last meeting to conclude the SI, the evaluation (and related Stage 3 impacts) of each of the options above should be done during the Rel-19 WI phase. Therefore, we propose:

Proposal 3: RAN3 to decide in Rel-19 normative phase which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to CU-CP;

- Option 2: DU and CU-UP independently send their own measurements of the Average Packet Delay components to CU-CP over F1 and E1, respectively, and CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: DU sends its own measurements of the Average Packet Delay components to CU-UP over F1 and CU-UP forwards such measurements to CU-CP transparently over E1.

In the next Section 2.4 we provide some further considerations on additional information (both predictions and measurements) that could be useful to be exchanged over E1/F1 for supporting Rel-18 RAN AI/ML use cases in split architecture.

## 2.4 On other AI/ML-related information to be transferred over E1 and F1 interfaces

In Rel-18 RAN3 agreed to signal prediction information (Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted RRC connections, UE trajectory prediction), UE performance (Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay, Average Packet Loss), measured UE trajectory and measured Energy Cost over Xn interface. In addition, for the split architecture scenario, RAN3 agreed that AI/ML Model Training is located in CU/CU-CP (other than in OAM) and AI/ML Model Inference function is located in CU/CU-CP. Previous sections already addressed the exchange over F1 of the DU’s measured EC, as well as of the UE performance over F1 and E1.

Concerning the UE trajectory prediction, we think it could be useful to be transferred over F1 from the CU to the DU in order to assist resource allocation in those cells that are predicted to be visited by the UE.

By referring to Figure 3, let’s assume that a UE is handed over from Cell A in Node 1 (source gNB) to Cell B in Node 2 (next-hop target gNB) as a consequence of an inference-based mobility action, based on which the UE trajectory prediction has been sent from Node 1 to Node 2 via the HANDOVER REQUEST message over Xn. We think that sending the UE trajectory prediction over F1 to the gNB-DUs controlling the subsequent intra-gNB cells in Node 2 which are expected to be visited by the UE could be useful for timely resource allocation in those cells (Cell C and Cell D in Node 2 of Figure 3), hence improving the overall handover performance.

Figure 3 - UE trajectory prediction transferred from CU to DUs in the target node side.

Based on the above we propose:

Proposal 4: RAN3 to discuss whether the UE trajectory prediction received from the source gNB over Xn can be transferred from the target CU to target DU(s) over the F1 for timely resource allocation in target gNB cells which are expected to be visited by the UE.

For other prediction information, at least for the inter-gNB cases, we have not seen any benefit of transferring it from CU/CU-CP to DU/CU-UP, however we are open to discuss the intra-gNB cases since sending such kind of information could be helpful to timely resource reservation or scheduling. Based on that, we propose the following:

- Proposal 5: RAN3 to discuss whether other prediction information (Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted RRC connections) needs to be transferred from CU/CU-CP to DU/CU-UP for the intra-gNB cases.

In Rel-18 we discussed whether the current UE traffic information should be transferred over E1, and RAN3 reached the below agreement in RAN3#117bis-e meeting [6]:

UE traffic metric takes the data volume for a UE as the starting point.

Considering that the CU-CP is the location of the inference function, we think that the CU-UP can transfer the current UE traffic over E1 so that the CU-CP can infer the predicted UE traffic – which, in turn, is not needed to be transferred to the CU-UP over E1 – and derive potential actions for, e.g., LB purposes such as offload some UEs from CU-UP1 to CU-UP2, both CU-UP1 to CU-UP2 being controlled by the same CU-CP. Therefore, we propose:

Proposal 6: RAN3 to discuss whether to transfer the current UE traffic over E1 from CU-UP to CU-CP to allow the CU-CP to infer the predicted UE traffic and derive potential mobility actions for the UE(s), or to assist the AI/ML model in the CU-CP to understand whether it needs to be retrained.

Finally, another information that is worth to consider is the measured UE trajectory, for which it should be discussed whether it needs to be transferred from DU to CU over F1. Likewise, the UE History Information IE can be obtained by the CU directly, hence not via the DU. Therefore, we think that the measured UE trajectory can follow a similar procedure without the involvement of the DU.

Proposal 7: RAN3 to discuss and agree that the measured UE trajectory is collected by the CU directly, without DU involvement.

A TP for the TR 38.743 reflecting the above proposals is provided in the Annex of this paper.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals:

Observation 1: RAN3 is requested to only discuss and decide on how to enable the reporting over F1 of the DU’s measured EC to the CU(-CP).

Proposal 1: RAN3 to discuss and decide on how to enable the reporting of the measured DU’s EC to CU(-CP) during the Rel-19 normative phase for RAN AI/ML, i.e., whether new F1AP procedures should be specified or legacy procedures can be enhanced instead.

Observation 2: After an AI/ML-based mobility action, and if the target node is a split gNB, the target CU/CU-CP needs to collect UE performance information, so that the target CU/CU-CP can report the aggregated UE performance to the source gNB.

- Proposal 2: RAN3 to agree on exchanging over F1 (DU to CU/CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

Proposal 3: RAN3 to decide in Rel-19 normative phase which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to CU-CP;

- Option 2: DU and CU-UP independently send their own measurements of the Average Packet Delay components to CU-CP over F1 and E1, respectively, and CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: DU sends its own measurements of the Average Packet Delay components to CU-UP over F1 and CU-UP forwards such measurements to CU-CP transparently over E1.

Proposal 4: RAN3 to discuss whether the UE trajectory prediction received from the source gNB over Xn can be transferred from the target CU to target DU(s) over the F1 for timely resource allocation in target gNB cells which are expected to be visited by the UE.

- Proposal 5: RAN3 to discuss whether other prediction information (Predicted Radio Resource Status, Predicted Number of Active UEs, Predicted RRC connections) needs to be transferred from CU/CU-CP to DU/CU-UP for the intra-gNB cases.

Proposal 6: RAN3 to discuss whether to transfer the current UE traffic over E1 from CU-UP to CU-CP to allow the CU-CP to infer the predicted UE traffic and derive potential mobility actions for the UE(s), or to assist the AI/ML model in the CU-CP to understand whether it needs to be retrained.

Proposal 7: RAN3 to discuss and agree that the measured UE trajectory is collected by the CU directly, without DU involvement.

# 4. Reference

- 3GPP RAN3#124 Chairman’s notes (RAN3_124_agenda_20240524_eom2.doc)

- R3-244298, Rel-18 AI/ML for NG-RAN leftovers: Energy Saving Enhancements, Huawei

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#121b Chairman’s notes (RAN3_121bis_agenda_20231013_EOM1.doc)

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP RAN3#117b Chairman’s notes (RAN3_117bis-e_agenda_20221018_EOM1.doc)

- 3GPP RAN3#121 Chairman’s notes (RAN3_121_agenda_20230825_EOM1.doc)

# 5. Annex - TP to TR 38.743 (based on R3-243971)

-------------------------------------------------------------Start of Change-----------------------------------------------------------------

## 5.2	Split architecture support for Rel-18 use cases

Editor Note: Capture the description and its potential standard impacts.

### 5.2.1 Use case description

The split architecture should be enhanced to support the Rel-18 use cases, e.g., Load Balancing, Energy Saving, and Mobility Optimization.

In case of CU(-CP)-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP);

- AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP).

### 5.2.2 Potential Standard impacts

The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

E1 interface:

- Current UE traffic from gNB-CU-UP to gNB-CU-CP.

F1 interface:

-	Measured Energy Cost from gNB-DU to gNB-CU(-CP);

-	Average UE Throughput DL, Average UE Throughput UL, Average Packet Loss from gNB-DU to gNB-CU(-	CP);

- 	UE trajectory prediction from gNB-CU(-CP) to gNB-DU.

The collection and reporting of the Average Packet Delay UL/DL in split architecture scenarios will be discussed during the Rel-19 normative phase.